# PicOS 4.6 BGP EVPN Functional Test Cases — Modules 1, 2, 3

Platform: PicOS 4.6
RFC Reference: RFC 4271 (BGP-4), RFC 4760 (Multiprotocol Extensions), RFC 4364 (BGP/MPLS IP VPNs), RFC 7432 (EVPN), RFC 8326 (Graceful BGP Session Shutdown)
Generated: 2026-03-27

## Table of Contents

- 1. FUNCTIONAL TESTING

  - 1.1 VPN Address-Family Activation & Next-Hop Rewrite (MODULE 1)

    - 1.1.1 VPNv4/VPNv6 Activation

      - 1.1.1.1 MOD1-01: Peer-group ipv4-vpn activation

      - 1.1.1.2 MOD1-02: Peer-group ipv4-vpn deactivation

      - 1.1.1.3 MOD1-03: Peer-group ipv6-vpn activation

    - 1.1.2 Next-Hop Rewrite

      - 1.1.2.1 MOD1-04: next-hop-self

      - 1.1.2.2 MOD1-05: next-hop-self force

      - 1.1.2.3 MOD1-06: remove next-hop-self

  - 1.2 Prefix Limits & Conditional Advertisement (MODULE 2)

    - 1.2.1 Prefix Limits

      - 1.2.1.1 MOD2-01: maximum-prefix below threshold

      - 1.2.1.2 MOD2-02: maximum-prefix over threshold

      - 1.2.1.3 MOD2-03: maximum-prefix-out

    - 1.2.2 Conditional Advertisement

      - 1.2.2.1 MOD2-04: exist-map positive case

      - 1.2.2.2 MOD2-05: exist-map negative case

      - 1.2.2.3 MOD2-06: non-exist-map positive case

      - 1.2.2.4 MOD2-07: next-hop route-map match

  - 1.3 First-AS Enforcement & Graceful Shutdown (MODULE 3)

    - 1.3.1 First-AS Enforcement

      - 1.3.1.1 MOD3-01: neighbor enforce-first-as enabled

      - 1.3.1.2 MOD3-02: neighbor enforce-first-as disabled

      - 1.3.1.3 MOD3-03: peer-group enforce-first-as inheritance

    - 1.3.2 Graceful Shutdown

      - 1.3.2.1 MOD3-04: neighbor graceful-shutdown enable

      - 1.3.2.2 MOD3-05: neighbor graceful-shutdown disable

      - 1.3.2.3 MOD3-06: peer-group graceful-shutdown

## 1. FUNCTIONAL TESTING

### 1.1 VPN Address-Family Activation & Next-Hop Rewrite (MODULE 1)

RFC Sections: RFC 4760 MP-BGP address-family procedures, RFC 4364 VPN route advertisement, RFC 7432 EVPN route exchange
Priority: P0
Cases: 6

#### 1.1.1 VPNv4/VPNv6 Activation

##### 1.1.1.1 Verify peer-group ipv4-vpn activation advertises EVPN routes

| Field | Content |
| --- | --- |
| Test Name | MOD1-01: Peer-group ipv4-vpn Activation |
| Purpose Of The Test | Verify that enabling set protocols bgp peer-group RR-PG ipv4-vpn activate true allows Leaf1 to advertise tenant IPv4 reachability through the EVPN fabric. |
| Test Topo & Precondition | Topology:DUT-A (SW1, RR1, lo: 10.255.0.1)DUT-B (SW2, RR2, lo: 10.255.0.2)DUT-C (SW3, Leaf1/VTEP, lo: 10.255.0.3, VTEP: 10.0.0.3)DUT-D (SW4, Leaf2/VTEP, lo: 10.255.0.4, VTEP: 10.0.0.4)DUT-A ----- DUT-CDUT-A ----- DUT-DDUT-B ----- DUT-CDUT-B ----- DUT-DPreconditions:1. Four PicOS 4.6 switches are built as a stable EVPN fabric2. DUT-C and DUT-D peer to DUT-A and DUT-B through peer-group RR-PG3. set protocols evpn enable is already configured on all DUTs4. DUT-C has tenant IPv4 reachability to advertise through the fabric5. ipv4-vpn under RR-PG is disabled before the testConfiguration:DUT-C:set protocols bgp peer-group RR-PG ipv4-vpn activate truecommit |
| Test Procedure | 1. Configure DUT-C with ipv4-vpn activate true under RR-PG2. Wait 30 seconds for route advertisement3. Execute run show bgp evpn summary on DUT-A4. Execute run show bgp evpn route on DUT-A5. Execute run show bgp evpn route on DUT-B |
| Expected Results | 1. run show bgp evpn summary on DUT-A shows the EVPN peer to DUT-C in established state2. run show bgp evpn route on DUT-A contains EVPN routes originated by DUT-C3. run show bgp evpn route on DUT-B contains the same DUT-C EVPN routes after reflection |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Peer-group scoped address-family activation |

##### 1.1.1.2 Verify peer-group ipv4-vpn deactivation withdraws EVPN routes

| Field | Content |
| --- | --- |
| Test Name | MOD1-02: Peer-group ipv4-vpn Deactivation |
| Purpose Of The Test | Verify that disabling set protocols bgp peer-group RR-PG ipv4-vpn activate false withdraws DUT-C advertised routes while keeping the EVPN peer session up. |
| Test Topo & Precondition | Topology: Same as MOD1-01Preconditions:1. MOD1-01 completed and DUT-A or DUT-B already receives DUT-C EVPN routes2. DUT-C EVPN peering remains established before the configuration changeConfiguration:DUT-C:set protocols bgp peer-group RR-PG ipv4-vpn activate falsecommit |
| Test Procedure | 1. Configure DUT-C with ipv4-vpn activate false under RR-PG2. Wait 30 seconds for route withdrawal3. Execute run show bgp evpn summary on DUT-A4. Execute run show bgp evpn route on DUT-A5. Execute run show bgp evpn route on DUT-B |
| Expected Results | 1. run show bgp evpn summary on DUT-A still shows the EVPN peer to DUT-C established2. run show bgp evpn route on DUT-A no longer contains the DUT-C EVPN routes affected by this address-family deactivation3. run show bgp evpn route on DUT-B shows the same withdrawal result |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Address-family disable should withdraw routes, not tear down the peer session |

##### 1.1.1.3 Verify peer-group ipv6-vpn activation advertises IPv6 tenant reachability

| Field | Content |
| --- | --- |
| Test Name | MOD1-03: Peer-group ipv6-vpn Activation |
| Purpose Of The Test | Verify that enabling set protocols bgp peer-group RR-PG ipv6-vpn activate true allows Leaf1 to advertise tenant IPv6 reachability through the EVPN fabric. |
| Test Topo & Precondition | Topology: Same as MOD1-01Preconditions:1. Four-switch EVPN fabric is stable2. DUT-C has tenant IPv6 reachability available for advertisement3. ipv6-vpn under RR-PG is disabled before the testConfiguration:DUT-C:set protocols bgp peer-group RR-PG ipv6-vpn activate truecommit |
| Test Procedure | 1. Configure DUT-C with ipv6-vpn activate true under RR-PG2. Wait 30 seconds for route advertisement3. Execute run show bgp evpn summary on DUT-A4. Execute run show bgp evpn route on DUT-A5. Execute run show route ipv6 on DUT-D |
| Expected Results | 1. run show bgp evpn summary on DUT-A shows DUT-C EVPN peer established2. run show bgp evpn route on DUT-A contains DUT-C EVPN routes after the new address-family is activated3. run show route ipv6 on DUT-D contains the IPv6 tenant reachability learned through the fabric |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | IPv6 address-family activation behavior |

#### 1.1.2 Next-Hop Rewrite

##### 1.1.2.1 Verify next-hop-self rewrites the advertised next-hop

| Field | Content |
| --- | --- |
| Test Name | MOD1-04: next-hop-self Rewrites Advertised Next-Hop |
| Purpose Of The Test | Verify that set protocols bgp peer-group RR-PG ipv4-vpn next-hop-self rewrites the next-hop attribute of the advertised route to the local DUT-C address. |
| Test Topo & Precondition | Topology: Same as MOD1-01Preconditions:1. DUT-C already advertises EVPN routes through RR-PG2. DUT-A can display the current route next-hop before the configuration changeConfiguration:DUT-C:set protocols bgp peer-group RR-PG ipv4-vpn next-hop-selfcommit |
| Test Procedure | 1. Execute run show bgp evpn route detail on DUT-A and record the route next-hop from DUT-C2. Configure DUT-C with ipv4-vpn next-hop-self under RR-PG3. Wait 30 seconds for route update4. Execute run show bgp evpn route detail on DUT-A |
| Expected Results | 1. run show bgp evpn route detail on DUT-A shows the route next-hop changed to DUT-C local address2. The EVPN route remains present and valid after the attribute change |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Peer-group scoped next-hop rewrite |

##### 1.1.2.2 Verify next-hop-self force overrides the existing next-hop

| Field | Content |
| --- | --- |
| Test Name | MOD1-05: next-hop-self force Overrides Existing Next-Hop |
| Purpose Of The Test | Verify that set protocols bgp peer-group RR-PG ipv4-vpn next-hop-self force rewrites the next-hop even when the route already carries another next-hop value. |
| Test Topo & Precondition | Topology: Same as MOD1-01Preconditions:1. DUT-C reflects or re-advertises a route whose current next-hop is not DUT-C local address2. DUT-A and DUT-B can display the route before the changeConfiguration:DUT-C:set protocols bgp peer-group RR-PG ipv4-vpn next-hop-self forcecommit |
| Test Procedure | 1. Execute run show bgp evpn route detail on DUT-A and record the current next-hop2. Configure DUT-C with ipv4-vpn next-hop-self force under RR-PG3. Wait 30 seconds for route update4. Execute run show bgp evpn route detail on DUT-A5. Execute run show bgp evpn route detail on DUT-B |
| Expected Results | 1. run show bgp evpn route detail on DUT-A shows the next-hop rewritten to DUT-C local address2. run show bgp evpn route detail on DUT-B shows the same rewrite result3. The original next-hop is no longer selected for the affected route |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Forced next-hop rewrite behavior |

##### 1.1.2.3 Verify removing next-hop-self restores the original next-hop

| Field | Content |
| --- | --- |
| Test Name | MOD1-06: Remove next-hop-self and Restore Original Next-Hop |
| Purpose Of The Test | Verify that removing next-hop-self returns route advertisement to its original next-hop behavior without resetting the session. |
| Test Topo & Precondition | Topology: Same as MOD1-01Preconditions:1. MOD1-04 or MOD1-05 has completed successfully2. DUT-A currently shows rewritten next-hop for DUT-C routeConfiguration:DUT-C:delete protocols bgp peer-group RR-PG ipv4-vpn next-hop-selfcommit |
| Test Procedure | 1. Configure DUT-C to remove ipv4-vpn next-hop-self under RR-PG2. Wait 30 seconds for route refresh3. Execute run show bgp evpn route detail on DUT-A4. Execute run show bgp evpn summary on DUT-A |
| Expected Results | 1. DUT-A shows the affected route with its original non-rewritten next-hop2. The EVPN peer state to DUT-C remains established throughout the change3. The route remains present after the attribute update |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P1 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Attribute rollback behavior |

### 1.2 Prefix Limits & Conditional Advertisement (MODULE 2)

RFC Sections: RFC 4271 route advertisement and policy control
Priority: P0
Cases: 7

#### 1.2.1 Prefix Limits

##### 1.2.1.1 Verify maximum-prefix allows routes below threshold

| Field | Content |
| --- | --- |
| Test Name | MOD2-01: maximum-prefix Below Threshold |
| Purpose Of The Test | Verify that inbound VRF routes are received normally when the advertised route count is below the configured maximum-prefix threshold. |
| Test Topo & Precondition | Topology:DUT-C (SW3, Leaf1, VRF TENANT-A) ---- DUT-D (SW4, Leaf2, VRF TENANT-A)fabric control plane via DUT-A and DUT-B remains stablePreconditions:1. DUT-C and DUT-D are established BGP neighbors in vrf TENANT-A2. DUT-D advertises 9 IPv4 tenant routes to DUT-C3. Peer-group EDGE-PG already exists in vrf TENANT-A on DUT-CConfiguration:DUT-C:set protocols bgp vrf TENANT-A peer-group EDGE-PG ipv4-unicast maximum-prefix 10commit |
| Test Procedure | 1. Configure DUT-C with maximum-prefix 10 on EDGE-PG2. Wait 30 seconds for route update stabilization3. Execute run show route ipv4 on DUT-C |
| Expected Results | 1. run show route ipv4 on DUT-C contains all 9 tenant routes received from DUT-D2. No visible prefix-limit protection behavior is triggered |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Inbound prefix limit normal case |

##### 1.2.1.2 Verify maximum-prefix protects when received routes exceed threshold

| Field | Content |
| --- | --- |
| Test Name | MOD2-02: maximum-prefix Over Threshold |
| Purpose Of The Test | Verify that the device triggers visible protection behavior when the number of received routes exceeds the configured maximum-prefix value. |
| Test Topo & Precondition | Topology: Same as MOD2-01Preconditions:1. DUT-D is prepared to advertise 11 IPv4 tenant routes to DUT-C2. DUT-C can display the route count before and after the changeConfiguration:DUT-C:set protocols bgp vrf TENANT-A peer-group EDGE-PG ipv4-unicast maximum-prefix 10commit |
| Test Procedure | 1. Configure DUT-C with maximum-prefix 10 on EDGE-PG2. Wait 30 seconds for prefix-limit evaluation3. Execute run show route ipv4 on DUT-C |
| Expected Results | 1. DUT-C does not continue to accept the full 11-route set as a normal below-threshold case2. A visible protection effect appears when the threshold is exceeded |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Over-limit behavior |

##### 1.2.1.3 Verify maximum-prefix-out limits advertised routes

| Field | Content |
| --- | --- |
| Test Name | MOD2-03: maximum-prefix-out Limits Advertised Routes |
| Purpose Of The Test | Verify that outbound route advertisement is limited by the configured maximum-prefix-out value. |
| Test Topo & Precondition | Topology: Same as MOD2-01Preconditions:1. DUT-C can advertise 11 IPv4 tenant routes to DUT-D2. DUT-D can display the routes learned from DUT-CConfiguration:DUT-C:set protocols bgp vrf TENANT-A peer-group EDGE-PG ipv4-unicast maximum-prefix-out 10commit |
| Test Procedure | 1. Configure DUT-C with maximum-prefix-out 10 on EDGE-PG2. Wait 30 seconds for outbound update refresh3. Execute run show route ipv4 on DUT-D |
| Expected Results | 1. run show route ipv4 on DUT-D learns no more than 10 tenant routes from DUT-C2. Routes beyond the configured limit are not visible on DUT-D |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Outbound route limit behavior |

#### 1.2.2 Conditional Advertisement

##### 1.2.2.1 Verify conditional advertisement with exist-map when trigger route exists

| Field | Content |
| --- | --- |
| Test Name | MOD2-04: exist-map Positive Case |
| Purpose Of The Test | Verify that the route in advertise-map is advertised only when the trigger route matched by exist-map exists. |
| Test Topo & Precondition | Topology: Same as MOD2-01Preconditions:1. DUT-C has route-map EXIST-MAP and ADV-MAP defined2. DUT-C has a trigger route whose next-hop is 192.0.2.43. DUT-D can display the advertised tenant route from DUT-CConfiguration:DUT-C:set routing route-map EXIST-MAP order 10 match ipv4-addr next-hop address 192.0.2.4set protocols bgp vrf TENANT-A neighbor 10.0.34.4 ipv4-unicast conditional-advertise advertise-map ADV-MAP exist-map EXIST-MAPcommit |
| Test Procedure | 1. Configure DUT-C route-map EXIST-MAP to match next-hop 192.0.2.42. Configure DUT-C conditional advertisement with advertise-map ADV-MAP exist-map EXIST-MAP3. Wait 30 seconds for policy evaluation4. Execute run show route ipv4 on DUT-D |
| Expected Results | 1. run show route ipv4 on DUT-D contains the route controlled by ADV-MAP2. The route is present only when the trigger route exists on DUT-C |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | exist-map positive trigger path |

##### 1.2.2.2 Verify conditional advertisement with exist-map when trigger route is missing

| Field | Content |
| --- | --- |
| Test Name | MOD2-05: exist-map Negative Case |
| Purpose Of The Test | Verify that the route in advertise-map is suppressed when the trigger route for exist-map is absent or does not match. |
| Test Topo & Precondition | Topology: Same as MOD2-01Preconditions:1. DUT-C has route-map EXIST-MAP and ADV-MAP defined2. The trigger route is absent or does not match next-hop 192.0.2.4Configuration:DUT-C:set routing route-map EXIST-MAP order 10 match ipv4-addr next-hop address 192.0.2.4set protocols bgp vrf TENANT-A neighbor 10.0.34.4 ipv4-unicast conditional-advertise advertise-map ADV-MAP exist-map EXIST-MAPcommit |
| Test Procedure | 1. Ensure the trigger route is absent or unmatched on DUT-C2. Configure DUT-C conditional advertisement with exist-map EXIST-MAP3. Wait 30 seconds for policy evaluation4. Execute run show route ipv4 on DUT-D |
| Expected Results | 1. run show route ipv4 on DUT-D does not contain the route controlled by ADV-MAP2. The neighbor session remains established while the route stays suppressed |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | exist-map negative trigger path |

##### 1.2.2.3 Verify conditional advertisement with non-exist-map when trigger route is missing

| Field | Content |
| --- | --- |
| Test Name | MOD2-06: non-exist-map Positive Case |
| Purpose Of The Test | Verify that the route in advertise-map is advertised when the trigger route for non-exist-map is absent. |
| Test Topo & Precondition | Topology: Same as MOD2-01Preconditions:1. DUT-C has route-map NONEXIST-MAP and ADV-MAP defined2. The trigger route is absent on DUT-C before the test startsConfiguration:DUT-C:set protocols bgp vrf TENANT-A neighbor 10.0.34.4 ipv4-unicast conditional-advertise advertise-map ADV-MAP non-exist-map NONEXIST-MAPcommit |
| Test Procedure | 1. Configure DUT-C conditional advertisement with advertise-map ADV-MAP non-exist-map NONEXIST-MAP2. Wait 30 seconds for policy evaluation3. Execute run show route ipv4 on DUT-D |
| Expected Results | 1. run show route ipv4 on DUT-D contains the route controlled by ADV-MAP2. The route is present only while the trigger route remains absent on DUT-C |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | non-exist-map positive trigger path |

##### 1.2.2.4 Verify route-map next-hop match controls conditional advertisement

| Field | Content |
| --- | --- |
| Test Name | MOD2-07: Route-Map Next-Hop Match Controls Conditional Advertisement |
| Purpose Of The Test | Verify that match ipv4-addr next-hop address changes the result of the conditional advertisement when the trigger route next-hop changes between match and mismatch states. |
| Test Topo & Precondition | Topology: Same as MOD2-01Preconditions:1. DUT-C uses EXIST-MAP with match ipv4-addr next-hop address 192.0.2.42. DUT-D can display the advertised route before and after the trigger changeConfiguration:DUT-C:set routing route-map EXIST-MAP order 10 match ipv4-addr next-hop address 192.0.2.4set protocols bgp vrf TENANT-A neighbor 10.0.34.4 ipv4-unicast conditional-advertise advertise-map ADV-MAP exist-map EXIST-MAPcommit |
| Test Procedure | 1. Configure DUT-C trigger route to match next-hop 192.0.2.42. Wait 30 seconds3. Execute run show route ipv4 on DUT-D4. Change the trigger route next-hop on DUT-C so it no longer matches 192.0.2.45. Wait 30 seconds6. Execute run show route ipv4 on DUT-D |
| Expected Results | 1. In the match state, DUT-D contains the route controlled by ADV-MAP2. In the mismatch state, DUT-D no longer contains the route controlled by ADV-MAP |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P1 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Next-hop route-map trigger behavior |

### 1.3 First-AS Enforcement & Graceful Shutdown (MODULE 3)

RFC Sections: RFC 4271 AS_PATH validation and policy behavior, RFC 8326 graceful BGP maintenance behavior
Priority: P1
Cases: 6

#### 1.3.1 First-AS Enforcement

##### 1.3.1.1 Verify neighbor enforce-first-as rejects an invalid AS_PATH

| Field | Content |
| --- | --- |
| Test Name | MOD3-01: Neighbor enforce-first-as Enabled |
| Purpose Of The Test | Verify that DUT-C rejects a route when the leading AS in AS_PATH does not match the configured neighbor AS and neighbor enforce-first-as is enabled. |
| Test Topo & Precondition | Topology: Same as MOD2-01Preconditions:1. DUT-C and DUT-D are established neighbors in vrf TENANT-A2. DUT-D can advertise a route whose leading AS is intentionally invalid for DUT-C policyConfiguration:DUT-C:set protocols bgp vrf TENANT-A neighbor 10.0.34.4 enforce-first-as enable truecommit |
| Test Procedure | 1. Configure DUT-C with enforce-first-as enable true for DUT-D neighbor2. Wait 30 seconds for policy enforcement3. Execute run show route ipv4 on DUT-C |
| Expected Results | 1. run show route ipv4 on DUT-C does not contain the route with the invalid leading AS2. The DUT-C to DUT-D session remains established while the invalid route is rejected |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P1 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Neighbor-level first-AS policy |

##### 1.3.1.2 Verify neighbor enforce-first-as disabled allows an invalid AS_PATH

| Field | Content |
| --- | --- |
| Test Name | MOD3-02: Neighbor enforce-first-as Disabled |
| Purpose Of The Test | Verify that DUT-C accepts the same route after enforce-first-as is disabled on the neighbor. |
| Test Topo & Precondition | Topology: Same as MOD2-01Preconditions:1. DUT-D advertises the same invalid leading-AS route used in MOD3-012. DUT-C can observe that the route is absent before the policy is disabledConfiguration:DUT-C:set protocols bgp vrf TENANT-A neighbor 10.0.34.4 enforce-first-as enable falsecommit |
| Test Procedure | 1. Configure DUT-C with enforce-first-as enable false for DUT-D neighbor2. Wait 30 seconds for policy update3. Execute run show route ipv4 on DUT-C |
| Expected Results | 1. run show route ipv4 on DUT-C contains the route that was previously rejected in MOD3-01 |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P1 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Neighbor-level disable behavior |

##### 1.3.1.3 Verify peer-group enforce-first-as policy is inherited by member neighbors

| Field | Content |
| --- | --- |
| Test Name | MOD3-03: Peer-group enforce-first-as Inheritance |
| Purpose Of The Test | Verify that enforce-first-as configured on peer-group EDGE-PG is applied to member neighbors consistently. |
| Test Topo & Precondition | Topology: Same as MOD2-01Preconditions:1. DUT-D is a member of peer-group EDGE-PG in vrf TENANT-A on DUT-C2. DUT-D advertises a route whose leading AS is invalid for policy validationConfiguration:DUT-C:set protocols bgp vrf TENANT-A peer-group EDGE-PG enforce-first-as enable truecommit |
| Test Procedure | 1. Configure DUT-C peer-group EDGE-PG with enforce-first-as enable true2. Wait 30 seconds for policy inheritance3. Execute run show route ipv4 on DUT-C |
| Expected Results | 1. run show route ipv4 on DUT-C does not contain the invalid leading-AS route learned from the member neighbor2. The observed behavior matches the direct-neighbor policy in MOD3-01 |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P1 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Peer-group inheritance behavior |

#### 1.3.2 Graceful Shutdown

##### 1.3.2.1 Verify neighbor graceful-shutdown drains the preferred path

| Field | Content |
| --- | --- |
| Test Name | MOD3-04: Neighbor Graceful Shutdown Enable |
| Purpose Of The Test | Verify that enabling neighbor graceful shutdown causes the route learned from that neighbor to lose preference while an alternate path remains available. |
| Test Topo & Precondition | Topology:DUT-C has two available paths to a tenant destination:Primary path via DUT-DBackup path via the EVPN fabric through DUT-A and DUT-BPreconditions:1. DUT-C currently prefers a route learned from DUT-D2. An alternate path exists before the test startsConfiguration:DUT-C:set protocols bgp vrf TENANT-A neighbor 10.0.34.4 graceful-shutdown enabled truecommit |
| Test Procedure | 1. Execute run show route ipv4 on DUT-C and record the current preferred path2. Configure DUT-C with graceful-shutdown enabled true for DUT-D neighbor3. Wait 30 seconds for best-path re-evaluation4. Execute run show route ipv4 on DUT-C |
| Expected Results | 1. run show route ipv4 on DUT-C no longer prefers the route learned from DUT-D2. DUT-C selects the alternate path instead of losing tenant reachability |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P1 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Neighbor-level maintenance drain |

##### 1.3.2.2 Verify disabling neighbor graceful-shutdown restores path selection

| Field | Content |
| --- | --- |
| Test Name | MOD3-05: Neighbor Graceful Shutdown Disable |
| Purpose Of The Test | Verify that disabling neighbor graceful shutdown allows the original path to participate in best-path selection again. |
| Test Topo & Precondition | Topology: Same as MOD3-04Preconditions:1. MOD3-04 completed and DUT-C currently prefers the alternate path2. DUT-D path remains otherwise healthyConfiguration:DUT-C:set protocols bgp vrf TENANT-A neighbor 10.0.34.4 graceful-shutdown enabled falsecommit |
| Test Procedure | 1. Configure DUT-C with graceful-shutdown enabled false for DUT-D neighbor2. Wait 30 seconds for best-path re-evaluation3. Execute run show route ipv4 on DUT-C |
| Expected Results | 1. run show route ipv4 on DUT-C allows the DUT-D path to participate in normal selection again2. The preferred path is restored if DUT-D path is otherwise best |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P1 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Neighbor-level recovery behavior |

##### 1.3.2.3 Verify peer-group graceful-shutdown affects all member neighbors

| Field | Content |
| --- | --- |
| Test Name | MOD3-06: Peer-group Graceful Shutdown |
| Purpose Of The Test | Verify that enabling graceful shutdown on peer-group EDGE-PG affects all member neighbors in the group. |
| Test Topo & Precondition | Topology:DUT-C peers with multiple members under EDGE-PG in vrf TENANT-Aall peers remain reachable through the four-switch fabricPreconditions:1. At least two active member neighbors exist under EDGE-PG2. DUT-C can display route preference for prefixes learned through those membersConfiguration:DUT-C:set protocols bgp vrf TENANT-A peer-group EDGE-PG graceful-shutdown enabled truecommit |
| Test Procedure | 1. Execute run show route ipv4 on DUT-C and record route preference for prefixes learned through EDGE-PG members2. Configure DUT-C with graceful-shutdown enabled true on peer-group EDGE-PG3. Wait 30 seconds for policy propagation4. Execute run show route ipv4 on DUT-C |
| Expected Results | 1. run show route ipv4 on DUT-C shows routes learned through EDGE-PG members uniformly deprioritized or drained2. The result affects all member neighbors instead of a single neighbor only |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P2 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Peer-group maintenance drain |