# BGP EVPN New CLI Features — Functional Test Cases (Part 1: MOD-01 to MOD-03)

| Item | Detail |
| --- | --- |
| Platform | PicOS Switches |
| Feature | BGP EVPN New CLI Commands |
| Version | PicOS 4.6 |
| Generation Date | 2026-03-31 |
| Total Cases (This Part) | 25 |

## Table of Contents

- FUNCTIONAL TESTING 1.1 VPN Address Family Activation & Next-Hop-Self (MOD-01) 1.1.1 VPN Address Family Activation 1.1.1.1 ~ 1.1.1.5 1.1.2 VPN Next-Hop-Self 1.1.2.1 ~ 1.1.2.5 1.2 Maximum Prefix Control (MOD-02) 1.2.1 Maximum Prefix Inbound 1.2.1.1 ~ 1.2.1.4 1.2.2 Maximum Prefix Outbound 1.2.2.1 ~ 1.2.2.3 1.3 Conditional Advertisement (MOD-03) 1.3.1 Exist-Map Conditional Advertisement 1.3.1.1 ~ 1.3.1.4 1.3.2 Non-Exist-Map Conditional Advertisement 1.3.2.1 ~ 1.3.2.4

## 1. FUNCTIONAL TESTING

### 1.1 VPN Address Family Activation & Next-Hop-Self (MOD-01)

#### 1.1.1 VPN Address Family Activation

#### 1.1.1.1 Verify activating ipv4-vpn address family on peer-group enables VPNv4 route exchange

| Field | Content |
| --- | --- |
| Test Name | MOD01-01: Activate ipv4-vpn on peer-group |
| Purpose Of The Test | Verify that enabling ipv4-vpn address family on the LEAF peer-group on a Route Reflector allows VPNv4 AFI/SAFI negotiation with leaf peers |
| Test Topo & Precondition | Topology:Leaf-1(DUT-A) ---- Spine-1(DUT-C,RR) ---- Leaf-2(DUT-B)Leaf-1(DUT-A) ---- Spine-2(DUT-D,RR) ---- Leaf-2(DUT-B)Loopbacks: DUT-A:10.0.0.1, DUT-B:10.0.0.2, DUT-C:10.0.0.3, DUT-D:10.0.0.4DUT-A:ge-1/1/1(10.1.1.0/31)--DUT-C:ge-1/1/1(10.1.1.1/31)DUT-A:ge-1/1/2(10.1.2.0/31)--DUT-D:ge-1/1/1(10.1.2.1/31)DUT-B:ge-1/1/1(10.1.3.0/31)--DUT-C:ge-1/1/2(10.1.3.1/31)DUT-B:ge-1/1/2(10.1.4.0/31)--DUT-D:ge-1/1/2(10.1.4.1/31)All AS 65000, DUT-C/DUT-D are Route ReflectorsPreconditions:1. All four DUTs are booted with base configuration applied2. OSPF adjacencies are established on all underlay links and loopbacks are mutually reachable3. iBGP EVPN sessions are established between leaves and spines via loopbacksConfiguration:DUT-A:set l3-interface loopback lo address 10.0.0.1 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.0 prefix-length 31set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.1set protocols ospf network 10.0.0.1/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.1set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueDUT-B:set l3-interface loopback lo address 10.0.0.2 prefix-length 32set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.0 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.2set protocols ospf network 10.0.0.2/32 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.2set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueDUT-C:set l3-interface loopback lo address 10.0.0.3 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.1 prefix-length 31set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.3set protocols ospf network 10.0.0.3/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.3set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate trueset protocols bgp peer-group LEAF ipv4-vpn activate trueDUT-D:set l3-interface loopback lo address 10.0.0.4 prefix-length 32set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.1 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.4set protocols ospf network 10.0.0.4/32 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.4set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate trueset protocols bgp peer-group LEAF ipv4-vpn activate true |
| Test Procedure | 1. Configure set protocols bgp peer-group LEAF ipv4-vpn activate true on DUT-C2. Configure set protocols bgp peer-group LEAF ipv4-vpn activate true on DUT-D3. Wait 30 seconds for BGP sessions to reconverge4. Execute run show bgp ipv4 vpn summary on DUT-C5. Execute run show bgp ipv4 vpn summary on DUT-A6. Execute run show bgp neighbor 10.0.0.1 on DUT-C |
| Expected Results | 1. run show bgp ipv4 vpn summary on DUT-C shows neighbors 10.0.0.1 and 10.0.0.2 with VPNv4 AFI/SAFI negotiated and state as Established2. run show bgp ipv4 vpn summary on DUT-A shows neighbors 10.0.0.3 and 10.0.0.4 with VPNv4 AFI/SAFI negotiated and state as Established3. run show bgp neighbor 10.0.0.1 on DUT-C displays ipv4-vpn address family as active in the capability negotiation section |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Validates new CLI command: set protocols bgp peer-group ipv4-vpn activate true |

#### 1.1.1.2 Verify activating ipv6-vpn address family on peer-group enables VPNv6 route exchange

| Field | Content |
| --- | --- |
| Test Name | MOD01-02: Activate ipv6-vpn on peer-group |
| Purpose Of The Test | Verify that enabling ipv6-vpn address family on the LEAF peer-group on a Route Reflector allows VPNv6 AFI/SAFI negotiation with leaf peers |
| Test Topo & Precondition | Topology:Leaf-1(DUT-A) ---- Spine-1(DUT-C,RR) ---- Leaf-2(DUT-B)Leaf-1(DUT-A) ---- Spine-2(DUT-D,RR) ---- Leaf-2(DUT-B)Loopbacks: DUT-A:10.0.0.1, DUT-B:10.0.0.2, DUT-C:10.0.0.3, DUT-D:10.0.0.4DUT-A:ge-1/1/1(10.1.1.0/31)--DUT-C:ge-1/1/1(10.1.1.1/31)DUT-A:ge-1/1/2(10.1.2.0/31)--DUT-D:ge-1/1/1(10.1.2.1/31)DUT-B:ge-1/1/1(10.1.3.0/31)--DUT-C:ge-1/1/2(10.1.3.1/31)DUT-B:ge-1/1/2(10.1.4.0/31)--DUT-D:ge-1/1/2(10.1.4.1/31)All AS 65000, DUT-C/DUT-D are Route ReflectorsPreconditions:1. All four DUTs are booted with base configuration applied2. OSPF adjacencies are established on all underlay links and loopbacks are mutually reachable3. iBGP EVPN sessions are established between leaves and spines via loopbacksConfiguration:DUT-A:set l3-interface loopback lo address 10.0.0.1 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.0 prefix-length 31set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.1set protocols ospf network 10.0.0.1/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.1set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueDUT-B:set l3-interface loopback lo address 10.0.0.2 prefix-length 32set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.0 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.2set protocols ospf network 10.0.0.2/32 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.2set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueDUT-C:set l3-interface loopback lo address 10.0.0.3 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.1 prefix-length 31set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.3set protocols ospf network 10.0.0.3/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.3set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate trueset protocols bgp peer-group LEAF ipv6-vpn activate trueDUT-D:set l3-interface loopback lo address 10.0.0.4 prefix-length 32set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.1 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.4set protocols ospf network 10.0.0.4/32 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.4set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate trueset protocols bgp peer-group LEAF ipv6-vpn activate true |
| Test Procedure | 1. Configure set protocols bgp peer-group LEAF ipv6-vpn activate true on DUT-C2. Configure set protocols bgp peer-group LEAF ipv6-vpn activate true on DUT-D3. Wait 30 seconds for BGP sessions to reconverge4. Execute run show bgp ipv6 vpn summary on DUT-C5. Execute run show bgp ipv6 vpn summary on DUT-A6. Execute run show bgp neighbor 10.0.0.1 on DUT-C |
| Expected Results | 1. run show bgp ipv6 vpn summary on DUT-C shows neighbors 10.0.0.1 and 10.0.0.2 with VPNv6 AFI/SAFI negotiated and state as Established2. run show bgp ipv6 vpn summary on DUT-A shows neighbors 10.0.0.3 and 10.0.0.4 with VPNv6 AFI/SAFI negotiated and state as Established3. run show bgp neighbor 10.0.0.1 on DUT-C displays ipv6-vpn address family as active in the capability negotiation section |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Validates new CLI command: set protocols bgp peer-group ipv6-vpn activate true |

#### 1.1.1.3 Verify deactivating ipv4-vpn on peer-group stops VPNv4 route exchange

| Field | Content |
| --- | --- |
| Test Name | MOD01-03: Deactivate ipv4-vpn on peer-group |
| Purpose Of The Test | Verify that disabling ipv4-vpn address family on the LEAF peer-group causes VPNv4 routes to be withdrawn and AFI/SAFI to be de-negotiated |
| Test Topo & Precondition | Topology:Leaf-1(DUT-A) ---- Spine-1(DUT-C,RR) ---- Leaf-2(DUT-B)Leaf-1(DUT-A) ---- Spine-2(DUT-D,RR) ---- Leaf-2(DUT-B)Loopbacks: DUT-A:10.0.0.1, DUT-B:10.0.0.2, DUT-C:10.0.0.3, DUT-D:10.0.0.4DUT-A:ge-1/1/1(10.1.1.0/31)--DUT-C:ge-1/1/1(10.1.1.1/31)DUT-A:ge-1/1/2(10.1.2.0/31)--DUT-D:ge-1/1/1(10.1.2.1/31)DUT-B:ge-1/1/1(10.1.3.0/31)--DUT-C:ge-1/1/2(10.1.3.1/31)DUT-B:ge-1/1/2(10.1.4.0/31)--DUT-D:ge-1/1/2(10.1.4.1/31)All AS 65000, DUT-C/DUT-D are Route ReflectorsPreconditions:1. All four DUTs are booted with base configuration applied2. OSPF adjacencies are established on all underlay links and loopbacks are mutually reachable3. iBGP EVPN sessions are established between leaves and spines via loopbacks4. ipv4-vpn is already activated on DUT-C and DUT-D LEAF peer-groupConfiguration:DUT-A:set l3-interface loopback lo address 10.0.0.1 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.0 prefix-length 31set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.1set protocols ospf network 10.0.0.1/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.1set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueDUT-B:set l3-interface loopback lo address 10.0.0.2 prefix-length 32set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.0 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.2set protocols ospf network 10.0.0.2/32 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.2set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueDUT-C:set l3-interface loopback lo address 10.0.0.3 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.1 prefix-length 31set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.3set protocols ospf network 10.0.0.3/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.3set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate trueset protocols bgp peer-group LEAF ipv4-vpn activate trueDUT-D:set l3-interface loopback lo address 10.0.0.4 prefix-length 32set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.1 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.4set protocols ospf network 10.0.0.4/32 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.4set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate trueset protocols bgp peer-group LEAF ipv4-vpn activate true |
| Test Procedure | 1. Execute run show bgp ipv4 vpn summary on DUT-C to record current VPNv4 session state2. Configure set protocols bgp peer-group LEAF ipv4-vpn activate false on DUT-C3. Wait 30 seconds for BGP sessions to reconverge4. Execute run show bgp ipv4 vpn summary on DUT-C5. Execute run show bgp ipv4 vpn summary on DUT-A6. Execute run show bgp neighbor 10.0.0.1 on DUT-C |
| Expected Results | 1. run show bgp ipv4 vpn summary on DUT-C before deactivation shows VPNv4 sessions as Established2. run show bgp ipv4 vpn summary on DUT-C after deactivation shows no VPNv4 sessions with 10.0.0.1 and 10.0.0.2, or sessions without VPNv4 AFI/SAFI3. run show bgp ipv4 vpn summary on DUT-A shows VPNv4 session with 10.0.0.3 is no longer Established for VPNv44. run show bgp neighbor 10.0.0.1 on DUT-C no longer displays ipv4-vpn as an active address family |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Validates new CLI command: set protocols bgp peer-group ipv4-vpn activate false |

#### 1.1.1.4 Verify activating both ipv4-vpn and ipv6-vpn simultaneously on same peer-group

| Field | Content |
| --- | --- |
| Test Name | MOD01-04: Activate both ipv4-vpn and ipv6-vpn on peer-group |
| Purpose Of The Test | Verify that enabling both ipv4-vpn and ipv6-vpn address families simultaneously on the same peer-group results in both VPNv4 and VPNv6 AFI/SAFI being negotiated |
| Test Topo & Precondition | Topology:Leaf-1(DUT-A) ---- Spine-1(DUT-C,RR) ---- Leaf-2(DUT-B)Leaf-1(DUT-A) ---- Spine-2(DUT-D,RR) ---- Leaf-2(DUT-B)Loopbacks: DUT-A:10.0.0.1, DUT-B:10.0.0.2, DUT-C:10.0.0.3, DUT-D:10.0.0.4DUT-A:ge-1/1/1(10.1.1.0/31)--DUT-C:ge-1/1/1(10.1.1.1/31)DUT-A:ge-1/1/2(10.1.2.0/31)--DUT-D:ge-1/1/1(10.1.2.1/31)DUT-B:ge-1/1/1(10.1.3.0/31)--DUT-C:ge-1/1/2(10.1.3.1/31)DUT-B:ge-1/1/2(10.1.4.0/31)--DUT-D:ge-1/1/2(10.1.4.1/31)All AS 65000, DUT-C/DUT-D are Route ReflectorsPreconditions:1. All four DUTs are booted with base configuration applied2. OSPF adjacencies are established on all underlay links and loopbacks are mutually reachable3. iBGP EVPN sessions are established between leaves and spines via loopbacksConfiguration:DUT-A:set l3-interface loopback lo address 10.0.0.1 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.0 prefix-length 31set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.1set protocols ospf network 10.0.0.1/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.1set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueDUT-B:set l3-interface loopback lo address 10.0.0.2 prefix-length 32set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.0 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.2set protocols ospf network 10.0.0.2/32 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.2set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueDUT-C:set l3-interface loopback lo address 10.0.0.3 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.1 prefix-length 31set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.3set protocols ospf network 10.0.0.3/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.3set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate trueset protocols bgp peer-group LEAF ipv4-vpn activate trueset protocols bgp peer-group LEAF ipv6-vpn activate trueDUT-D:set l3-interface loopback lo address 10.0.0.4 prefix-length 32set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.1 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.4set protocols ospf network 10.0.0.4/32 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.4set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate trueset protocols bgp peer-group LEAF ipv4-vpn activate trueset protocols bgp peer-group LEAF ipv6-vpn activate true |
| Test Procedure | 1. Configure set protocols bgp peer-group LEAF ipv4-vpn activate true on DUT-C2. Configure set protocols bgp peer-group LEAF ipv6-vpn activate true on DUT-C3. Configure set protocols bgp peer-group LEAF ipv4-vpn activate true on DUT-D4. Configure set protocols bgp peer-group LEAF ipv6-vpn activate true on DUT-D5. Wait 30 seconds for BGP sessions to reconverge6. Execute run show bgp ipv4 vpn summary on DUT-C7. Execute run show bgp ipv6 vpn summary on DUT-C8. Execute run show bgp neighbor 10.0.0.1 on DUT-C |
| Expected Results | 1. run show bgp ipv4 vpn summary on DUT-C shows neighbors 10.0.0.1 and 10.0.0.2 with VPNv4 AFI/SAFI Established2. run show bgp ipv6 vpn summary on DUT-C shows neighbors 10.0.0.1 and 10.0.0.2 with VPNv6 AFI/SAFI Established3. run show bgp neighbor 10.0.0.1 on DUT-C shows both ipv4-vpn and ipv6-vpn address families as active |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P1 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Validates simultaneous activation of both VPN address families on the same peer-group |

#### 1.1.1.5 Verify deactivating and re-activating ipv4-vpn on peer-group recovers VPN routes

| Field | Content |
| --- | --- |
| Test Name | MOD01-05: Deactivate then re-activate ipv4-vpn on peer-group |
| Purpose Of The Test | Verify that after deactivating ipv4-vpn (routes withdrawn) and then re-activating it, VPNv4 routes are fully recovered |
| Test Topo & Precondition | Topology:Leaf-1(DUT-A) ---- Spine-1(DUT-C,RR) ---- Leaf-2(DUT-B)Leaf-1(DUT-A) ---- Spine-2(DUT-D,RR) ---- Leaf-2(DUT-B)Loopbacks: DUT-A:10.0.0.1, DUT-B:10.0.0.2, DUT-C:10.0.0.3, DUT-D:10.0.0.4DUT-A:ge-1/1/1(10.1.1.0/31)--DUT-C:ge-1/1/1(10.1.1.1/31)DUT-A:ge-1/1/2(10.1.2.0/31)--DUT-D:ge-1/1/1(10.1.2.1/31)DUT-B:ge-1/1/1(10.1.3.0/31)--DUT-C:ge-1/1/2(10.1.3.1/31)DUT-B:ge-1/1/2(10.1.4.0/31)--DUT-D:ge-1/1/2(10.1.4.1/31)All AS 65000, DUT-C/DUT-D are Route ReflectorsPreconditions:1. All four DUTs are booted with base configuration applied2. OSPF adjacencies are established on all underlay links and loopbacks are mutually reachable3. iBGP EVPN sessions are established between leaves and spines via loopbacks4. ipv4-vpn is already activated on DUT-C and DUT-D LEAF peer-groupConfiguration:DUT-A:set l3-interface loopback lo address 10.0.0.1 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.0 prefix-length 31set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.1set protocols ospf network 10.0.0.1/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.1set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueDUT-B:set l3-interface loopback lo address 10.0.0.2 prefix-length 32set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.0 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.2set protocols ospf network 10.0.0.2/32 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.2set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueDUT-C:set l3-interface loopback lo address 10.0.0.3 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.1 prefix-length 31set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.3set protocols ospf network 10.0.0.3/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.3set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate trueset protocols bgp peer-group LEAF ipv4-vpn activate trueDUT-D:set l3-interface loopback lo address 10.0.0.4 prefix-length 32set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.1 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.4set protocols ospf network 10.0.0.4/32 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.4set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate trueset protocols bgp peer-group LEAF ipv4-vpn activate true |
| Test Procedure | 1. Execute run show bgp ipv4 vpn summary on DUT-C to record baseline VPNv4 route count2. Configure set protocols bgp peer-group LEAF ipv4-vpn activate false on DUT-C3. Wait 30 seconds for BGP to reconverge4. Execute run show bgp ipv4 vpn summary on DUT-C5. Configure set protocols bgp peer-group LEAF ipv4-vpn activate true on DUT-C6. Wait 30 seconds for BGP to reconverge7. Execute run show bgp ipv4 vpn summary on DUT-C8. Execute run show bgp ipv4 vpn summary on DUT-A |
| Expected Results | 1. run show bgp ipv4 vpn summary on DUT-C before deactivation shows VPNv4 sessions Established with route counts2. run show bgp ipv4 vpn summary on DUT-C after deactivation shows VPNv4 sessions are no longer Established or have zero VPNv4 routes3. run show bgp ipv4 vpn summary on DUT-C after re-activation shows VPNv4 sessions Established with route counts matching the baseline4. run show bgp ipv4 vpn summary on DUT-A shows VPNv4 session with 10.0.0.3 as Established with routes restored |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P1 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Validates VPN route recovery after address family toggle on peer-group |

#### 1.1.2 VPN Next-Hop-Self

#### 1.1.2.1 Verify next-hop-self for ipv4-vpn changes next-hop to local address on route reflector

| Field | Content |
| --- | --- |
| Test Name | MOD01-06: ipv4-vpn next-hop-self on RR |
| Purpose Of The Test | Verify that configuring next-hop-self for ipv4-vpn on a Route Reflector’s peer-group causes reflected VPNv4 routes to carry the RR’s loopback as the next-hop |
| Test Topo & Precondition | Topology:Leaf-1(DUT-A) ---- Spine-1(DUT-C,RR) ---- Leaf-2(DUT-B)Leaf-1(DUT-A) ---- Spine-2(DUT-D,RR) ---- Leaf-2(DUT-B)Loopbacks: DUT-A:10.0.0.1, DUT-B:10.0.0.2, DUT-C:10.0.0.3, DUT-D:10.0.0.4DUT-A:ge-1/1/1(10.1.1.0/31)--DUT-C:ge-1/1/1(10.1.1.1/31)DUT-A:ge-1/1/2(10.1.2.0/31)--DUT-D:ge-1/1/1(10.1.2.1/31)DUT-B:ge-1/1/1(10.1.3.0/31)--DUT-C:ge-1/1/2(10.1.3.1/31)DUT-B:ge-1/1/2(10.1.4.0/31)--DUT-D:ge-1/1/2(10.1.4.1/31)All AS 65000, DUT-C/DUT-D are Route ReflectorsPreconditions:1. All four DUTs are booted with base configuration applied2. OSPF adjacencies are established on all underlay links and loopbacks are mutually reachable3. iBGP EVPN sessions are established between leaves and spines via loopbacks4. ipv4-vpn is activated on DUT-C and DUT-D LEAF peer-group5. DUT-A originates at least one VPNv4 route for verificationConfiguration:DUT-A:set l3-interface loopback lo address 10.0.0.1 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.0 prefix-length 31set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.1set protocols ospf network 10.0.0.1/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.1set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueset protocols bgp peer-group EVPN-OVERLAY ipv4-vpn activate trueDUT-B:set l3-interface loopback lo address 10.0.0.2 prefix-length 32set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.0 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.2set protocols ospf network 10.0.0.2/32 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.2set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueset protocols bgp peer-group EVPN-OVERLAY ipv4-vpn activate trueDUT-C:set l3-interface loopback lo address 10.0.0.3 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.1 prefix-length 31set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.3set protocols ospf network 10.0.0.3/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.3set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate trueset protocols bgp peer-group LEAF ipv4-vpn activate trueset protocols bgp peer-group LEAF ipv4-vpn next-hop-selfDUT-D:set l3-interface loopback lo address 10.0.0.4 prefix-length 32set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.1 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.4set protocols ospf network 10.0.0.4/32 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.4set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate trueset protocols bgp peer-group LEAF ipv4-vpn activate true |
| Test Procedure | 1. Configure set protocols bgp peer-group LEAF ipv4-vpn next-hop-self on DUT-C2. Wait 30 seconds for BGP to reconverge3. Execute run show bgp ipv4 vpn on DUT-B to inspect VPNv4 routes reflected via DUT-C4. Execute run show bgp ipv4 vpn <route-from-DUT-A> on DUT-B for detailed path information |
| Expected Results | 1. run show bgp ipv4 vpn on DUT-B shows VPNv4 routes originating from DUT-A2. VPNv4 routes reflected via DUT-C (10.0.0.3) on DUT-B display next-hop as 10.0.0.3 (DUT-C loopback) instead of the original next-hop 10.0.0.1 (DUT-A loopback)3. VPNv4 routes reflected via DUT-D (10.0.0.4) on DUT-B retain the original next-hop 10.0.0.1 (DUT-A loopback) since next-hop-self is not configured on DUT-D |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Validates new CLI command: set protocols bgp peer-group ipv4-vpn next-hop-self |

#### 1.1.2.2 Verify next-hop-self for ipv6-vpn changes next-hop to local address

| Field | Content |
| --- | --- |
| Test Name | MOD01-07: ipv6-vpn next-hop-self on RR |
| Purpose Of The Test | Verify that configuring next-hop-self for ipv6-vpn on a Route Reflector’s peer-group causes reflected VPNv6 routes to carry the RR’s loopback as the next-hop |
| Test Topo & Precondition | Topology:Leaf-1(DUT-A) ---- Spine-1(DUT-C,RR) ---- Leaf-2(DUT-B)Leaf-1(DUT-A) ---- Spine-2(DUT-D,RR) ---- Leaf-2(DUT-B)Loopbacks: DUT-A:10.0.0.1, DUT-B:10.0.0.2, DUT-C:10.0.0.3, DUT-D:10.0.0.4DUT-A:ge-1/1/1(10.1.1.0/31)--DUT-C:ge-1/1/1(10.1.1.1/31)DUT-A:ge-1/1/2(10.1.2.0/31)--DUT-D:ge-1/1/1(10.1.2.1/31)DUT-B:ge-1/1/1(10.1.3.0/31)--DUT-C:ge-1/1/2(10.1.3.1/31)DUT-B:ge-1/1/2(10.1.4.0/31)--DUT-D:ge-1/1/2(10.1.4.1/31)All AS 65000, DUT-C/DUT-D are Route ReflectorsPreconditions:1. All four DUTs are booted with base configuration applied2. OSPF adjacencies are established on all underlay links and loopbacks are mutually reachable3. iBGP EVPN sessions are established between leaves and spines via loopbacks4. ipv6-vpn is activated on DUT-C and DUT-D LEAF peer-group5. DUT-A originates at least one VPNv6 route for verificationConfiguration:DUT-A:set l3-interface loopback lo address 10.0.0.1 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.0 prefix-length 31set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.1set protocols ospf network 10.0.0.1/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.1set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueset protocols bgp peer-group EVPN-OVERLAY ipv6-vpn activate trueDUT-B:set l3-interface loopback lo address 10.0.0.2 prefix-length 32set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.0 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.2set protocols ospf network 10.0.0.2/32 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.2set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueset protocols bgp peer-group EVPN-OVERLAY ipv6-vpn activate trueDUT-C:set l3-interface loopback lo address 10.0.0.3 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.1 prefix-length 31set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.3set protocols ospf network 10.0.0.3/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.3set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate trueset protocols bgp peer-group LEAF ipv6-vpn activate trueset protocols bgp peer-group LEAF ipv6-vpn next-hop-selfDUT-D:set l3-interface loopback lo address 10.0.0.4 prefix-length 32set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.1 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.4set protocols ospf network 10.0.0.4/32 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.4set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate trueset protocols bgp peer-group LEAF ipv6-vpn activate true |
| Test Procedure | 1. Configure set protocols bgp peer-group LEAF ipv6-vpn next-hop-self on DUT-C2. Wait 30 seconds for BGP to reconverge3. Execute run show bgp ipv6 vpn on DUT-B to inspect VPNv6 routes reflected via DUT-C4. Execute run show bgp ipv6 vpn <route-from-DUT-A> on DUT-B for detailed path information |
| Expected Results | 1. run show bgp ipv6 vpn on DUT-B shows VPNv6 routes originating from DUT-A2. VPNv6 routes reflected via DUT-C on DUT-B display next-hop as 10.0.0.3 (DUT-C loopback) instead of the original next-hop 10.0.0.13. VPNv6 routes reflected via DUT-D on DUT-B retain the original next-hop 10.0.0.1 since next-hop-self is not configured on DUT-D |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Validates new CLI command: set protocols bgp peer-group ipv6-vpn next-hop-self |

#### 1.1.2.3 Verify next-hop-self force for ipv4-vpn overrides next-hop for all routes including iBGP-learned

| Field | Content |
| --- | --- |
| Test Name | MOD01-08: ipv4-vpn next-hop-self force on RR |
| Purpose Of The Test | Verify that the force option of next-hop-self for ipv4-vpn overrides the next-hop attribute for all reflected VPNv4 routes, including those learned via iBGP from other RR peers |
| Test Topo & Precondition | Topology:Leaf-1(DUT-A) ---- Spine-1(DUT-C,RR) ---- Leaf-2(DUT-B)Leaf-1(DUT-A) ---- Spine-2(DUT-D,RR) ---- Leaf-2(DUT-B)Loopbacks: DUT-A:10.0.0.1, DUT-B:10.0.0.2, DUT-C:10.0.0.3, DUT-D:10.0.0.4DUT-A:ge-1/1/1(10.1.1.0/31)--DUT-C:ge-1/1/1(10.1.1.1/31)DUT-A:ge-1/1/2(10.1.2.0/31)--DUT-D:ge-1/1/1(10.1.2.1/31)DUT-B:ge-1/1/1(10.1.3.0/31)--DUT-C:ge-1/1/2(10.1.3.1/31)DUT-B:ge-1/1/2(10.1.4.0/31)--DUT-D:ge-1/1/2(10.1.4.1/31)All AS 65000, DUT-C/DUT-D are Route ReflectorsPreconditions:1. All four DUTs are booted with base configuration applied2. OSPF adjacencies are established on all underlay links and loopbacks are mutually reachable3. iBGP EVPN sessions are established between leaves and spines via loopbacks4. ipv4-vpn is activated on all DUTs5. DUT-A originates VPNv4 routes that are reflected through both RRsConfiguration:DUT-A:set l3-interface loopback lo address 10.0.0.1 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.0 prefix-length 31set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.1set protocols ospf network 10.0.0.1/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.1set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueset protocols bgp peer-group EVPN-OVERLAY ipv4-vpn activate trueDUT-B:set l3-interface loopback lo address 10.0.0.2 prefix-length 32set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.0 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.2set protocols ospf network 10.0.0.2/32 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.2set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueset protocols bgp peer-group EVPN-OVERLAY ipv4-vpn activate trueDUT-C:set l3-interface loopback lo address 10.0.0.3 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.1 prefix-length 31set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.3set protocols ospf network 10.0.0.3/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.3set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate trueset protocols bgp peer-group LEAF ipv4-vpn activate trueset protocols bgp peer-group LEAF ipv4-vpn next-hop-self forceDUT-D:set l3-interface loopback lo address 10.0.0.4 prefix-length 32set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.1 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.4set protocols ospf network 10.0.0.4/32 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.4set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate trueset protocols bgp peer-group LEAF ipv4-vpn activate true |
| Test Procedure | 1. Configure set protocols bgp peer-group LEAF ipv4-vpn next-hop-self force on DUT-C2. Wait 30 seconds for BGP to reconverge3. Execute run show bgp ipv4 vpn on DUT-B4. Execute run show bgp ipv4 vpn <route-from-DUT-A> on DUT-B for detailed path attributes |
| Expected Results | 1. run show bgp ipv4 vpn on DUT-B shows VPNv4 routes from DUT-A reflected through DUT-C2. All VPNv4 routes reflected via DUT-C display next-hop as 10.0.0.3 regardless of whether they were originally learned via iBGP3. The force keyword overrides next-hop even for iBGP-originated routes, unlike non-force mode which only modifies eBGP-learned routes |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Validates new CLI command: set protocols bgp peer-group ipv4-vpn next-hop-self force. The force option is critical for iBGP-learned routes in RR topologies. |

#### 1.1.2.4 Verify next-hop-self force for ipv6-vpn overrides next-hop for iBGP-learned routes

| Field | Content |
| --- | --- |
| Test Name | MOD01-09: ipv6-vpn next-hop-self force on RR |
| Purpose Of The Test | Verify that the force option of next-hop-self for ipv6-vpn overrides the next-hop attribute for all reflected VPNv6 routes including iBGP-learned ones |
| Test Topo & Precondition | Topology:Leaf-1(DUT-A) ---- Spine-1(DUT-C,RR) ---- Leaf-2(DUT-B)Leaf-1(DUT-A) ---- Spine-2(DUT-D,RR) ---- Leaf-2(DUT-B)Loopbacks: DUT-A:10.0.0.1, DUT-B:10.0.0.2, DUT-C:10.0.0.3, DUT-D:10.0.0.4DUT-A:ge-1/1/1(10.1.1.0/31)--DUT-C:ge-1/1/1(10.1.1.1/31)DUT-A:ge-1/1/2(10.1.2.0/31)--DUT-D:ge-1/1/1(10.1.2.1/31)DUT-B:ge-1/1/1(10.1.3.0/31)--DUT-C:ge-1/1/2(10.1.3.1/31)DUT-B:ge-1/1/2(10.1.4.0/31)--DUT-D:ge-1/1/2(10.1.4.1/31)All AS 65000, DUT-C/DUT-D are Route ReflectorsPreconditions:1. All four DUTs are booted with base configuration applied2. OSPF adjacencies are established on all underlay links and loopbacks are mutually reachable3. iBGP EVPN sessions are established between leaves and spines via loopbacks4. ipv6-vpn is activated on all DUTs5. DUT-A originates VPNv6 routes that are reflected through both RRsConfiguration:DUT-A:set l3-interface loopback lo address 10.0.0.1 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.0 prefix-length 31set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.1set protocols ospf network 10.0.0.1/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.1set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueset protocols bgp peer-group EVPN-OVERLAY ipv6-vpn activate trueDUT-B:set l3-interface loopback lo address 10.0.0.2 prefix-length 32set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.0 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.2set protocols ospf network 10.0.0.2/32 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.2set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueset protocols bgp peer-group EVPN-OVERLAY ipv6-vpn activate trueDUT-C:set l3-interface loopback lo address 10.0.0.3 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.1 prefix-length 31set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.3set protocols ospf network 10.0.0.3/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.3set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate trueset protocols bgp peer-group LEAF ipv6-vpn activate trueset protocols bgp peer-group LEAF ipv6-vpn next-hop-self forceDUT-D:set l3-interface loopback lo address 10.0.0.4 prefix-length 32set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.1 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.4set protocols ospf network 10.0.0.4/32 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.4set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate trueset protocols bgp peer-group LEAF ipv6-vpn activate true |
| Test Procedure | 1. Configure set protocols bgp peer-group LEAF ipv6-vpn next-hop-self force on DUT-C2. Wait 30 seconds for BGP to reconverge3. Execute run show bgp ipv6 vpn on DUT-B4. Execute run show bgp ipv6 vpn <route-from-DUT-A> on DUT-B for detailed path attributes |
| Expected Results | 1. run show bgp ipv6 vpn on DUT-B shows VPNv6 routes from DUT-A reflected through DUT-C2. All VPNv6 routes reflected via DUT-C display next-hop as 10.0.0.3 regardless of whether they were originally learned via iBGP3. The force keyword overrides next-hop even for iBGP-originated VPNv6 routes |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P1 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Validates new CLI command: set protocols bgp peer-group ipv6-vpn next-hop-self force |

#### 1.1.2.5 Verify removing next-hop-self from ipv4-vpn peer-group restores original next-hop

| Field | Content |
| --- | --- |
| Test Name | MOD01-10: Remove ipv4-vpn next-hop-self restores original NH |
| Purpose Of The Test | Verify that after removing next-hop-self configuration for ipv4-vpn from the peer-group, the reflected VPNv4 routes revert to carrying the original originator’s next-hop |
| Test Topo & Precondition | Topology:Leaf-1(DUT-A) ---- Spine-1(DUT-C,RR) ---- Leaf-2(DUT-B)Leaf-1(DUT-A) ---- Spine-2(DUT-D,RR) ---- Leaf-2(DUT-B)Loopbacks: DUT-A:10.0.0.1, DUT-B:10.0.0.2, DUT-C:10.0.0.3, DUT-D:10.0.0.4DUT-A:ge-1/1/1(10.1.1.0/31)--DUT-C:ge-1/1/1(10.1.1.1/31)DUT-A:ge-1/1/2(10.1.2.0/31)--DUT-D:ge-1/1/1(10.1.2.1/31)DUT-B:ge-1/1/1(10.1.3.0/31)--DUT-C:ge-1/1/2(10.1.3.1/31)DUT-B:ge-1/1/2(10.1.4.0/31)--DUT-D:ge-1/1/2(10.1.4.1/31)All AS 65000, DUT-C/DUT-D are Route ReflectorsPreconditions:1. All four DUTs are booted with base configuration applied2. OSPF adjacencies are established on all underlay links and loopbacks are mutually reachable3. iBGP EVPN sessions are established between leaves and spines via loopbacks4. ipv4-vpn is activated on DUT-C LEAF peer-group with next-hop-self already configured5. DUT-A originates VPNv4 routes for verificationConfiguration:DUT-A:set l3-interface loopback lo address 10.0.0.1 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.0 prefix-length 31set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.1set protocols ospf network 10.0.0.1/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.1set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueset protocols bgp peer-group EVPN-OVERLAY ipv4-vpn activate trueDUT-B:set l3-interface loopback lo address 10.0.0.2 prefix-length 32set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.0 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.2set protocols ospf network 10.0.0.2/32 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.2set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueset protocols bgp peer-group EVPN-OVERLAY ipv4-vpn activate trueDUT-C:set l3-interface loopback lo address 10.0.0.3 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.1 prefix-length 31set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.3set protocols ospf network 10.0.0.3/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.3set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate trueset protocols bgp peer-group LEAF ipv4-vpn activate trueset protocols bgp peer-group LEAF ipv4-vpn next-hop-selfDUT-D:set l3-interface loopback lo address 10.0.0.4 prefix-length 32set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.1 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.4set protocols ospf network 10.0.0.4/32 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.4set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate trueset protocols bgp peer-group LEAF ipv4-vpn activate true |
| Test Procedure | 1. Execute run show bgp ipv4 vpn <route-from-DUT-A> on DUT-B to record current next-hop for routes reflected via DUT-C2. Remove delete protocols bgp peer-group LEAF ipv4-vpn next-hop-self on DUT-C3. Wait 30 seconds for BGP to reconverge4. Execute run show bgp ipv4 vpn <route-from-DUT-A> on DUT-B |
| Expected Results | 1. run show bgp ipv4 vpn <route> on DUT-B before removal shows next-hop as 10.0.0.3 for routes via DUT-C2. run show bgp ipv4 vpn <route> on DUT-B after removal shows next-hop reverted to 10.0.0.1 (original DUT-A loopback) for routes via DUT-C |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P1 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Validates that removal of next-hop-self configuration properly restores the original next-hop behavior |

### 1.2 Maximum Prefix Control (MOD-02)

#### 1.2.1 Maximum Prefix Inbound

#### 1.2.1.1 Verify configuring maximum-prefix for ipv4-unicast on peer-group

| Field | Content |
| --- | --- |
| Test Name | MOD02-01: ipv4-unicast maximum-prefix on peer-group |
| Purpose Of The Test | Verify that configuring maximum-prefix for ipv4-unicast on a peer-group correctly applies the inbound prefix limit to all peers in that group |
| Test Topo & Precondition | Topology:Leaf-1(DUT-A) ---- Spine-1(DUT-C,RR) ---- Leaf-2(DUT-B)Leaf-1(DUT-A) ---- Spine-2(DUT-D,RR) ---- Leaf-2(DUT-B)Loopbacks: DUT-A:10.0.0.1, DUT-B:10.0.0.2, DUT-C:10.0.0.3, DUT-D:10.0.0.4DUT-A:ge-1/1/1(10.1.1.0/31)--DUT-C:ge-1/1/1(10.1.1.1/31)DUT-A:ge-1/1/2(10.1.2.0/31)--DUT-D:ge-1/1/1(10.1.2.1/31)DUT-B:ge-1/1/1(10.1.3.0/31)--DUT-C:ge-1/1/2(10.1.3.1/31)DUT-B:ge-1/1/2(10.1.4.0/31)--DUT-D:ge-1/1/2(10.1.4.1/31)All AS 65000, DUT-C/DUT-D are Route ReflectorsPreconditions:1. All four DUTs are booted with base configuration applied2. OSPF adjacencies are established on all underlay links and loopbacks are mutually reachable3. iBGP EVPN sessions are established between leaves and spines via loopbacksConfiguration:DUT-A:set l3-interface loopback lo address 10.0.0.1 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.0 prefix-length 31set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.1set protocols ospf network 10.0.0.1/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.1set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueset protocols bgp peer-group EVPN-OVERLAY ipv4-unicast maximum-prefix 100DUT-B:set l3-interface loopback lo address 10.0.0.2 prefix-length 32set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.0 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.2set protocols ospf network 10.0.0.2/32 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.2set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueDUT-C:set l3-interface loopback lo address 10.0.0.3 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.1 prefix-length 31set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.3set protocols ospf network 10.0.0.3/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.3set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate trueDUT-D:set l3-interface loopback lo address 10.0.0.4 prefix-length 32set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.1 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.4set protocols ospf network 10.0.0.4/32 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.4set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate true |
| Test Procedure | 1. Configure set protocols bgp peer-group EVPN-OVERLAY ipv4-unicast maximum-prefix 100 on DUT-A2. Wait 10 seconds for configuration to take effect3. Execute run show bgp neighbor 10.0.0.3 on DUT-A4. Execute run show bgp neighbor 10.0.0.4 on DUT-A5. Execute run show running-config protocols bgp on DUT-A |
| Expected Results | 1. run show bgp neighbor 10.0.0.3 on DUT-A displays maximum-prefix limit as 100 for ipv4-unicast address family2. run show bgp neighbor 10.0.0.4 on DUT-A displays maximum-prefix limit as 100 for ipv4-unicast address family, inherited from peer-group EVPN-OVERLAY3. run show running-config protocols bgp on DUT-A shows maximum-prefix 100 under peer-group EVPN-OVERLAY ipv4-unicast section |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Validates new CLI command: set protocols bgp peer-group ipv4-unicast maximum-prefix |

#### 1.2.1.2 Verify configuring maximum-prefix for ipv6-unicast on peer-group

| Field | Content |
| --- | --- |
| Test Name | MOD02-02: ipv6-unicast maximum-prefix on peer-group |
| Purpose Of The Test | Verify that configuring maximum-prefix for ipv6-unicast on a peer-group correctly applies the inbound prefix limit to all peers in that group |
| Test Topo & Precondition | Topology:Leaf-1(DUT-A) ---- Spine-1(DUT-C,RR) ---- Leaf-2(DUT-B)Leaf-1(DUT-A) ---- Spine-2(DUT-D,RR) ---- Leaf-2(DUT-B)Loopbacks: DUT-A:10.0.0.1, DUT-B:10.0.0.2, DUT-C:10.0.0.3, DUT-D:10.0.0.4DUT-A:ge-1/1/1(10.1.1.0/31)--DUT-C:ge-1/1/1(10.1.1.1/31)DUT-A:ge-1/1/2(10.1.2.0/31)--DUT-D:ge-1/1/1(10.1.2.1/31)DUT-B:ge-1/1/1(10.1.3.0/31)--DUT-C:ge-1/1/2(10.1.3.1/31)DUT-B:ge-1/1/2(10.1.4.0/31)--DUT-D:ge-1/1/2(10.1.4.1/31)All AS 65000, DUT-C/DUT-D are Route ReflectorsPreconditions:1. All four DUTs are booted with base configuration applied2. OSPF adjacencies are established on all underlay links and loopbacks are mutually reachable3. iBGP EVPN sessions are established between leaves and spines via loopbacksConfiguration:DUT-A:set l3-interface loopback lo address 10.0.0.1 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.0 prefix-length 31set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.1set protocols ospf network 10.0.0.1/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.1set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueset protocols bgp peer-group EVPN-OVERLAY ipv6-unicast maximum-prefix 50DUT-B:set l3-interface loopback lo address 10.0.0.2 prefix-length 32set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.0 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.2set protocols ospf network 10.0.0.2/32 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.2set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueDUT-C:set l3-interface loopback lo address 10.0.0.3 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.1 prefix-length 31set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.3set protocols ospf network 10.0.0.3/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.3set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate trueDUT-D:set l3-interface loopback lo address 10.0.0.4 prefix-length 32set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.1 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.4set protocols ospf network 10.0.0.4/32 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.4set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate true |
| Test Procedure | 1. Configure set protocols bgp peer-group EVPN-OVERLAY ipv6-unicast maximum-prefix 50 on DUT-A2. Wait 10 seconds for configuration to take effect3. Execute run show bgp neighbor 10.0.0.3 on DUT-A4. Execute run show running-config protocols bgp on DUT-A |
| Expected Results | 1. run show bgp neighbor 10.0.0.3 on DUT-A displays maximum-prefix limit as 50 for ipv6-unicast address family2. run show running-config protocols bgp on DUT-A shows maximum-prefix 50 under peer-group EVPN-OVERLAY ipv6-unicast section |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Validates new CLI command: set protocols bgp peer-group ipv6-unicast maximum-prefix |

#### 1.2.1.3 Verify BGP session tears down when received prefixes exceed maximum-prefix limit

| Field | Content |
| --- | --- |
| Test Name | MOD02-03: Session teardown on maximum-prefix exceed |
| Purpose Of The Test | Verify that the BGP session transitions to Idle state when the number of received ipv4-unicast prefixes exceeds the configured maximum-prefix limit |
| Test Topo & Precondition | Topology:Leaf-1(DUT-A) ---- Spine-1(DUT-C,RR) ---- Leaf-2(DUT-B)Leaf-1(DUT-A) ---- Spine-2(DUT-D,RR) ---- Leaf-2(DUT-B)Loopbacks: DUT-A:10.0.0.1, DUT-B:10.0.0.2, DUT-C:10.0.0.3, DUT-D:10.0.0.4DUT-A:ge-1/1/1(10.1.1.0/31)--DUT-C:ge-1/1/1(10.1.1.1/31)DUT-A:ge-1/1/2(10.1.2.0/31)--DUT-D:ge-1/1/1(10.1.2.1/31)DUT-B:ge-1/1/1(10.1.3.0/31)--DUT-C:ge-1/1/2(10.1.3.1/31)DUT-B:ge-1/1/2(10.1.4.0/31)--DUT-D:ge-1/1/2(10.1.4.1/31)All AS 65000, DUT-C/DUT-D are Route ReflectorsPreconditions:1. All four DUTs are booted with base configuration applied2. OSPF adjacencies are established on all underlay links and loopbacks are mutually reachable3. iBGP EVPN sessions are established between leaves and spines via loopbacks4. DUT-C is configured to redistribute connected and static routes into BGP to generate sufficient prefixesConfiguration:DUT-A:set l3-interface loopback lo address 10.0.0.1 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.0 prefix-length 31set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.1set protocols ospf network 10.0.0.1/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.1set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueset protocols bgp peer-group EVPN-OVERLAY ipv4-unicast maximum-prefix 2DUT-B:set l3-interface loopback lo address 10.0.0.2 prefix-length 32set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.0 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.2set protocols ospf network 10.0.0.2/32 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.2set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueDUT-C:set l3-interface loopback lo address 10.0.0.3 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.1 prefix-length 31set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.3set protocols ospf network 10.0.0.3/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.3set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate trueset ip route 10.10.1.0/24 null0set ip route 10.10.2.0/24 null0set ip route 10.10.3.0/24 null0set protocols bgp ipv4-unicast redistribute staticDUT-D:set l3-interface loopback lo address 10.0.0.4 prefix-length 32set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.1 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.4set protocols ospf network 10.0.0.4/32 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.4set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate true |
| Test Procedure | 1. Configure set protocols bgp peer-group EVPN-OVERLAY ipv4-unicast maximum-prefix 2 on DUT-A2. Configure three static routes on DUT-C: set ip route 10.10.1.0/24 null0, set ip route 10.10.2.0/24 null0, set ip route 10.10.3.0/24 null03. Configure set protocols bgp ipv4-unicast redistribute static on DUT-C to advertise the static routes via BGP4. Wait 30 seconds for BGP to propagate the routes5. Execute run show bgp summary on DUT-A6. Execute run show bgp neighbor 10.0.0.3 on DUT-A7. Execute run show log on DUT-A |
| Expected Results | 1. run show bgp summary on DUT-A shows the session with 10.0.0.3 in Idle state or with PfxCt exceeded indication2. run show bgp neighbor 10.0.0.3 on DUT-A shows the session state as Idle and the last reset reason as prefix-count exceeded3. run show log on DUT-A contains a log message indicating maximum prefix limit (2) exceeded for neighbor 10.0.0.3 in ipv4-unicast address family |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Validates that the maximum-prefix enforcement triggers session teardown when the limit is exceeded |

#### 1.2.1.4 Verify maximum-prefix configuration in VRF context

| Field | Content |
| --- | --- |
| Test Name | MOD02-04: maximum-prefix in VRF TENANT1 |
| Purpose Of The Test | Verify that configuring maximum-prefix for ipv4-unicast within a VRF context correctly applies the inbound prefix limit to peers in that VRF |
| Test Topo & Precondition | Topology:Leaf-1(DUT-A) ---- Spine-1(DUT-C,RR) ---- Leaf-2(DUT-B)Leaf-1(DUT-A) ---- Spine-2(DUT-D,RR) ---- Leaf-2(DUT-B)Loopbacks: DUT-A:10.0.0.1, DUT-B:10.0.0.2, DUT-C:10.0.0.3, DUT-D:10.0.0.4DUT-A:ge-1/1/1(10.1.1.0/31)--DUT-C:ge-1/1/1(10.1.1.1/31)DUT-A:ge-1/1/2(10.1.2.0/31)--DUT-D:ge-1/1/1(10.1.2.1/31)DUT-B:ge-1/1/1(10.1.3.0/31)--DUT-C:ge-1/1/2(10.1.3.1/31)DUT-B:ge-1/1/2(10.1.4.0/31)--DUT-D:ge-1/1/2(10.1.4.1/31)All AS 65000, DUT-C/DUT-D are Route ReflectorsPreconditions:1. All four DUTs are booted with base configuration applied2. OSPF adjacencies are established on all underlay links and loopbacks are mutually reachable3. iBGP EVPN sessions are established between leaves and spines via loopbacks4. VRF TENANT1 is configured on DUT-A and DUT-B with a BGP peer-group inside the VRFConfiguration:DUT-A:set l3-interface loopback lo address 10.0.0.1 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.0 prefix-length 31set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.1set protocols ospf network 10.0.0.1/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.1set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueset vrf TENANT1set vrf TENANT1 rd 10.0.0.1:100set protocols bgp vrf TENANT1 local-as 65000set protocols bgp vrf TENANT1 router-id 10.0.0.1set protocols bgp vrf TENANT1 peer-group TENANT1-PEERSset protocols bgp vrf TENANT1 peer-group TENANT1-PEERS remote-as 65000set protocols bgp vrf TENANT1 peer-group TENANT1-PEERS ipv4-unicast maximum-prefix 50DUT-B:set l3-interface loopback lo address 10.0.0.2 prefix-length 32set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.0 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.2set protocols ospf network 10.0.0.2/32 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.2set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueset vrf TENANT1set vrf TENANT1 rd 10.0.0.2:100set protocols bgp vrf TENANT1 local-as 65000set protocols bgp vrf TENANT1 router-id 10.0.0.2set protocols bgp vrf TENANT1 peer-group TENANT1-PEERSset protocols bgp vrf TENANT1 peer-group TENANT1-PEERS remote-as 65000DUT-C:set l3-interface loopback lo address 10.0.0.3 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.1 prefix-length 31set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.3set protocols ospf network 10.0.0.3/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.3set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate trueDUT-D:set l3-interface loopback lo address 10.0.0.4 prefix-length 32set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.1 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.4set protocols ospf network 10.0.0.4/32 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.4set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate true |
| Test Procedure | 1. Configure VRF TENANT1 on DUT-A with set vrf TENANT1 and set vrf TENANT1 rd 10.0.0.1:1002. Configure BGP in VRF: set protocols bgp vrf TENANT1 local-as 650003. Configure set protocols bgp vrf TENANT1 peer-group TENANT1-PEERS ipv4-unicast maximum-prefix 50 on DUT-A4. Wait 10 seconds for configuration to take effect5. Execute run show bgp vrf TENANT1 summary on DUT-A6. Execute run show running-config protocols bgp vrf TENANT1 on DUT-A |
| Expected Results | 1. run show bgp vrf TENANT1 summary on DUT-A shows the VRF BGP instance is active2. run show running-config protocols bgp vrf TENANT1 on DUT-A shows maximum-prefix 50 configured under TENANT1-PEERS peer-group ipv4-unicast section |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P1 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Validates new CLI command: set protocols bgp vrf peer-group ipv4-unicast maximum-prefix |

#### 1.2.2 Maximum Prefix Outbound

#### 1.2.2.1 Verify configuring maximum-prefix-out for ipv4-unicast on peer-group

| Field | Content |
| --- | --- |
| Test Name | MOD02-05: ipv4-unicast maximum-prefix-out on peer-group |
| Purpose Of The Test | Verify that configuring maximum-prefix-out for ipv4-unicast on a peer-group correctly applies the outbound prefix limit |
| Test Topo & Precondition | Topology:Leaf-1(DUT-A) ---- Spine-1(DUT-C,RR) ---- Leaf-2(DUT-B)Leaf-1(DUT-A) ---- Spine-2(DUT-D,RR) ---- Leaf-2(DUT-B)Loopbacks: DUT-A:10.0.0.1, DUT-B:10.0.0.2, DUT-C:10.0.0.3, DUT-D:10.0.0.4DUT-A:ge-1/1/1(10.1.1.0/31)--DUT-C:ge-1/1/1(10.1.1.1/31)DUT-A:ge-1/1/2(10.1.2.0/31)--DUT-D:ge-1/1/1(10.1.2.1/31)DUT-B:ge-1/1/1(10.1.3.0/31)--DUT-C:ge-1/1/2(10.1.3.1/31)DUT-B:ge-1/1/2(10.1.4.0/31)--DUT-D:ge-1/1/2(10.1.4.1/31)All AS 65000, DUT-C/DUT-D are Route ReflectorsPreconditions:1. All four DUTs are booted with base configuration applied2. OSPF adjacencies are established on all underlay links and loopbacks are mutually reachable3. iBGP EVPN sessions are established between leaves and spines via loopbacksConfiguration:DUT-A:set l3-interface loopback lo address 10.0.0.1 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.0 prefix-length 31set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.1set protocols ospf network 10.0.0.1/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.1set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueset protocols bgp peer-group EVPN-OVERLAY ipv4-unicast maximum-prefix-out 100DUT-B:set l3-interface loopback lo address 10.0.0.2 prefix-length 32set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.0 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.2set protocols ospf network 10.0.0.2/32 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.2set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueDUT-C:set l3-interface loopback lo address 10.0.0.3 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.1 prefix-length 31set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.3set protocols ospf network 10.0.0.3/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.3set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate trueDUT-D:set l3-interface loopback lo address 10.0.0.4 prefix-length 32set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.1 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.4set protocols ospf network 10.0.0.4/32 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.4set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate true |
| Test Procedure | 1. Configure set protocols bgp peer-group EVPN-OVERLAY ipv4-unicast maximum-prefix-out 100 on DUT-A2. Wait 10 seconds for configuration to take effect3. Execute run show bgp neighbor 10.0.0.3 on DUT-A4. Execute run show running-config protocols bgp on DUT-A |
| Expected Results | 1. run show bgp neighbor 10.0.0.3 on DUT-A displays maximum-prefix-out limit as 100 for ipv4-unicast address family2. run show running-config protocols bgp on DUT-A shows maximum-prefix-out 100 under peer-group EVPN-OVERLAY ipv4-unicast section |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Validates new CLI command: set protocols bgp peer-group ipv4-unicast maximum-prefix-out |

#### 1.2.2.2 Verify configuring maximum-prefix-out for ipv6-unicast on peer-group

| Field | Content |
| --- | --- |
| Test Name | MOD02-06: ipv6-unicast maximum-prefix-out on peer-group |
| Purpose Of The Test | Verify that configuring maximum-prefix-out for ipv6-unicast on a peer-group correctly applies the outbound prefix limit |
| Test Topo & Precondition | Topology:Leaf-1(DUT-A) ---- Spine-1(DUT-C,RR) ---- Leaf-2(DUT-B)Leaf-1(DUT-A) ---- Spine-2(DUT-D,RR) ---- Leaf-2(DUT-B)Loopbacks: DUT-A:10.0.0.1, DUT-B:10.0.0.2, DUT-C:10.0.0.3, DUT-D:10.0.0.4DUT-A:ge-1/1/1(10.1.1.0/31)--DUT-C:ge-1/1/1(10.1.1.1/31)DUT-A:ge-1/1/2(10.1.2.0/31)--DUT-D:ge-1/1/1(10.1.2.1/31)DUT-B:ge-1/1/1(10.1.3.0/31)--DUT-C:ge-1/1/2(10.1.3.1/31)DUT-B:ge-1/1/2(10.1.4.0/31)--DUT-D:ge-1/1/2(10.1.4.1/31)All AS 65000, DUT-C/DUT-D are Route ReflectorsPreconditions:1. All four DUTs are booted with base configuration applied2. OSPF adjacencies are established on all underlay links and loopbacks are mutually reachable3. iBGP EVPN sessions are established between leaves and spines via loopbacksConfiguration:DUT-A:set l3-interface loopback lo address 10.0.0.1 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.0 prefix-length 31set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.1set protocols ospf network 10.0.0.1/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.1set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueset protocols bgp peer-group EVPN-OVERLAY ipv6-unicast maximum-prefix-out 50DUT-B:set l3-interface loopback lo address 10.0.0.2 prefix-length 32set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.0 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.2set protocols ospf network 10.0.0.2/32 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.2set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueDUT-C:set l3-interface loopback lo address 10.0.0.3 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.1 prefix-length 31set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.3set protocols ospf network 10.0.0.3/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.3set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate trueDUT-D:set l3-interface loopback lo address 10.0.0.4 prefix-length 32set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.1 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.4set protocols ospf network 10.0.0.4/32 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.4set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate true |
| Test Procedure | 1. Configure set protocols bgp peer-group EVPN-OVERLAY ipv6-unicast maximum-prefix-out 50 on DUT-A2. Wait 10 seconds for configuration to take effect3. Execute run show bgp neighbor 10.0.0.3 on DUT-A4. Execute run show running-config protocols bgp on DUT-A |
| Expected Results | 1. run show bgp neighbor 10.0.0.3 on DUT-A displays maximum-prefix-out limit as 50 for ipv6-unicast address family2. run show running-config protocols bgp on DUT-A shows maximum-prefix-out 50 under peer-group EVPN-OVERLAY ipv6-unicast section |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P1 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Validates new CLI command: set protocols bgp peer-group ipv6-unicast maximum-prefix-out |

#### 1.2.2.3 Verify outbound advertisements stop when prefixes exceed maximum-prefix-out limit

| Field | Content |
| --- | --- |
| Test Name | MOD02-07: Outbound prefix suppression on maximum-prefix-out exceed |
| Purpose Of The Test | Verify that when the number of locally originated prefixes exceeds the maximum-prefix-out limit, excess routes are not advertised to the peer |
| Test Topo & Precondition | Topology:Leaf-1(DUT-A) ---- Spine-1(DUT-C,RR) ---- Leaf-2(DUT-B)Leaf-1(DUT-A) ---- Spine-2(DUT-D,RR) ---- Leaf-2(DUT-B)Loopbacks: DUT-A:10.0.0.1, DUT-B:10.0.0.2, DUT-C:10.0.0.3, DUT-D:10.0.0.4DUT-A:ge-1/1/1(10.1.1.0/31)--DUT-C:ge-1/1/1(10.1.1.1/31)DUT-A:ge-1/1/2(10.1.2.0/31)--DUT-D:ge-1/1/1(10.1.2.1/31)DUT-B:ge-1/1/1(10.1.3.0/31)--DUT-C:ge-1/1/2(10.1.3.1/31)DUT-B:ge-1/1/2(10.1.4.0/31)--DUT-D:ge-1/1/2(10.1.4.1/31)All AS 65000, DUT-C/DUT-D are Route ReflectorsPreconditions:1. All four DUTs are booted with base configuration applied2. OSPF adjacencies are established on all underlay links and loopbacks are mutually reachable3. iBGP EVPN sessions are established between leaves and spines via loopbacks4. DUT-A has multiple static routes redistributed into BGP to generate outbound prefixesConfiguration:DUT-A:set l3-interface loopback lo address 10.0.0.1 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.0 prefix-length 31set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.1set protocols ospf network 10.0.0.1/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.1set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueset protocols bgp peer-group EVPN-OVERLAY ipv4-unicast maximum-prefix-out 2set ip route 10.20.1.0/24 null0set ip route 10.20.2.0/24 null0set ip route 10.20.3.0/24 null0set ip route 10.20.4.0/24 null0set protocols bgp ipv4-unicast redistribute staticDUT-B:set l3-interface loopback lo address 10.0.0.2 prefix-length 32set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.0 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.2set protocols ospf network 10.0.0.2/32 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.2set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueDUT-C:set l3-interface loopback lo address 10.0.0.3 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.1 prefix-length 31set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.3set protocols ospf network 10.0.0.3/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.3set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate trueDUT-D:set l3-interface loopback lo address 10.0.0.4 prefix-length 32set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.1 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.4set protocols ospf network 10.0.0.4/32 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.4set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate true |
| Test Procedure | 1. Configure set protocols bgp peer-group EVPN-OVERLAY ipv4-unicast maximum-prefix-out 2 on DUT-A2. Configure four static routes on DUT-A: set ip route 10.20.1.0/24 null0 through set ip route 10.20.4.0/24 null03. Configure set protocols bgp ipv4-unicast redistribute static on DUT-A4. Wait 30 seconds for BGP to process outbound updates5. Execute run show bgp neighbor 10.0.0.3 advertised-routes on DUT-A6. Execute run show bgp ipv4 unicast on DUT-C |
| Expected Results | 1. run show bgp neighbor 10.0.0.3 advertised-routes on DUT-A shows at most 2 ipv4-unicast prefixes advertised, not all 4 static routes2. run show bgp ipv4 unicast on DUT-C shows at most 2 prefixes received from DUT-A (10.0.0.1), confirming the outbound limit is enforced |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P1 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Validates that maximum-prefix-out enforcement suppresses excess outbound advertisements beyond the configured limit |

### 1.3 Conditional Advertisement (MOD-03)

#### 1.3.1 Exist-Map Conditional Advertisement

#### 1.3.1.1 Verify configuring conditional-advertise with exist-map on neighbor

| Field | Content |
| --- | --- |
| Test Name | MOD03-01: Conditional-advertise exist-map on neighbor |
| Purpose Of The Test | Verify that the conditional-advertise command with exist-map can be successfully configured on a specific BGP neighbor for ipv4-unicast address family |
| Test Topo & Precondition | Topology:Leaf-1(DUT-A) ---- Spine-1(DUT-C,RR) ---- Leaf-2(DUT-B)Leaf-1(DUT-A) ---- Spine-2(DUT-D,RR) ---- Leaf-2(DUT-B)Loopbacks: DUT-A:10.0.0.1, DUT-B:10.0.0.2, DUT-C:10.0.0.3, DUT-D:10.0.0.4DUT-A:ge-1/1/1(10.1.1.0/31)--DUT-C:ge-1/1/1(10.1.1.1/31)DUT-A:ge-1/1/2(10.1.2.0/31)--DUT-D:ge-1/1/1(10.1.2.1/31)DUT-B:ge-1/1/1(10.1.3.0/31)--DUT-C:ge-1/1/2(10.1.3.1/31)DUT-B:ge-1/1/2(10.1.4.0/31)--DUT-D:ge-1/1/2(10.1.4.1/31)All AS 65000, DUT-C/DUT-D are Route ReflectorsPreconditions:1. All four DUTs are booted with base configuration applied2. OSPF adjacencies are established on all underlay links and loopbacks are mutually reachable3. iBGP EVPN sessions are established between leaves and spines via loopbacks4. Route-maps ADVERTISE-MAP and EXIST-MAP are created with associated prefix-lists on DUT-AConfiguration:DUT-A:set l3-interface loopback lo address 10.0.0.1 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.0 prefix-length 31set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.1set protocols ospf network 10.0.0.1/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.1set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueset routing route-map ADVERTISE-MAP order 10 action permitset routing route-map ADVERTISE-MAP order 10 match ip-address prefix-list ADVERTISE-PREFIXset routing prefix-list ADVERTISE-PREFIX seq 10 action permit prefix 192.168.1.0/24set routing route-map EXIST-MAP order 10 action permitset routing route-map EXIST-MAP order 10 match ip-address prefix-list EXIST-PREFIXset routing prefix-list EXIST-PREFIX seq 10 action permit prefix 172.16.0.0/16set ip route 192.168.1.0/24 null0set protocols bgp ipv4-unicast redistribute staticset protocols bgp neighbor 10.0.0.3 ipv4-unicast conditional-advertise advertise-map ADVERTISE-MAP exist-map EXIST-MAPDUT-B:set l3-interface loopback lo address 10.0.0.2 prefix-length 32set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.0 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.2set protocols ospf network 10.0.0.2/32 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.2set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueDUT-C:set l3-interface loopback lo address 10.0.0.3 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.1 prefix-length 31set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.3set protocols ospf network 10.0.0.3/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.3set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate trueDUT-D:set l3-interface loopback lo address 10.0.0.4 prefix-length 32set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.1 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.4set protocols ospf network 10.0.0.4/32 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.4set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate true |
| Test Procedure | 1. Configure route-map ADVERTISE-MAP and prefix-list ADVERTISE-PREFIX on DUT-A2. Configure route-map EXIST-MAP and prefix-list EXIST-PREFIX on DUT-A3. Configure set protocols bgp neighbor 10.0.0.3 ipv4-unicast conditional-advertise advertise-map ADVERTISE-MAP exist-map EXIST-MAP on DUT-A4. Wait 10 seconds for configuration to take effect5. Execute run show running-config protocols bgp on DUT-A6. Execute run show bgp neighbor 10.0.0.3 on DUT-A |
| Expected Results | 1. run show running-config protocols bgp on DUT-A shows conditional-advertise advertise-map ADVERTISE-MAP exist-map EXIST-MAP under neighbor 10.0.0.3 ipv4-unicast section2. run show bgp neighbor 10.0.0.3 on DUT-A displays conditional advertisement configuration with advertise-map and exist-map references |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Validates new CLI command: set protocols bgp neighbor ipv4-unicast conditional-advertise advertise-map exist-map |

#### 1.3.1.2 Verify routes are advertised when exist-map matched route is present in BGP table

| Field | Content |
| --- | --- |
| Test Name | MOD03-02: Exist-map route present triggers advertisement |
| Purpose Of The Test | Verify that when a route matching the exist-map is present in the BGP table, the routes matching the advertise-map are advertised to the neighbor |
| Test Topo & Precondition | Topology:Leaf-1(DUT-A) ---- Spine-1(DUT-C,RR) ---- Leaf-2(DUT-B)Leaf-1(DUT-A) ---- Spine-2(DUT-D,RR) ---- Leaf-2(DUT-B)Loopbacks: DUT-A:10.0.0.1, DUT-B:10.0.0.2, DUT-C:10.0.0.3, DUT-D:10.0.0.4DUT-A:ge-1/1/1(10.1.1.0/31)--DUT-C:ge-1/1/1(10.1.1.1/31)DUT-A:ge-1/1/2(10.1.2.0/31)--DUT-D:ge-1/1/1(10.1.2.1/31)DUT-B:ge-1/1/1(10.1.3.0/31)--DUT-C:ge-1/1/2(10.1.3.1/31)DUT-B:ge-1/1/2(10.1.4.0/31)--DUT-D:ge-1/1/2(10.1.4.1/31)All AS 65000, DUT-C/DUT-D are Route ReflectorsPreconditions:1. All four DUTs are booted with base configuration applied2. OSPF adjacencies are established on all underlay links and loopbacks are mutually reachable3. iBGP EVPN sessions are established between leaves and spines via loopbacks4. Route-maps ADVERTISE-MAP and EXIST-MAP are created on DUT-A5. Static route 192.168.1.0/24 is configured on DUT-A (advertise-map target)6. Route 172.16.0.0/16 (exist-map target) is present in DUT-A BGP table via DUT-B advertisementConfiguration:DUT-A:set l3-interface loopback lo address 10.0.0.1 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.0 prefix-length 31set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.1set protocols ospf network 10.0.0.1/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.1set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueset routing route-map ADVERTISE-MAP order 10 action permitset routing route-map ADVERTISE-MAP order 10 match ip-address prefix-list ADVERTISE-PREFIXset routing prefix-list ADVERTISE-PREFIX seq 10 action permit prefix 192.168.1.0/24set routing route-map EXIST-MAP order 10 action permitset routing route-map EXIST-MAP order 10 match ip-address prefix-list EXIST-PREFIXset routing prefix-list EXIST-PREFIX seq 10 action permit prefix 172.16.0.0/16set ip route 192.168.1.0/24 null0set ip route 172.16.0.0/16 10.1.1.1set protocols bgp ipv4-unicast redistribute staticset protocols bgp neighbor 10.0.0.3 ipv4-unicast conditional-advertise advertise-map ADVERTISE-MAP exist-map EXIST-MAPDUT-B:set l3-interface loopback lo address 10.0.0.2 prefix-length 32set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.0 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.2set protocols ospf network 10.0.0.2/32 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.2set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueDUT-C:set l3-interface loopback lo address 10.0.0.3 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.1 prefix-length 31set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.3set protocols ospf network 10.0.0.3/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.3set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate trueDUT-D:set l3-interface loopback lo address 10.0.0.4 prefix-length 32set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.1 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.4set protocols ospf network 10.0.0.4/32 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.4set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate true |
| Test Procedure | 1. Configure route-maps, prefix-lists, and conditional-advertise on DUT-A as per preconditions2. Configure set ip route 172.16.0.0/16 10.1.1.1 on DUT-A to inject the exist-map target route3. Configure set ip route 192.168.1.0/24 null0 on DUT-A as the advertise-map target route4. Configure set protocols bgp ipv4-unicast redistribute static on DUT-A5. Wait 60 seconds for BGP conditional advertisement timer to evaluate6. Execute run show bgp neighbor 10.0.0.3 advertised-routes on DUT-A7. Execute run show bgp ipv4 unicast 192.168.1.0/24 on DUT-C |
| Expected Results | 1. run show bgp neighbor 10.0.0.3 advertised-routes on DUT-A shows 192.168.1.0/24 is advertised to neighbor 10.0.0.32. run show bgp ipv4 unicast 192.168.1.0/24 on DUT-C shows the route is received from 10.0.0.1 (DUT-A) with valid next-hop |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Validates exist-map conditional logic: route present in exist-map = advertise-map routes are sent |

#### 1.3.1.3 Verify routes are withdrawn when exist-map matched route disappears from BGP table

| Field | Content |
| --- | --- |
| Test Name | MOD03-03: Exist-map route removal triggers withdrawal |
| Purpose Of The Test | Verify that when the route matching the exist-map is removed from the BGP table, the routes matching the advertise-map are withdrawn from the neighbor |
| Test Topo & Precondition | Topology:Leaf-1(DUT-A) ---- Spine-1(DUT-C,RR) ---- Leaf-2(DUT-B)Leaf-1(DUT-A) ---- Spine-2(DUT-D,RR) ---- Leaf-2(DUT-B)Loopbacks: DUT-A:10.0.0.1, DUT-B:10.0.0.2, DUT-C:10.0.0.3, DUT-D:10.0.0.4DUT-A:ge-1/1/1(10.1.1.0/31)--DUT-C:ge-1/1/1(10.1.1.1/31)DUT-A:ge-1/1/2(10.1.2.0/31)--DUT-D:ge-1/1/1(10.1.2.1/31)DUT-B:ge-1/1/1(10.1.3.0/31)--DUT-C:ge-1/1/2(10.1.3.1/31)DUT-B:ge-1/1/2(10.1.4.0/31)--DUT-D:ge-1/1/2(10.1.4.1/31)All AS 65000, DUT-C/DUT-D are Route ReflectorsPreconditions:1. All four DUTs are booted with base configuration applied2. OSPF adjacencies are established on all underlay links and loopbacks are mutually reachable3. iBGP EVPN sessions are established between leaves and spines via loopbacks4. Conditional-advertise with exist-map is configured on DUT-A toward 10.0.0.35. Route 172.16.0.0/16 (exist-map target) is currently present in DUT-A BGP table6. Route 192.168.1.0/24 (advertise-map target) is currently being advertised to 10.0.0.3Configuration:DUT-A:set l3-interface loopback lo address 10.0.0.1 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.0 prefix-length 31set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.1set protocols ospf network 10.0.0.1/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.1set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueset routing route-map ADVERTISE-MAP order 10 action permitset routing route-map ADVERTISE-MAP order 10 match ip-address prefix-list ADVERTISE-PREFIXset routing prefix-list ADVERTISE-PREFIX seq 10 action permit prefix 192.168.1.0/24set routing route-map EXIST-MAP order 10 action permitset routing route-map EXIST-MAP order 10 match ip-address prefix-list EXIST-PREFIXset routing prefix-list EXIST-PREFIX seq 10 action permit prefix 172.16.0.0/16set ip route 192.168.1.0/24 null0set ip route 172.16.0.0/16 10.1.1.1set protocols bgp ipv4-unicast redistribute staticset protocols bgp neighbor 10.0.0.3 ipv4-unicast conditional-advertise advertise-map ADVERTISE-MAP exist-map EXIST-MAPDUT-B:set l3-interface loopback lo address 10.0.0.2 prefix-length 32set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.0 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.2set protocols ospf network 10.0.0.2/32 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.2set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueDUT-C:set l3-interface loopback lo address 10.0.0.3 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.1 prefix-length 31set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.3set protocols ospf network 10.0.0.3/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.3set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate trueDUT-D:set l3-interface loopback lo address 10.0.0.4 prefix-length 32set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.1 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.4set protocols ospf network 10.0.0.4/32 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.4set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate true |
| Test Procedure | 1. Execute run show bgp neighbor 10.0.0.3 advertised-routes on DUT-A to record current advertised routes2. Remove the exist-map target route: delete ip route 172.16.0.0/16 10.1.1.1 on DUT-A3. Wait 60 seconds for BGP conditional advertisement timer to re-evaluate4. Execute run show bgp neighbor 10.0.0.3 advertised-routes on DUT-A5. Execute run show bgp ipv4 unicast 192.168.1.0/24 on DUT-C |
| Expected Results | 1. run show bgp neighbor 10.0.0.3 advertised-routes on DUT-A before removal shows 192.168.1.0/24 advertised2. run show bgp neighbor 10.0.0.3 advertised-routes on DUT-A after removal does NOT show 192.168.1.0/243. run show bgp ipv4 unicast 192.168.1.0/24 on DUT-C shows the route is no longer present or is withdrawn |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Validates exist-map conditional logic: route absent from exist-map = advertise-map routes are withdrawn |

#### 1.3.1.4 Verify conditional-advertise with exist-map on peer-group

| Field | Content |
| --- | --- |
| Test Name | MOD03-04: Conditional-advertise exist-map on peer-group |
| Purpose Of The Test | Verify that the conditional-advertise command with exist-map can be applied to a peer-group and affects all peers in that group |
| Test Topo & Precondition | Topology:Leaf-1(DUT-A) ---- Spine-1(DUT-C,RR) ---- Leaf-2(DUT-B)Leaf-1(DUT-A) ---- Spine-2(DUT-D,RR) ---- Leaf-2(DUT-B)Loopbacks: DUT-A:10.0.0.1, DUT-B:10.0.0.2, DUT-C:10.0.0.3, DUT-D:10.0.0.4DUT-A:ge-1/1/1(10.1.1.0/31)--DUT-C:ge-1/1/1(10.1.1.1/31)DUT-A:ge-1/1/2(10.1.2.0/31)--DUT-D:ge-1/1/1(10.1.2.1/31)DUT-B:ge-1/1/1(10.1.3.0/31)--DUT-C:ge-1/1/2(10.1.3.1/31)DUT-B:ge-1/1/2(10.1.4.0/31)--DUT-D:ge-1/1/2(10.1.4.1/31)All AS 65000, DUT-C/DUT-D are Route ReflectorsPreconditions:1. All four DUTs are booted with base configuration applied2. OSPF adjacencies are established on all underlay links and loopbacks are mutually reachable3. iBGP EVPN sessions are established between leaves and spines via loopbacks4. Route-maps ADVERTISE-MAP and EXIST-MAP are created on DUT-A5. Route 172.16.0.0/16 (exist-map target) is present in DUT-A BGP tableConfiguration:DUT-A:set l3-interface loopback lo address 10.0.0.1 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.0 prefix-length 31set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.1set protocols ospf network 10.0.0.1/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.1set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueset routing route-map ADVERTISE-MAP order 10 action permitset routing route-map ADVERTISE-MAP order 10 match ip-address prefix-list ADVERTISE-PREFIXset routing prefix-list ADVERTISE-PREFIX seq 10 action permit prefix 192.168.1.0/24set routing route-map EXIST-MAP order 10 action permitset routing route-map EXIST-MAP order 10 match ip-address prefix-list EXIST-PREFIXset routing prefix-list EXIST-PREFIX seq 10 action permit prefix 172.16.0.0/16set ip route 192.168.1.0/24 null0set ip route 172.16.0.0/16 10.1.1.1set protocols bgp ipv4-unicast redistribute staticset protocols bgp peer-group EVPN-OVERLAY ipv4-unicast conditional-advertise advertise-map ADVERTISE-MAP exist-map EXIST-MAPDUT-B:set l3-interface loopback lo address 10.0.0.2 prefix-length 32set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.0 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.2set protocols ospf network 10.0.0.2/32 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.2set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueDUT-C:set l3-interface loopback lo address 10.0.0.3 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.1 prefix-length 31set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.3set protocols ospf network 10.0.0.3/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.3set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate trueDUT-D:set l3-interface loopback lo address 10.0.0.4 prefix-length 32set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.1 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.4set protocols ospf network 10.0.0.4/32 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.4set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate true |
| Test Procedure | 1. Configure set protocols bgp peer-group EVPN-OVERLAY ipv4-unicast conditional-advertise advertise-map ADVERTISE-MAP exist-map EXIST-MAP on DUT-A2. Wait 60 seconds for BGP conditional advertisement timer to evaluate3. Execute run show bgp neighbor 10.0.0.3 advertised-routes on DUT-A4. Execute run show bgp neighbor 10.0.0.4 advertised-routes on DUT-A5. Execute run show running-config protocols bgp on DUT-A |
| Expected Results | 1. run show bgp neighbor 10.0.0.3 advertised-routes on DUT-A shows 192.168.1.0/24 is advertised (exist-map route 172.16.0.0/16 is present)2. run show bgp neighbor 10.0.0.4 advertised-routes on DUT-A shows 192.168.1.0/24 is also advertised, confirming peer-group applies to all members3. run show running-config protocols bgp on DUT-A shows conditional-advertise configuration under peer-group EVPN-OVERLAY ipv4-unicast section |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P1 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Validates new CLI command: set protocols bgp peer-group ipv4-unicast conditional-advertise advertise-map exist-map |

#### 1.3.2 Non-Exist-Map Conditional Advertisement

#### 1.3.2.1 Verify configuring conditional-advertise with non-exist-map on neighbor

| Field | Content |
| --- | --- |
| Test Name | MOD03-05: Conditional-advertise non-exist-map on neighbor |
| Purpose Of The Test | Verify that the conditional-advertise command with non-exist-map can be successfully configured on a specific BGP neighbor for ipv4-unicast address family |
| Test Topo & Precondition | Topology:Leaf-1(DUT-A) ---- Spine-1(DUT-C,RR) ---- Leaf-2(DUT-B)Leaf-1(DUT-A) ---- Spine-2(DUT-D,RR) ---- Leaf-2(DUT-B)Loopbacks: DUT-A:10.0.0.1, DUT-B:10.0.0.2, DUT-C:10.0.0.3, DUT-D:10.0.0.4DUT-A:ge-1/1/1(10.1.1.0/31)--DUT-C:ge-1/1/1(10.1.1.1/31)DUT-A:ge-1/1/2(10.1.2.0/31)--DUT-D:ge-1/1/1(10.1.2.1/31)DUT-B:ge-1/1/1(10.1.3.0/31)--DUT-C:ge-1/1/2(10.1.3.1/31)DUT-B:ge-1/1/2(10.1.4.0/31)--DUT-D:ge-1/1/2(10.1.4.1/31)All AS 65000, DUT-C/DUT-D are Route ReflectorsPreconditions:1. All four DUTs are booted with base configuration applied2. OSPF adjacencies are established on all underlay links and loopbacks are mutually reachable3. iBGP EVPN sessions are established between leaves and spines via loopbacks4. Route-maps ADVERTISE-MAP and NON-EXIST-MAP are created with associated prefix-lists on DUT-AConfiguration:DUT-A:set l3-interface loopback lo address 10.0.0.1 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.0 prefix-length 31set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.1set protocols ospf network 10.0.0.1/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.1set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueset routing route-map ADVERTISE-MAP order 10 action permitset routing route-map ADVERTISE-MAP order 10 match ip-address prefix-list ADVERTISE-PREFIXset routing prefix-list ADVERTISE-PREFIX seq 10 action permit prefix 192.168.1.0/24set routing route-map NON-EXIST-MAP order 10 action permitset routing route-map NON-EXIST-MAP order 10 match ip-address prefix-list NON-EXIST-PREFIXset routing prefix-list NON-EXIST-PREFIX seq 10 action permit prefix 172.16.0.0/16set ip route 192.168.1.0/24 null0set protocols bgp ipv4-unicast redistribute staticset protocols bgp neighbor 10.0.0.3 ipv4-unicast conditional-advertise advertise-map ADVERTISE-MAP non-exist-map NON-EXIST-MAPDUT-B:set l3-interface loopback lo address 10.0.0.2 prefix-length 32set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.0 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.2set protocols ospf network 10.0.0.2/32 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.2set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueDUT-C:set l3-interface loopback lo address 10.0.0.3 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.1 prefix-length 31set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.3set protocols ospf network 10.0.0.3/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.3set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate trueDUT-D:set l3-interface loopback lo address 10.0.0.4 prefix-length 32set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.1 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.4set protocols ospf network 10.0.0.4/32 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.4set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate true |
| Test Procedure | 1. Configure route-map ADVERTISE-MAP, NON-EXIST-MAP, and associated prefix-lists on DUT-A2. Configure set protocols bgp neighbor 10.0.0.3 ipv4-unicast conditional-advertise advertise-map ADVERTISE-MAP non-exist-map NON-EXIST-MAP on DUT-A3. Wait 10 seconds for configuration to take effect4. Execute run show running-config protocols bgp on DUT-A5. Execute run show bgp neighbor 10.0.0.3 on DUT-A |
| Expected Results | 1. run show running-config protocols bgp on DUT-A shows conditional-advertise advertise-map ADVERTISE-MAP non-exist-map NON-EXIST-MAP under neighbor 10.0.0.3 ipv4-unicast section2. run show bgp neighbor 10.0.0.3 on DUT-A displays conditional advertisement configuration with advertise-map and non-exist-map references |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Validates new CLI command: set protocols bgp neighbor ipv4-unicast conditional-advertise advertise-map non-exist-map |

#### 1.3.2.2 Verify routes are advertised when non-exist-map matched route is absent from BGP table

| Field | Content |
| --- | --- |
| Test Name | MOD03-06: Non-exist-map route absent triggers advertisement |
| Purpose Of The Test | Verify that when no route matching the non-exist-map is present in the BGP table, the routes matching the advertise-map are advertised to the neighbor |
| Test Topo & Precondition | Topology:Leaf-1(DUT-A) ---- Spine-1(DUT-C,RR) ---- Leaf-2(DUT-B)Leaf-1(DUT-A) ---- Spine-2(DUT-D,RR) ---- Leaf-2(DUT-B)Loopbacks: DUT-A:10.0.0.1, DUT-B:10.0.0.2, DUT-C:10.0.0.3, DUT-D:10.0.0.4DUT-A:ge-1/1/1(10.1.1.0/31)--DUT-C:ge-1/1/1(10.1.1.1/31)DUT-A:ge-1/1/2(10.1.2.0/31)--DUT-D:ge-1/1/1(10.1.2.1/31)DUT-B:ge-1/1/1(10.1.3.0/31)--DUT-C:ge-1/1/2(10.1.3.1/31)DUT-B:ge-1/1/2(10.1.4.0/31)--DUT-D:ge-1/1/2(10.1.4.1/31)All AS 65000, DUT-C/DUT-D are Route ReflectorsPreconditions:1. All four DUTs are booted with base configuration applied2. OSPF adjacencies are established on all underlay links and loopbacks are mutually reachable3. iBGP EVPN sessions are established between leaves and spines via loopbacks4. Route-maps ADVERTISE-MAP and NON-EXIST-MAP are created on DUT-A5. Static route 192.168.1.0/24 is configured on DUT-A (advertise-map target)6. Route 172.16.0.0/16 (non-exist-map target) is NOT present in the BGP tableConfiguration:DUT-A:set l3-interface loopback lo address 10.0.0.1 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.0 prefix-length 31set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.1set protocols ospf network 10.0.0.1/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.1set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueset routing route-map ADVERTISE-MAP order 10 action permitset routing route-map ADVERTISE-MAP order 10 match ip-address prefix-list ADVERTISE-PREFIXset routing prefix-list ADVERTISE-PREFIX seq 10 action permit prefix 192.168.1.0/24set routing route-map NON-EXIST-MAP order 10 action permitset routing route-map NON-EXIST-MAP order 10 match ip-address prefix-list NON-EXIST-PREFIXset routing prefix-list NON-EXIST-PREFIX seq 10 action permit prefix 172.16.0.0/16set ip route 192.168.1.0/24 null0set protocols bgp ipv4-unicast redistribute staticset protocols bgp neighbor 10.0.0.3 ipv4-unicast conditional-advertise advertise-map ADVERTISE-MAP non-exist-map NON-EXIST-MAPDUT-B:set l3-interface loopback lo address 10.0.0.2 prefix-length 32set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.0 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.2set protocols ospf network 10.0.0.2/32 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.2set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueDUT-C:set l3-interface loopback lo address 10.0.0.3 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.1 prefix-length 31set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.3set protocols ospf network 10.0.0.3/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.3set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate trueDUT-D:set l3-interface loopback lo address 10.0.0.4 prefix-length 32set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.1 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.4set protocols ospf network 10.0.0.4/32 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.4set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate true |
| Test Procedure | 1. Configure conditional-advertise with non-exist-map on DUT-A as per preconditions2. Execute run show bgp ipv4 unicast 172.16.0.0/16 on DUT-A to observe whether the non-exist-map target route exists in the BGP table3. Wait 60 seconds for BGP conditional advertisement timer to evaluate4. Execute run show bgp neighbor 10.0.0.3 advertised-routes on DUT-A5. Execute run show bgp ipv4 unicast 192.168.1.0/24 on DUT-C |
| Expected Results | 1. run show bgp ipv4 unicast 172.16.0.0/16 on DUT-A shows no matching route (route not found)2. run show bgp neighbor 10.0.0.3 advertised-routes on DUT-A shows 192.168.1.0/24 is advertised to neighbor 10.0.0.33. run show bgp ipv4 unicast 192.168.1.0/24 on DUT-C shows the route is received from 10.0.0.1 (DUT-A) |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Validates non-exist-map conditional logic: route absent from non-exist-map = advertise-map routes are sent |

#### 1.3.2.3 Verify routes are withdrawn when non-exist-map matched route appears in BGP table

| Field | Content |
| --- | --- |
| Test Name | MOD03-07: Non-exist-map route appearance triggers withdrawal |
| Purpose Of The Test | Verify that when a route matching the non-exist-map appears in the BGP table, the routes matching the advertise-map are withdrawn from the neighbor |
| Test Topo & Precondition | Topology:Leaf-1(DUT-A) ---- Spine-1(DUT-C,RR) ---- Leaf-2(DUT-B)Leaf-1(DUT-A) ---- Spine-2(DUT-D,RR) ---- Leaf-2(DUT-B)Loopbacks: DUT-A:10.0.0.1, DUT-B:10.0.0.2, DUT-C:10.0.0.3, DUT-D:10.0.0.4DUT-A:ge-1/1/1(10.1.1.0/31)--DUT-C:ge-1/1/1(10.1.1.1/31)DUT-A:ge-1/1/2(10.1.2.0/31)--DUT-D:ge-1/1/1(10.1.2.1/31)DUT-B:ge-1/1/1(10.1.3.0/31)--DUT-C:ge-1/1/2(10.1.3.1/31)DUT-B:ge-1/1/2(10.1.4.0/31)--DUT-D:ge-1/1/2(10.1.4.1/31)All AS 65000, DUT-C/DUT-D are Route ReflectorsPreconditions:1. All four DUTs are booted with base configuration applied2. OSPF adjacencies are established on all underlay links and loopbacks are mutually reachable3. iBGP EVPN sessions are established between leaves and spines via loopbacks4. Conditional-advertise with non-exist-map is configured on DUT-A toward 10.0.0.35. Route 172.16.0.0/16 (non-exist-map target) is NOT currently in the BGP table6. Route 192.168.1.0/24 (advertise-map target) is currently being advertised to 10.0.0.3Configuration:DUT-A:set l3-interface loopback lo address 10.0.0.1 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.0 prefix-length 31set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.1set protocols ospf network 10.0.0.1/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.1set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueset routing route-map ADVERTISE-MAP order 10 action permitset routing route-map ADVERTISE-MAP order 10 match ip-address prefix-list ADVERTISE-PREFIXset routing prefix-list ADVERTISE-PREFIX seq 10 action permit prefix 192.168.1.0/24set routing route-map NON-EXIST-MAP order 10 action permitset routing route-map NON-EXIST-MAP order 10 match ip-address prefix-list NON-EXIST-PREFIXset routing prefix-list NON-EXIST-PREFIX seq 10 action permit prefix 172.16.0.0/16set ip route 192.168.1.0/24 null0set protocols bgp ipv4-unicast redistribute staticset protocols bgp neighbor 10.0.0.3 ipv4-unicast conditional-advertise advertise-map ADVERTISE-MAP non-exist-map NON-EXIST-MAPDUT-B:set l3-interface loopback lo address 10.0.0.2 prefix-length 32set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.0 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.2set protocols ospf network 10.0.0.2/32 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.2set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueDUT-C:set l3-interface loopback lo address 10.0.0.3 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.1 prefix-length 31set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.3set protocols ospf network 10.0.0.3/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.3set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate trueDUT-D:set l3-interface loopback lo address 10.0.0.4 prefix-length 32set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.1 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.4set protocols ospf network 10.0.0.4/32 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.4set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate true |
| Test Procedure | 1. Execute run show bgp neighbor 10.0.0.3 advertised-routes on DUT-A to record current advertised routes2. Add the non-exist-map target route: set ip route 172.16.0.0/16 10.1.1.1 on DUT-A3. Configure set protocols bgp ipv4-unicast redistribute static on DUT-A (if not already) so 172.16.0.0/16 enters the BGP table4. Wait 60 seconds for BGP conditional advertisement timer to re-evaluate5. Execute run show bgp neighbor 10.0.0.3 advertised-routes on DUT-A6. Execute run show bgp ipv4 unicast 192.168.1.0/24 on DUT-C |
| Expected Results | 1. run show bgp neighbor 10.0.0.3 advertised-routes on DUT-A before injection shows 192.168.1.0/24 advertised2. run show bgp neighbor 10.0.0.3 advertised-routes on DUT-A after injection does NOT show 192.168.1.0/243. run show bgp ipv4 unicast 192.168.1.0/24 on DUT-C shows the route is no longer present or is withdrawn |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Validates non-exist-map conditional logic: route present in non-exist-map = advertise-map routes are withdrawn |

#### 1.3.2.4 Verify conditional-advertise with non-exist-map for ipv6-unicast address family

| Field | Content |
| --- | --- |
| Test Name | MOD03-08: Conditional-advertise non-exist-map for ipv6-unicast |
| Purpose Of The Test | Verify that the conditional-advertise command with non-exist-map works correctly for ipv6-unicast address family using IPv6 prefix-lists and route-maps |
| Test Topo & Precondition | Topology:Leaf-1(DUT-A) ---- Spine-1(DUT-C,RR) ---- Leaf-2(DUT-B)Leaf-1(DUT-A) ---- Spine-2(DUT-D,RR) ---- Leaf-2(DUT-B)Loopbacks: DUT-A:10.0.0.1, DUT-B:10.0.0.2, DUT-C:10.0.0.3, DUT-D:10.0.0.4DUT-A:ge-1/1/1(10.1.1.0/31)--DUT-C:ge-1/1/1(10.1.1.1/31)DUT-A:ge-1/1/2(10.1.2.0/31)--DUT-D:ge-1/1/1(10.1.2.1/31)DUT-B:ge-1/1/1(10.1.3.0/31)--DUT-C:ge-1/1/2(10.1.3.1/31)DUT-B:ge-1/1/2(10.1.4.0/31)--DUT-D:ge-1/1/2(10.1.4.1/31)All AS 65000, DUT-C/DUT-D are Route ReflectorsPreconditions:1. All four DUTs are booted with base configuration applied2. OSPF adjacencies are established on all underlay links and loopbacks are mutually reachable3. iBGP EVPN sessions are established between leaves and spines via loopbacks4. IPv6 route-maps ADV-MAP-V6 and NON-EXIST-MAP-V6 are created with associated IPv6 prefix-lists on DUT-A5. IPv6 static route 2001:db8:1::/48 (advertise-map target) is configured on DUT-A6. IPv6 route 2001:db8:ff::/48 (non-exist-map target) is NOT present in BGP tableConfiguration:DUT-A:set l3-interface loopback lo address 10.0.0.1 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.0 prefix-length 31set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.1set protocols ospf network 10.0.0.1/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.1set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueset routing route-map ADV-MAP-V6 order 10 action permitset routing route-map ADV-MAP-V6 order 10 match ipv6-address prefix-list ADV-PREFIX-V6set routing prefix-list ADV-PREFIX-V6 seq 10 action permit prefix 2001:db8:1::/48set routing route-map NON-EXIST-MAP-V6 order 10 action permitset routing route-map NON-EXIST-MAP-V6 order 10 match ipv6-address prefix-list NON-EXIST-PREFIX-V6set routing prefix-list NON-EXIST-PREFIX-V6 seq 10 action permit prefix 2001:db8:ff::/48set ipv6 route 2001:db8:1::/48 null0set protocols bgp ipv6-unicast redistribute staticset protocols bgp neighbor 10.0.0.3 ipv6-unicast conditional-advertise advertise-map ADV-MAP-V6 non-exist-map NON-EXIST-MAP-V6DUT-B:set l3-interface loopback lo address 10.0.0.2 prefix-length 32set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.0 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.0 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.2set protocols ospf network 10.0.0.2/32 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.2set protocols bgp peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY remote-as 65000set protocols bgp peer-group EVPN-OVERLAY update-source loset protocols bgp neighbor 10.0.0.3 peer-group EVPN-OVERLAYset protocols bgp neighbor 10.0.0.4 peer-group EVPN-OVERLAYset protocols bgp peer-group EVPN-OVERLAY evpn activate trueDUT-C:set l3-interface loopback lo address 10.0.0.3 prefix-length 32set vlans vlan-id 101 l3-interface vlan101set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 101set l3-interface vlan-interface vlan101 address 10.1.1.1 prefix-length 31set vlans vlan-id 103 l3-interface vlan103set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 103set l3-interface vlan-interface vlan103 address 10.1.3.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.3set protocols ospf network 10.0.0.3/32 area 0.0.0.0set protocols ospf network 10.1.1.0/31 area 0.0.0.0set protocols ospf network 10.1.3.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.3set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate trueDUT-D:set l3-interface loopback lo address 10.0.0.4 prefix-length 32set vlans vlan-id 102 l3-interface vlan102set interface gigabit-ethernet ge-1/1/1 family ethernet-switching native-vlan-id 102set l3-interface vlan-interface vlan102 address 10.1.2.1 prefix-length 31set vlans vlan-id 104 l3-interface vlan104set interface gigabit-ethernet ge-1/1/2 family ethernet-switching native-vlan-id 104set l3-interface vlan-interface vlan104 address 10.1.4.1 prefix-length 31set ip routing enable trueset protocols ospf router-id 10.0.0.4set protocols ospf network 10.0.0.4/32 area 0.0.0.0set protocols ospf network 10.1.2.0/31 area 0.0.0.0set protocols ospf network 10.1.4.0/31 area 0.0.0.0set protocols bgp local-as 65000set protocols bgp router-id 10.0.0.4set protocols bgp peer-group LEAFset protocols bgp peer-group LEAF remote-as 65000set protocols bgp peer-group LEAF update-source loset protocols bgp peer-group LEAF route-reflector-client trueset protocols bgp neighbor 10.0.0.1 peer-group LEAFset protocols bgp neighbor 10.0.0.2 peer-group LEAFset protocols bgp peer-group LEAF evpn activate true |
| Test Procedure | 1. Configure IPv6 route-maps ADV-MAP-V6, NON-EXIST-MAP-V6, and associated IPv6 prefix-lists on DUT-A2. Configure set ipv6 route 2001:db8:1::/48 null0 on DUT-A3. Configure set protocols bgp ipv6-unicast redistribute static on DUT-A4. Configure set protocols bgp neighbor 10.0.0.3 ipv6-unicast conditional-advertise advertise-map ADV-MAP-V6 non-exist-map NON-EXIST-MAP-V6 on DUT-A5. Wait 60 seconds for BGP conditional advertisement timer to evaluate6. Execute run show bgp neighbor 10.0.0.3 advertised-routes on DUT-A7. Execute run show bgp ipv6 unicast 2001:db8:1::/48 on DUT-C8. Execute run show running-config protocols bgp on DUT-A |
| Expected Results | 1. run show bgp neighbor 10.0.0.3 advertised-routes on DUT-A shows 2001:db8:1::/48 is advertised (non-exist-map target 2001:db8:ff::/48 is absent)2. run show bgp ipv6 unicast 2001:db8:1::/48 on DUT-C shows the route is received from 10.0.0.1 (DUT-A)3. run show running-config protocols bgp on DUT-A shows conditional-advertise advertise-map ADV-MAP-V6 non-exist-map NON-EXIST-MAP-V6 under neighbor 10.0.0.3 ipv6-unicast section |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P1 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Validates new CLI command: set protocols bgp neighbor ipv6-unicast conditional-advertise advertise-map non-exist-map . Uses IPv6 prefix-lists and route-maps. |