# PicOS Stacking Functional Test Cases — MOD-4: CoPP Rate Allocation

Platform: PicOS
Feature: Stacking CoPP Rate Division
Generated: 2026-03-16
Developer Requirement: TP-4, TP-5, TP-6
Total Cases: 25

## Table of Contents

- 1.4 CoPP Rate Allocation in Stacking

  - 1.4.1 Basic CoPP Rate Division

    - 1.4.1.1 Two-Device Stack Rate Division

    - 1.4.1.2 Three-Device Stack Rate Division

    - 1.4.1.3 Four-Device Stack Rate Division

    - 1.4.1.4 All Members Sending Simultaneously Total Rate

    - 1.4.1.5 Dynamic Rate Adjustment on Member Count Change

  - 1.4.2 Per-Queue CoPP Verification

    - 1.4.2.1 ARP Queue Rate in Stacking

    - 1.4.2.2 OSPF Queue Rate in Stacking

    - 1.4.2.3 BGP Queue Rate in Stacking

    - 1.4.2.4 ICMP Queue Rate in Stacking

    - 1.4.2.5 LACP Queue Rate in Stacking

    - 1.4.2.6 STP/BPDU Queue Rate in Stacking

  - 1.4.3 CoPP Rate After Stack Events

    - 1.4.3.1 Rate After Master/Standby Switchover

    - 1.4.3.2 Rate After Member Joins Stack

    - 1.4.3.3 Rate After Member Leaves Stack

    - 1.4.3.4 Rate After CoPP Limit Value Modification

    - 1.4.3.5 Rate Consistency Across All Members After Config Change

  - 1.4.4 CoPP Burst and Stress

    - 1.4.4.1 Single Member Exceeds Its Rate Share

    - 1.4.4.2 All Members Simultaneously Exceed Display Value

    - 1.4.4.3 Protocol Flap Under CoPP Limiting in Stack

    - 1.4.4.4 Long-Duration CoPP Rate Accuracy

  - 1.4.5 PicOS vs Ruijie Comparison Points

    - 1.4.5.1 PicOS Single Member Rate Equals Total Divided by N

    - 1.4.5.2 Document Expected Ruijie Behavior for Comparison

    - 1.4.5.3 PicOS Behavior When Only One Member Receives All Traffic

    - 1.4.5.4 Total System Capacity Across All Members

    - 1.4.5.5 Impact on Protocol Convergence Under CoPP Limiting

- Coverage Summary

## 1.4 CoPP Rate Allocation in Stacking

### 1.4.1 Basic CoPP Rate Division

#### 1.4.1.1 Two-Device Stack Rate Division

| Field | Content |
| --- | --- |
| Test Name | MOD4-001: Verify CoPP rate per device equals total/2 in a 2-device stack |
| Purpose Of The Test | Verify that when 2 devices form a stack and the CoPP rate limit is configured to 500 pps, each individual device enforces a rate limit of 250 pps (500/2) for control plane traffic |
| Test Topo & Precondition | Topology:Traffic Generator TG1 ---- (port 1) Stack-Member-1 (Master) ==stack-link== Stack-Member-2 (Standby) (port 1) ---- Traffic Generator TG2Preconditions:1. PicOS 4.6 installed on both switches2. Two switches configured as a stack with Member-1 as master and Member-2 as standby3. Stack is fully formed and stable (all members in ready state)4. Traffic generators TG1 and TG2 connected to ports on Member-1 and Member-2 respectivelyConfiguration:Stack Master:set protocols copp rate-limit 500commit |
| Test Procedure | 1. Execute run show copp rate-limit on the stack master to record the configured rate-limit value2. Execute run show stack status to confirm 2 members are active in the stack3. Configure TG1 to send ARP-request broadcast traffic at 400 pps towards Member-1 port 14. Wait 30 seconds for rate to stabilize5. Execute run show copp statistics on the stack master to record received/dropped packet counters for Member-16. Stop TG1 traffic7. Configure TG2 to send ARP-request broadcast traffic at 400 pps towards Member-2 port 18. Wait 30 seconds for rate to stabilize9. Execute run show copp statistics on the stack master to record received/dropped packet counters for Member-210. Stop TG2 traffic |
| Expected Results | 1. run show copp rate-limit displays rate-limit as 500 pps2. run show stack status shows 2 active members3. Member-1 copp statistics show approximately 250 pps received by CPU and approximately 150 pps dropped (400 - 250 = 150 dropped)4. Member-2 copp statistics show approximately 250 pps received by CPU and approximately 150 pps dropped (400 - 250 = 150 dropped)5. Per-device effective rate is 500/2 = 250 pps (±5% tolerance) |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-4. Core CoPP rate division behavior. |

#### 1.4.1.2 Three-Device Stack Rate Division

| Field | Content |
| --- | --- |
| Test Name | MOD4-002: Verify CoPP rate per device equals total/3 in a 3-device stack |
| Purpose Of The Test | Verify that when 3 devices form a stack and the CoPP rate limit is configured to 600 pps, each individual device enforces a rate limit of 200 pps (600/3) for control plane traffic |
| Test Topo & Precondition | Topology:TG1 ---- (port 1) Member-1 (Master) ==stack-link== Member-2 (Standby) ==stack-link== Member-3 (Member) (port 1) ---- TG3TG2 ---- (port 1) Member-2Preconditions:1. PicOS 4.6 installed on all three switches2. Three switches configured as a stack with Member-1 as master3. Stack is fully formed and stable (all members in ready state)4. Traffic generators TG1, TG2, and TG3 connected to ports on Member-1, Member-2, and Member-3 respectivelyConfiguration:Stack Master:set protocols copp rate-limit 600commit |
| Test Procedure | 1. Execute run show copp rate-limit on the stack master to record the configured rate-limit value2. Execute run show stack status to confirm 3 members are active in the stack3. Configure TG1 to send ARP-request broadcast traffic at 300 pps towards Member-1 port 14. Wait 30 seconds for rate to stabilize5. Execute run show copp statistics on the stack master to record received/dropped packet counters for Member-16. Stop TG1 traffic7. Configure TG2 to send ARP-request broadcast traffic at 300 pps towards Member-2 port 18. Wait 30 seconds for rate to stabilize9. Execute run show copp statistics on the stack master to record received/dropped packet counters for Member-210. Stop TG2 traffic11. Configure TG3 to send ARP-request broadcast traffic at 300 pps towards Member-3 port 112. Wait 30 seconds for rate to stabilize13. Execute run show copp statistics on the stack master to record received/dropped packet counters for Member-314. Stop TG3 traffic |
| Expected Results | 1. run show copp rate-limit displays rate-limit as 600 pps2. run show stack status shows 3 active members3. Member-1 copp statistics show approximately 200 pps received by CPU and approximately 100 pps dropped4. Member-2 copp statistics show approximately 200 pps received by CPU and approximately 100 pps dropped5. Member-3 copp statistics show approximately 200 pps received by CPU and approximately 100 pps dropped6. Per-device effective rate is 600/3 = 200 pps (±5% tolerance) |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-4. Core CoPP rate division behavior. |

#### 1.4.1.3 Four-Device Stack Rate Division

| Field | Content |
| --- | --- |
| Test Name | MOD4-003: Verify CoPP rate per device equals total/4 in a 4-device stack |
| Purpose Of The Test | Verify that when 4 devices form a stack and the CoPP rate limit is configured to 500 pps, each individual device enforces a rate limit of 125 pps (500/4) for control plane traffic |
| Test Topo & Precondition | Topology:TG1 ---- (port 1) Member-1 (Master) ==stack-link== Member-2 (Standby) ==stack-link== Member-3 ==stack-link== Member-4 (port 1) ---- TG4TG2 ---- (port 1) Member-2TG3 ---- (port 1) Member-3Preconditions:1. PicOS 4.6 installed on all four switches2. Four switches configured as a stack with Member-1 as master3. Stack is fully formed and stable (all members in ready state)4. Traffic generators TG1–TG4 connected to ports on Member-1 through Member-4 respectivelyConfiguration:Stack Master:set protocols copp rate-limit 500commit |
| Test Procedure | 1. Execute run show copp rate-limit on the stack master to record the configured rate-limit value2. Execute run show stack status to confirm 4 members are active in the stack3. Configure TG1 to send ARP-request broadcast traffic at 200 pps towards Member-1 port 14. Wait 30 seconds for rate to stabilize5. Execute run show copp statistics on the stack master to record received/dropped counters for Member-16. Stop TG1 traffic7. Repeat steps 3–6 for TG2 targeting Member-2, TG3 targeting Member-3, and TG4 targeting Member-48. Execute run show copp statistics on the stack master to record final counters for all members |
| Expected Results | 1. run show copp rate-limit displays rate-limit as 500 pps2. run show stack status shows 4 active members3. Each member’s copp statistics show approximately 125 pps received by CPU and approximately 75 pps dropped (200 - 125 = 75)4. Per-device effective rate is 500/4 = 125 pps (±5% tolerance) |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-4. Core CoPP rate division behavior. This is the canonical example from the CoPP stacking design document. |

#### 1.4.1.4 All Members Sending Simultaneously Total Rate

| Field | Content |
| --- | --- |
| Test Name | MOD4-004: Verify total CPU rate across all members does not exceed display value when all members send simultaneously |
| Purpose Of The Test | Verify that when all stack members receive control plane traffic simultaneously, the aggregate rate reaching the CPU across all members does not exceed the configured CoPP rate-limit display value |
| Test Topo & Precondition | Topology:TG1 ---- (port 1) Member-1 (Master) ==stack-link== Member-2 (Standby) ==stack-link== Member-3 ==stack-link== Member-4 (port 1) ---- TG4TG2 ---- (port 1) Member-2TG3 ---- (port 1) Member-3Preconditions:1. PicOS 4.6 installed on all four switches2. Four switches configured as a stack with Member-1 as master3. Stack is fully formed and stable4. Traffic generators TG1–TG4 connected to Member-1 through Member-4 respectivelyConfiguration:Stack Master:set protocols copp rate-limit 500commit |
| Test Procedure | 1. Execute run show copp rate-limit on the stack master2. Execute run show stack status to confirm 4 members active3. Execute run show copp statistics on the stack master to record baseline counters for all members4. Configure TG1, TG2, TG3, and TG4 to simultaneously send ARP-request broadcast traffic at 200 pps each (800 pps total) towards their respective member ports5. Wait 60 seconds for rates to stabilize6. Execute run show copp statistics on the stack master to record counters for all members7. Calculate aggregate CPU-received rate across all 4 members from counter deltas8. Stop all traffic generators |
| Expected Results | 1. run show copp rate-limit displays 500 pps2. Each member receives approximately 125 pps to CPU (500/4 = 125 pps per device)3. Each member drops approximately 75 pps (200 - 125 = 75)4. Aggregate CPU-received rate across all 4 members is approximately 500 pps (±5% tolerance)5. Total aggregate CPU rate does NOT exceed the configured 500 pps display value |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-4. Critical system-level constraint validation. |

#### 1.4.1.5 Dynamic Rate Adjustment on Member Count Change

| Field | Content |
| --- | --- |
| Test Name | MOD4-005: Verify CoPP rate per device adjusts dynamically when stack member count changes |
| Purpose Of The Test | Verify that when a stack member is added or removed, the per-device CoPP rate is automatically recalculated to total/N where N is the new member count, without requiring manual reconfiguration |
| Test Topo & Precondition | Topology:TG1 ---- (port 1) Member-1 (Master) ==stack-link== Member-2 (Standby) (port 1) ---- TG2Member-3 (not yet stacked, standalone) ---- (stacking cable available)Preconditions:1. PicOS 4.6 installed on all three switches2. Member-1 and Member-2 configured as a 2-device stack3. Member-3 is pre-configured for stacking but not yet connected4. Stack is fully formed and stable with 2 members5. TG1 and TG2 connected to Member-1 and Member-2Configuration:Stack Master:set protocols copp rate-limit 600commit |
| Test Procedure | 1. Execute run show copp rate-limit on the stack master2. Execute run show stack status to confirm 2 members active3. Configure TG1 to send ARP-request broadcast traffic at 400 pps towards Member-1 port 14. Wait 30 seconds for rate to stabilize5. Execute run show copp statistics on the stack master to record Member-1 CPU-received rate6. Stop TG1 traffic7. Connect Member-3 stacking cable to the stack ring8. Wait 120 seconds for Member-3 to join the stack and reach ready state9. Execute run show stack status to confirm 3 members are now active10. Configure TG1 to send ARP-request broadcast traffic at 400 pps towards Member-1 port 111. Wait 30 seconds for rate to stabilize12. Execute run show copp statistics on the stack master to record Member-1 CPU-received rate13. Stop TG1 traffic |
| Expected Results | 1. run show copp rate-limit displays 600 pps2. With 2 members: Member-1 CPU-received rate is approximately 300 pps (600/2)3. run show stack status confirms Member-3 joined successfully (3 active members)4. With 3 members: Member-1 CPU-received rate drops to approximately 200 pps (600/3)5. Rate adjustment is automatic — no manual reconfiguration required6. Rate change takes effect within the stack convergence window (≤120 seconds after member joins) |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-4, TP-5. Dynamic rate recalculation is critical for stacking CoPP correctness. |

### 1.4.2 Per-Queue CoPP Verification

#### 1.4.2.1 ARP Queue Rate in Stacking

| Field | Content |
| --- | --- |
| Test Name | MOD4-006: Verify ARP queue CoPP rate division in stacking |
| Purpose Of The Test | Verify that the ARP-specific CoPP queue rate is correctly divided among stack members (rate per device = ARP queue limit / N) and that ARP packets exceeding the per-device rate are dropped |
| Test Topo & Precondition | Topology:TG1 ---- (port 1, VLAN 100) Member-1 (Master) ==stack-link== Member-2 (Standby) (port 1, VLAN 100) ---- TG2Preconditions:1. PicOS 4.6 installed on both switches2. Two switches configured as a stack with Member-1 as master3. Stack is fully formed and stable4. VLAN 100 configured with SVI IP on both members5. TG1 and TG2 connected to Member-1 and Member-2 ports in VLAN 100Configuration:Stack Master:set protocols copp queue arp rate-limit 400set vlans vlan-id 100set interface vlan100 address 10.1.1.1/24commit |
| Test Procedure | 1. Execute run show copp rate-limit on the stack master to record ARP queue rate-limit2. Execute run show stack status to confirm 2 members active3. Configure TG1 to send ARP-request packets (src MAC: 00:01:00:00:00:01, target IP: 10.1.1.1) at 300 pps towards Member-1 port 14. Wait 30 seconds for rate to stabilize5. Execute run show copp statistics queue arp on the stack master to record ARP queue counters for Member-16. Stop TG1 traffic7. Configure TG2 to send ARP-request packets at 300 pps towards Member-2 port 18. Wait 30 seconds for rate to stabilize9. Execute run show copp statistics queue arp on the stack master to record ARP queue counters for Member-210. Stop TG2 traffic |
| Expected Results | 1. run show copp rate-limit displays ARP queue rate-limit as 400 pps2. Member-1 ARP queue statistics show approximately 200 pps received by CPU (400/2) and approximately 100 pps dropped3. Member-2 ARP queue statistics show approximately 200 pps received by CPU (400/2) and approximately 100 pps dropped4. ARP responses are generated only for packets within the per-device rate (200 pps) |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-4. ARP is critical for L2/L3 reachability. |

#### 1.4.2.2 OSPF Queue Rate in Stacking

| Field | Content |
| --- | --- |
| Test Name | MOD4-007: Verify OSPF queue CoPP rate division in stacking |
| Purpose Of The Test | Verify that the OSPF-specific CoPP queue rate is correctly divided among stack members and OSPF Hello/LSA packets exceeding the per-device rate are dropped |
| Test Topo & Precondition | Topology:TG1 ---- (port 1, IP: 10.1.1.2/24) Member-1 (Master, IP: 10.1.1.1/24) ==stack-link== Member-2 (Standby, IP: 10.2.2.1/24) (port 1, IP: 10.2.2.2/24) ---- TG2Preconditions:1. PicOS 4.6 installed on both switches2. Two switches configured as a stack with Member-1 as master3. Stack is fully formed and stable4. OSPF is configured on Member-1 port 1 (area 0.0.0.0) and Member-2 port 1 (area 0.0.0.0)5. TG1 and TG2 capable of generating OSPF protocol packets (Hello, protocol 89, dst 224.0.0.5)Configuration:Stack Master:set protocols copp queue ospf rate-limit 300set protocols ospf area 0.0.0.0 interface ge-1/1/1set protocols ospf area 0.0.0.0 interface ge-2/1/1commit |
| Test Procedure | 1. Execute run show copp rate-limit on the stack master to record OSPF queue rate-limit2. Execute run show stack status to confirm 2 members active3. Configure TG1 to send crafted OSPF Hello packets (protocol 89, dst 224.0.0.5) at 250 pps towards Member-1 port 14. Wait 30 seconds for rate to stabilize5. Execute run show copp statistics queue ospf on the stack master to record OSPF queue counters for Member-16. Stop TG1 traffic7. Configure TG2 to send crafted OSPF Hello packets at 250 pps towards Member-2 port 18. Wait 30 seconds for rate to stabilize9. Execute run show copp statistics queue ospf on the stack master to record OSPF queue counters for Member-210. Stop TG2 traffic |
| Expected Results | 1. run show copp rate-limit displays OSPF queue rate-limit as 300 pps2. Member-1 OSPF queue statistics show approximately 150 pps received by CPU (300/2) and approximately 100 pps dropped3. Member-2 OSPF queue statistics show approximately 150 pps received by CPU (300/2) and approximately 100 pps dropped4. Drop counters increment for packets exceeding the per-device share |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-4. OSPF is a critical routing protocol — excessive drops may cause adjacency flaps. |

#### 1.4.2.3 BGP Queue Rate in Stacking

| Field | Content |
| --- | --- |
| Test Name | MOD4-008: Verify BGP queue CoPP rate division in stacking |
| Purpose Of The Test | Verify that the BGP-specific CoPP queue rate is correctly divided among stack members and BGP packets (TCP port 179) exceeding the per-device rate are dropped |
| Test Topo & Precondition | Topology:TG1 ---- (port 1, IP: 10.1.1.2/24) Member-1 (Master, IP: 10.1.1.1/24) ==stack-link== Member-2 (Standby, IP: 10.2.2.1/24) (port 1, IP: 10.2.2.2/24) ---- TG2Preconditions:1. PicOS 4.6 installed on both switches2. Two switches configured as a stack with Member-1 as master3. Stack is fully formed and stable4. BGP is configured on the stack with neighbors reachable via Member-1 and Member-2 ports5. TG1 and TG2 capable of generating TCP packets with dst port 179Configuration:Stack Master:set protocols copp queue bgp rate-limit 200set protocols bgp local-as 65001set protocols bgp neighbor 10.1.1.2 remote-as 65002set protocols bgp neighbor 10.2.2.2 remote-as 65003commit |
| Test Procedure | 1. Execute run show copp rate-limit on the stack master to record BGP queue rate-limit2. Execute run show stack status to confirm 2 members active3. Configure TG1 to send TCP packets with dst port 179 at 150 pps towards Member-1 port 14. Wait 30 seconds for rate to stabilize5. Execute run show copp statistics queue bgp on the stack master to record BGP queue counters for Member-16. Stop TG1 traffic7. Configure TG2 to send TCP packets with dst port 179 at 150 pps towards Member-2 port 18. Wait 30 seconds for rate to stabilize9. Execute run show copp statistics queue bgp on the stack master to record BGP queue counters for Member-210. Stop TG2 traffic |
| Expected Results | 1. run show copp rate-limit displays BGP queue rate-limit as 200 pps2. Member-1 BGP queue statistics show approximately 100 pps received by CPU (200/2) and approximately 50 pps dropped3. Member-2 BGP queue statistics show approximately 100 pps received by CPU (200/2) and approximately 50 pps dropped4. Drop counters increment proportionally to excess traffic above 100 pps per device |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-4. BGP session stability depends on CoPP allowing sufficient BGP traffic. |

#### 1.4.2.4 ICMP Queue Rate in Stacking

| Field | Content |
| --- | --- |
| Test Name | MOD4-009: Verify ICMP queue CoPP rate division in stacking |
| Purpose Of The Test | Verify that the ICMP-specific CoPP queue rate is correctly divided among stack members and ICMP echo-request packets exceeding the per-device rate are dropped |
| Test Topo & Precondition | Topology:TG1 ---- (port 1, IP: 10.1.1.2/24) Member-1 (Master, IP: 10.1.1.1/24) ==stack-link== Member-2 (Standby, IP: 10.2.2.1/24) (port 1, IP: 10.2.2.2/24) ---- TG2Preconditions:1. PicOS 4.6 installed on both switches2. Two switches configured as a stack with Member-1 as master3. Stack is fully formed and stable4. L3 interfaces configured on both member ports with IP addresses5. TG1 and TG2 capable of generating ICMP echo-request packets at configurable ratesConfiguration:Stack Master:set protocols copp queue icmp rate-limit 1000set interface ge-1/1/1 address 10.1.1.1/24set interface ge-2/1/1 address 10.2.2.1/24commit |
| Test Procedure | 1. Execute run show copp rate-limit on the stack master to record ICMP queue rate-limit2. Execute run show stack status to confirm 2 members active3. Configure TG1 to send ICMP echo-request packets (dst 10.1.1.1) at 800 pps towards Member-1 port 14. Wait 30 seconds for rate to stabilize5. Execute run show copp statistics queue icmp on the stack master to record ICMP queue counters for Member-16. Stop TG1 traffic7. Configure TG2 to send ICMP echo-request packets (dst 10.2.2.1) at 800 pps towards Member-2 port 18. Wait 30 seconds for rate to stabilize9. Execute run show copp statistics queue icmp on the stack master to record ICMP queue counters for Member-210. Stop TG2 traffic |
| Expected Results | 1. run show copp rate-limit displays ICMP queue rate-limit as 1000 pps2. Member-1 ICMP queue statistics show approximately 500 pps received by CPU (1000/2) and approximately 300 pps dropped3. Member-2 ICMP queue statistics show approximately 500 pps received by CPU (1000/2) and approximately 300 pps dropped4. ICMP echo-reply is only generated for packets within the per-device rate (500 pps) |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P1 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-4. ICMP is lower priority but important for troubleshooting and monitoring. |

#### 1.4.2.5 LACP Queue Rate in Stacking

| Field | Content |
| --- | --- |
| Test Name | MOD4-010: Verify LACP queue CoPP rate division in stacking |
| Purpose Of The Test | Verify that the LACP-specific CoPP queue rate is correctly divided among stack members and LACP PDU packets exceeding the per-device rate are dropped |
| Test Topo & Precondition | Topology:TG1 ---- (port 1, LAG member) Member-1 (Master) ==stack-link== Member-2 (Standby) (port 1, LAG member) ---- TG2Preconditions:1. PicOS 4.6 installed on both switches2. Two switches configured as a stack with Member-1 as master3. Stack is fully formed and stable4. LACP is configured on ports connected to TG1 and TG25. TG1 and TG2 capable of generating LACP PDU packets (dst MAC: 01:80:C2:00:00:02, EtherType: 0x8809)Configuration:Stack Master:set protocols copp queue lacp rate-limit 200set interface ge-1/1/1 ether-options 802.3ad ae1set interface ge-2/1/1 ether-options 802.3ad ae1set interface ae1 aggregated-ether-options lacp activecommit |
| Test Procedure | 1. Execute run show copp rate-limit on the stack master to record LACP queue rate-limit2. Execute run show stack status to confirm 2 members active3. Configure TG1 to send LACP PDU packets (EtherType 0x8809, subtype 0x01, dst 01:80:C2:00:00:02) at 150 pps towards Member-1 port 14. Wait 30 seconds for rate to stabilize5. Execute run show copp statistics queue lacp on the stack master to record LACP queue counters for Member-16. Stop TG1 traffic7. Configure TG2 to send LACP PDU packets at 150 pps towards Member-2 port 18. Wait 30 seconds for rate to stabilize9. Execute run show copp statistics queue lacp on the stack master to record LACP queue counters for Member-210. Stop TG2 traffic |
| Expected Results | 1. run show copp rate-limit displays LACP queue rate-limit as 200 pps2. Member-1 LACP queue statistics show approximately 100 pps received by CPU (200/2) and approximately 50 pps dropped3. Member-2 LACP queue statistics show approximately 100 pps received by CPU (200/2) and approximately 50 pps dropped4. LACP session state may flap if legitimate LACP PDUs are dropped due to rate limiting |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-4. LACP timeout depends on receiving PDUs — CoPP drops can break LAG. |

#### 1.4.2.6 STP/BPDU Queue Rate in Stacking

| Field | Content |
| --- | --- |
| Test Name | MOD4-011: Verify STP/BPDU queue CoPP rate division in stacking |
| Purpose Of The Test | Verify that the STP/BPDU-specific CoPP queue rate is correctly divided among stack members and BPDU packets exceeding the per-device rate are dropped |
| Test Topo & Precondition | Topology:TG1 ---- (port 1, STP enabled) Member-1 (Master) ==stack-link== Member-2 (Standby) (port 1, STP enabled) ---- TG2Preconditions:1. PicOS 4.6 installed on both switches2. Two switches configured as a stack with Member-1 as master3. Stack is fully formed and stable4. RSTP (Rapid Spanning Tree Protocol) enabled on Member-1 and Member-2 ports5. TG1 and TG2 capable of generating BPDU packets (dst MAC: 01:80:C2:00:00:00, LLC SSAP/DSAP 0x42)Configuration:Stack Master:set protocols copp queue stp rate-limit 300set protocols rstp interface ge-1/1/1set protocols rstp interface ge-2/1/1commit |
| Test Procedure | 1. Execute run show copp rate-limit on the stack master to record STP queue rate-limit2. Execute run show stack status to confirm 2 members active3. Configure TG1 to send BPDU packets (dst 01:80:C2:00:00:00) at 250 pps towards Member-1 port 14. Wait 30 seconds for rate to stabilize5. Execute run show copp statistics queue stp on the stack master to record STP queue counters for Member-16. Stop TG1 traffic7. Configure TG2 to send BPDU packets at 250 pps towards Member-2 port 18. Wait 30 seconds for rate to stabilize9. Execute run show copp statistics queue stp on the stack master to record STP queue counters for Member-210. Stop TG2 traffic |
| Expected Results | 1. run show copp rate-limit displays STP queue rate-limit as 300 pps2. Member-1 STP queue statistics show approximately 150 pps received by CPU (300/2) and approximately 100 pps dropped3. Member-2 STP queue statistics show approximately 150 pps received by CPU (300/2) and approximately 100 pps dropped4. STP topology may become unstable if legitimate BPDUs are dropped due to rate limiting |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-4. STP convergence relies on timely BPDU reception — CoPP division directly affects loop prevention. |

### 1.4.3 CoPP Rate After Stack Events

#### 1.4.3.1 Rate After Master/Standby Switchover

| Field | Content |
| --- | --- |
| Test Name | MOD4-012: Verify CoPP rate per device remains correct after master/standby switchover |
| Purpose Of The Test | Verify that after a master/standby switchover event, the CoPP rate division (total/N) is maintained correctly on all stack members and the new master enforces the same per-device rate |
| Test Topo & Precondition | Topology:TG1 ---- (port 1) Member-1 (Master, priority 200) ==stack-link== Member-2 (Standby, priority 150) (port 1) ---- TG2Preconditions:1. PicOS 4.6 installed on both switches2. Two switches configured as a stack with Member-1 as master (priority 200), Member-2 as standby (priority 150)3. Stack is fully formed and stable4. TG1 and TG2 connected to Member-1 and Member-2 respectivelyConfiguration:Stack Master:set protocols copp rate-limit 500set stack member 1 priority 200set stack member 2 priority 150commit |
| Test Procedure | 1. Execute run show copp rate-limit on the stack master to record rate-limit (500 pps)2. Execute run show stack status to confirm Member-1 is master and Member-2 is standby3. Configure TG1 to send ARP-request traffic at 400 pps towards Member-1 port 14. Wait 30 seconds for rate to stabilize5. Execute run show copp statistics on the stack master to record Member-1 CPU-received rate (baseline)6. Stop TG1 traffic7. Execute run request stack master-switchover on the stack master to trigger switchover8. Wait 120 seconds for switchover to complete and stack to restabilize9. Execute run show stack status on the new master (Member-2) to confirm Member-2 is now master10. Configure TG1 to send ARP-request traffic at 400 pps towards Member-1 port 111. Wait 30 seconds for rate to stabilize12. Execute run show copp statistics on the new master (Member-2) to record Member-1 CPU-received rate13. Execute run show copp rate-limit on the new master (Member-2)14. Stop TG1 traffic |
| Expected Results | 1. Before switchover: Member-1 CPU-received rate is approximately 250 pps (500/2)2. run show stack status confirms Member-2 is now master after switchover3. After switchover: run show copp rate-limit on new master still shows 500 pps4. After switchover: Member-1 CPU-received rate remains approximately 250 pps (500/2)5. CoPP rate division is preserved across the switchover event — no rate change or temporary unlimited window |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-5. Switchover must not create a CoPP bypass window. |

#### 1.4.3.2 Rate After Member Joins Stack

| Field | Content |
| --- | --- |
| Test Name | MOD4-013: Verify CoPP rate recalculates when a new member joins the stack |
| Purpose Of The Test | Verify that when a new member joins an existing stack, the CoPP per-device rate automatically decreases from total/N to total/(N+1) on all existing and new members |
| Test Topo & Precondition | Topology:TG1 ---- (port 1) Member-1 (Master) ==stack-link== Member-2 (Standby) (port 1) ---- TG2Member-3 (standalone, stacking cable disconnected)TG3 ---- (port 1) Member-3Preconditions:1. PicOS 4.6 installed on all three switches2. Member-1 and Member-2 form a 2-device stack3. Member-3 is standalone, pre-configured for stacking, stacking cable not yet connected4. Stack is fully formed and stable with 2 members5. TG1 connected to Member-1, TG2 to Member-2, TG3 to Member-3Configuration:Stack Master:set protocols copp rate-limit 600commit |
| Test Procedure | 1. Execute run show copp rate-limit on the stack master2. Execute run show stack status to confirm 2 members active3. Configure TG1 to send ARP-request traffic at 400 pps towards Member-1 port 14. Wait 30 seconds for rate to stabilize5. Execute run show copp statistics to record Member-1 CPU-received rate6. Stop TG1 traffic7. Connect Member-3 stacking cable to the stack ring8. Wait 120 seconds for Member-3 to join and reach ready state9. Execute run show stack status to confirm 3 members active10. Configure TG1 to send ARP-request traffic at 400 pps towards Member-1 port 111. Wait 30 seconds for rate to stabilize12. Execute run show copp statistics to record Member-1 CPU-received rate after join13. Stop TG1 traffic14. Configure TG3 to send ARP-request traffic at 400 pps towards Member-3 port 115. Wait 30 seconds for rate to stabilize16. Execute run show copp statistics to record Member-3 CPU-received rate17. Stop TG3 traffic |
| Expected Results | 1. With 2 members: Member-1 CPU-received rate is approximately 300 pps (600/2)2. run show stack status confirms 3 active members after Member-3 joins3. With 3 members: Member-1 CPU-received rate decreases to approximately 200 pps (600/3)4. Member-3 (newly joined) CPU-received rate is also approximately 200 pps (600/3)5. Rate recalculation occurs automatically upon member join |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-5. New member must receive the correct divided rate immediately upon joining. |

#### 1.4.3.3 Rate After Member Leaves Stack

| Field | Content |
| --- | --- |
| Test Name | MOD4-014: Verify CoPP rate recalculates when a member leaves the stack |
| Purpose Of The Test | Verify that when a stack member is removed (cable disconnect or shutdown), the CoPP per-device rate automatically increases from total/N to total/(N-1) on remaining members |
| Test Topo & Precondition | Topology:TG1 ---- (port 1) Member-1 (Master) ==stack-link== Member-2 (Standby) ==stack-link== Member-3 (Member) (port 1) ---- TG3TG2 ---- (port 1) Member-2Preconditions:1. PicOS 4.6 installed on all three switches2. Three switches configured as a stack with Member-1 as master3. Stack is fully formed and stable with 3 members4. TG1, TG2, TG3 connected to Member-1, Member-2, Member-3 respectivelyConfiguration:Stack Master:set protocols copp rate-limit 600commit |
| Test Procedure | 1. Execute run show copp rate-limit on the stack master2. Execute run show stack status to confirm 3 members active3. Configure TG1 to send ARP-request traffic at 300 pps towards Member-1 port 14. Wait 30 seconds for rate to stabilize5. Execute run show copp statistics to record Member-1 CPU-received rate with 3 members6. Stop TG1 traffic7. Disconnect Member-3 stacking cable from the stack ring8. Wait 120 seconds for Member-3 to be detected as departed and stack to reconverge9. Execute run show stack status to confirm only 2 members remain active10. Configure TG1 to send ARP-request traffic at 400 pps towards Member-1 port 111. Wait 30 seconds for rate to stabilize12. Execute run show copp statistics to record Member-1 CPU-received rate with 2 members13. Stop TG1 traffic |
| Expected Results | 1. With 3 members: Member-1 CPU-received rate is approximately 200 pps (600/3)2. run show stack status confirms only 2 active members after Member-3 leaves3. With 2 members: Member-1 CPU-received rate increases to approximately 300 pps (600/2)4. Rate recalculation occurs automatically upon member departure5. No CoPP bypass window during the reconvergence period |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-5. Remaining members must get higher per-device rates after a member departs. |

#### 1.4.3.4 Rate After CoPP Limit Value Modification

| Field | Content |
| --- | --- |
| Test Name | MOD4-015: Verify per-device CoPP rate updates when CoPP limit value is modified on running stack |
| Purpose Of The Test | Verify that when the CoPP rate-limit value is changed via CLI on a running stack, all members immediately adopt the new per-device rate (new_total/N) without requiring a stack restart |
| Test Topo & Precondition | Topology:TG1 ---- (port 1) Member-1 (Master) ==stack-link== Member-2 (Standby) (port 1) ---- TG2Preconditions:1. PicOS 4.6 installed on both switches2. Two switches configured as a stack with Member-1 as master3. Stack is fully formed and stable4. Initial CoPP rate-limit configured to 400 pps5. TG1 connected to Member-1Configuration:Stack Master:set protocols copp rate-limit 400commit |
| Test Procedure | 1. Execute run show copp rate-limit to confirm initial rate-limit is 400 pps2. Execute run show stack status to confirm 2 members active3. Configure TG1 to send ARP-request traffic at 300 pps towards Member-1 port 14. Wait 30 seconds for rate to stabilize5. Execute run show copp statistics to record Member-1 CPU-received rate (expected ~200 pps = 400/2)6. Stop TG1 traffic7. Configure new CoPP rate-limit on the stack master:set protocols copp rate-limit 800commit8. Wait 10 seconds for configuration to propagate9. Execute run show copp rate-limit to confirm new rate-limit is 800 pps10. Configure TG1 to send ARP-request traffic at 500 pps towards Member-1 port 111. Wait 30 seconds for rate to stabilize12. Execute run show copp statistics to record Member-1 CPU-received rate (expected ~400 pps = 800/2)13. Stop TG1 traffic |
| Expected Results | 1. Initial run show copp rate-limit shows 400 pps2. With rate-limit 400 pps and 2 members: Member-1 CPU-received rate is approximately 200 pps (400/2)3. After modification: run show copp rate-limit shows 800 pps4. With rate-limit 800 pps and 2 members: Member-1 CPU-received rate increases to approximately 400 pps (800/2)5. Configuration change takes effect immediately without stack restart |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P1 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-6. Runtime CoPP modification must propagate to all members. |

#### 1.4.3.5 Rate Consistency Across All Members After Config Change

| Field | Content |
| --- | --- |
| Test Name | MOD4-016: Verify CoPP rate is consistent across all stack members after configuration change |
| Purpose Of The Test | Verify that after changing the CoPP rate-limit on the master, all stack members (master, standby, and regular members) enforce the identical per-device rate — no member has stale or inconsistent rate values |
| Test Topo & Precondition | Topology:TG1 ---- (port 1) Member-1 (Master) ==stack-link== Member-2 (Standby) ==stack-link== Member-3 (Member) (port 1) ---- TG3TG2 ---- (port 1) Member-2Preconditions:1. PicOS 4.6 installed on all three switches2. Three switches configured as a stack with Member-1 as master3. Stack is fully formed and stable4. TG1, TG2, TG3 connected to Member-1, Member-2, Member-3 respectivelyConfiguration:Stack Master:set protocols copp rate-limit 300commit |
| Test Procedure | 1. Execute run show copp rate-limit to confirm initial rate-limit is 300 pps2. Execute run show stack status to confirm 3 members active3. Configure new CoPP rate-limit:set protocols copp rate-limit 900commit4. Wait 10 seconds for configuration to propagate to all members5. Execute run show copp rate-limit to confirm new rate-limit is 900 pps6. Configure TG1 to send ARP-request traffic at 500 pps towards Member-1 port 17. Wait 30 seconds for rate to stabilize8. Execute run show copp statistics to record Member-1 CPU-received rate9. Stop TG1 traffic10. Configure TG2 to send ARP-request traffic at 500 pps towards Member-2 port 111. Wait 30 seconds for rate to stabilize12. Execute run show copp statistics to record Member-2 CPU-received rate13. Stop TG2 traffic14. Configure TG3 to send ARP-request traffic at 500 pps towards Member-3 port 115. Wait 30 seconds for rate to stabilize16. Execute run show copp statistics to record Member-3 CPU-received rate17. Stop TG3 traffic |
| Expected Results | 1. run show copp rate-limit shows 900 pps after change2. Member-1 CPU-received rate is approximately 300 pps (900/3)3. Member-2 CPU-received rate is approximately 300 pps (900/3)4. Member-3 CPU-received rate is approximately 300 pps (900/3)5. All three members enforce identical per-device rates (no stale configuration)6. Deviation between any two members is less than 5% |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P1 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-6. Configuration consistency across stack is critical — stale rates on any member can cause unpredictable CoPP behavior. |

### 1.4.4 CoPP Burst and Stress

#### 1.4.4.1 Single Member Exceeds Its Rate Share

| Field | Content |
| --- | --- |
| Test Name | MOD4-017: Verify single member drops excess traffic when receiving more than its CoPP share (total/N) |
| Purpose Of The Test | Verify that when a single stack member receives control plane traffic exceeding its per-device CoPP share (total/N), the excess packets are dropped locally and do not affect other members’ CoPP budgets |
| Test Topo & Precondition | Topology:TG1 ---- (port 1) Member-1 (Master) ==stack-link== Member-2 (Standby) ==stack-link== Member-3 ==stack-link== Member-4 (port 1) ---- TG4TG2 ---- (port 1) Member-2TG3 ---- (port 1) Member-3Preconditions:1. PicOS 4.6 installed on all four switches2. Four switches configured as a stack with Member-1 as master3. Stack is fully formed and stable4. TG1–TG4 connected to Member-1 through Member-4Configuration:Stack Master:set protocols copp rate-limit 500commit |
| Test Procedure | 1. Execute run show copp rate-limit on the stack master (500 pps, per-device = 125 pps)2. Execute run show stack status to confirm 4 members active3. Configure TG1 to send ARP-request traffic at 500 pps towards Member-1 port 1 (4x its share)4. Configure TG2 to send ARP-request traffic at 50 pps towards Member-2 port 1 (under its share)5. Start TG1 and TG2 simultaneously6. Wait 60 seconds for rate to stabilize7. Execute run show copp statistics to record CPU-received and dropped counters for Member-1 and Member-28. Stop all traffic |
| Expected Results | 1. Member-1 CPU-received rate is approximately 125 pps (500/4), NOT 500 pps — drops approximately 375 pps2. Member-2 CPU-received rate is approximately 50 pps (below its 125 pps share) — drops 0 pps3. Member-1’s excess traffic does NOT consume Member-2’s unused CoPP budget4. Each member’s rate limiting is independent and enforced locally5. run show copp statistics on Member-1 shows a high drop counter; Member-2 shows zero or near-zero drops |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-4. PicOS design: rate is divided and enforced per device. Unused budget on one member cannot be borrowed by another. |

#### 1.4.4.2 All Members Simultaneously Exceed Display Value

| Field | Content |
| --- | --- |
| Test Name | MOD4-018: Verify CoPP behavior when all members simultaneously receive traffic exceeding the display value |
| Purpose Of The Test | Verify system stability when all stack members simultaneously receive control plane traffic that, in aggregate, far exceeds the configured CoPP rate-limit display value — each member should independently enforce its share |
| Test Topo & Precondition | Topology:TG1 ---- (port 1) Member-1 (Master) ==stack-link== Member-2 (Standby) ==stack-link== Member-3 ==stack-link== Member-4 (port 1) ---- TG4TG2 ---- (port 1) Member-2TG3 ---- (port 1) Member-3Preconditions:1. PicOS 4.6 installed on all four switches2. Four switches configured as a stack with Member-1 as master3. Stack is fully formed and stable4. TG1–TG4 connected to Member-1 through Member-4Configuration:Stack Master:set protocols copp rate-limit 500commit |
| Test Procedure | 1. Execute run show copp rate-limit on the stack master (500 pps)2. Execute run show stack status to confirm 4 members active3. Configure TG1, TG2, TG3, and TG4 to each send ARP-request traffic at 1000 pps towards their respective member ports (4000 pps total, 8x display value)4. Start all four traffic generators simultaneously5. Wait 60 seconds for rate to stabilize6. Execute run show copp statistics to record CPU-received and dropped counters for all 4 members7. Execute run show cpu-utilization on the stack master to record CPU usage8. Execute run show stack status to confirm stack stability (no member departures)9. Stop all traffic generators10. Wait 30 seconds11. Execute run show stack status to confirm stack remains stable |
| Expected Results | 1. Each member’s CPU-received rate is approximately 125 pps (500/4 = 125 pps)2. Each member drops approximately 875 pps (1000 - 125 = 875)3. Total aggregate CPU-received rate across all members is approximately 500 pps4. CPU utilization on stack master does not exceed critical threshold (e.g., 90%)5. run show stack status shows all 4 members remain active — no member ejected due to overload6. Stack remains stable throughout the stress test |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-4, TP-5. Stress test for extreme overload scenario. Stack stability under sustained attack-level traffic is critical. |

#### 1.4.4.3 Protocol Flap Under CoPP Limiting in Stack

| Field | Content |
| --- | --- |
| Test Name | MOD4-019: Verify protocol session stability under CoPP rate limiting in a stack environment |
| Purpose Of The Test | Verify that OSPF adjacency and LACP bundle remain stable when legitimate protocol traffic competes with high-volume control plane noise under CoPP rate limiting in a stack |
| Test Topo & Precondition | Topology:Router-A (OSPF, IP: 10.1.1.2/24) ---- (port 1) Member-1 (Master, IP: 10.1.1.1/24) ==stack-link== Member-2 (Standby) (port 1, LAG ae1) ---- Partner-Switch (LAG)TG1 ---- (port 2) Member-1TG2 ---- (port 1) Member-2 (port 2) ---- TG3Preconditions:1. PicOS 4.6 installed on both stack members2. Two switches configured as a stack with Member-1 as master3. OSPF adjacency established between Member-1 port 1 and Router-A4. LACP bundle (ae1) established between Member-2 port 1 and Partner-Switch5. TG1 connected to Member-1 port 2, TG2/TG3 connected to Member-2 portsConfiguration:Stack Master:set protocols copp rate-limit 200set protocols ospf area 0.0.0.0 interface ge-1/1/1set interface ge-2/1/1 ether-options 802.3ad ae1set interface ae1 aggregated-ether-options lacp activecommit |
| Test Procedure | 1. Execute run show ospf neighbor to confirm OSPF adjacency is FULL with Router-A2. Execute run show lacp interface ae1 to confirm LACP bundle is active3. Execute run show copp rate-limit to confirm 200 pps (per-device = 100 pps)4. Configure TG1 to send mixed broadcast/multicast traffic (ARP + random multicast) at 500 pps towards Member-1 port 25. Configure TG2 to send mixed broadcast/multicast traffic at 500 pps towards Member-26. Start TG1 and TG2 simultaneously7. Wait 300 seconds (5 minutes) with traffic running8. Execute run show ospf neighbor to record OSPF adjacency state9. Execute run show lacp interface ae1 to record LACP bundle state10. Execute run show copp statistics to record drop counters for all queues11. Execute run show log messages to search for OSPF adjacency flap or LACP timeout log entries12. Stop all traffic generators |
| Expected Results | 1. CoPP drops noise traffic — drop counters show significant drops on both members2. OSPF adjacency state: If OSPF Hello rate fits within the 100 pps per-device OSPF queue allocation, adjacency remains FULL; if OSPF queue is shared with noise and drops OSPF Hellos, adjacency may flap — log entries show “OSPF neighbor down”3. LACP bundle state: If LACP PDU rate fits within the per-device LACP queue allocation, bundle remains active; if LACP PDUs are dropped, bundle may flap — log entries show “LACP timeout”4. Protocol flap behavior is documented: specific queue saturation conditions that cause flaps are identified5. run show log messages entries correlate with CoPP drop events |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P1 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-4, TP-6. This test documents the threshold at which CoPP rate division causes protocol instability. Critical for capacity planning. |

#### 1.4.4.4 Long-Duration CoPP Rate Accuracy

| Field | Content |
| --- | --- |
| Test Name | MOD4-020: Verify CoPP rate accuracy remains stable over 1-hour sustained traffic |
| Purpose Of The Test | Verify that the per-device CoPP rate (total/N) is maintained accurately over a sustained 1-hour period without drift, memory leak, or counter overflow affecting the rate enforcement |
| Test Topo & Precondition | Topology:TG1 ---- (port 1) Member-1 (Master) ==stack-link== Member-2 (Standby) (port 1) ---- TG2Preconditions:1. PicOS 4.6 installed on both switches2. Two switches configured as a stack with Member-1 as master3. Stack is fully formed and stable4. TG1 and TG2 connected to Member-1 and Member-2 respectively5. System uptime > 10 minutes (past initial stabilization)Configuration:Stack Master:set protocols copp rate-limit 500commit |
| Test Procedure | 1. Execute run show copp rate-limit to confirm 500 pps2. Execute run show stack status to confirm 2 members active3. Execute run show copp statistics to record baseline counters (T0)4. Configure TG1 to send ARP-request traffic at 400 pps towards Member-1 port 15. Configure TG2 to send ARP-request traffic at 400 pps towards Member-2 port 16. Start TG1 and TG2 simultaneously7. Wait 900 seconds (15 minutes)8. Execute run show copp statistics to record counters (T1)9. Wait 900 seconds (15 minutes, cumulative 30 minutes)10. Execute run show copp statistics to record counters (T2)11. Wait 900 seconds (15 minutes, cumulative 45 minutes)12. Execute run show copp statistics to record counters (T3)13. Wait 900 seconds (15 minutes, cumulative 60 minutes)14. Execute run show copp statistics to record counters (T4)15. Execute run show cpu-utilization to record CPU usage after 1 hour16. Execute run show system memory to record memory usage after 1 hour17. Stop all traffic generators |
| Expected Results | 1. At each 15-minute interval (T1, T2, T3, T4), Member-1 CPU-received rate is approximately 250 pps (500/2) — calculated as delta/interval2. At each interval, Member-2 CPU-received rate is approximately 250 pps (500/2)3. Rate deviation between any two intervals is less than 5%4. No rate drift over time (T4 rate ≈ T1 rate)5. CPU utilization remains stable — no gradual increase indicating a leak6. Memory usage remains stable — no gradual increase indicating a leak7. CoPP counters do not overflow (64-bit counters expected) |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P1 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-4. Long-duration stability test. Counter overflow or memory leaks may not surface in short tests. |

### 1.4.5 PicOS vs Ruijie Comparison Points

#### 1.4.5.1 PicOS Single Member Rate Equals Total Divided by N

| Field | Content |
| --- | --- |
| Test Name | MOD4-021: Document and verify PicOS CoPP behavior — single member rate equals total/N |
| Purpose Of The Test | Explicitly verify and document the PicOS CoPP stacking design: in an N-device stack, each device’s effective CoPP rate is CLI_display_value / N. This is the defining behavioral characteristic of the PicOS implementation |
| Test Topo & Precondition | Topology:TG1 ---- (port 1) Member-1 (Master) ==stack-link== Member-2 (Standby) ==stack-link== Member-3 ==stack-link== Member-4 (port 1) ---- TG4TG2 ---- (port 1) Member-2TG3 ---- (port 1) Member-3Preconditions:1. PicOS 4.6 installed on all four switches2. Four switches configured as a stack with Member-1 as master3. Stack is fully formed and stable4. TG1 connected to Member-1 only (single member targeted)Configuration:Stack Master:set protocols copp rate-limit 1000commit |
| Test Procedure | 1. Execute run show copp rate-limit to confirm 1000 pps2. Execute run show stack status to confirm 4 members active3. Configure TG1 to send ARP-request traffic at 500 pps towards Member-1 port 14. Wait 30 seconds for rate to stabilize5. Execute run show copp statistics to record Member-1 CPU-received rate6. Stop TG1 traffic7. Configure TG1 to send ARP-request traffic at 250 pps towards Member-1 port 18. Wait 30 seconds for rate to stabilize9. Execute run show copp statistics to record Member-1 CPU-received rate10. Stop TG1 traffic11. Configure TG1 to send ARP-request traffic at 200 pps towards Member-1 port 112. Wait 30 seconds for rate to stabilize13. Execute run show copp statistics to record Member-1 CPU-received rate14. Stop TG1 traffic |
| Expected Results | 1. run show copp rate-limit displays 1000 pps (CLI display value)2. At 500 pps input: Member-1 CPU-received rate is approximately 250 pps (1000/4 = 250), drops approximately 250 pps — NOT 500 pps or 1000 pps3. At 250 pps input: Member-1 CPU-received rate is approximately 250 pps (at its limit, 0 drops)4. At 200 pps input: Member-1 CPU-received rate is approximately 200 pps (below its limit, 0 drops)5. PicOS behavior confirmed: Each device is hard-limited to total/N regardless of other members’ utilization |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-4. Reference test for PicOS vs Ruijie comparison. This documents PicOS’s fixed-division model. |

#### 1.4.5.2 Document Expected Ruijie Behavior for Comparison

| Field | Content |
| --- | --- |
| Test Name | MOD4-022: Document expected Ruijie CoPP behavior — each member full rate with master scheduling |
| Purpose Of The Test | Document the Ruijie CoPP stacking design for comparison: in Ruijie’s implementation, each device can send up to the full CLI display rate to the CPU, and the master handles scheduling. This test documents the behavioral difference from PicOS |
| Test Topo & Precondition | Topology:TG1 ---- (port 1) Ruijie-Member-1 (Master) ==stack-link== Ruijie-Member-2 (Standby) ==stack-link== Ruijie-Member-3 ==stack-link== Ruijie-Member-4 (port 1) ---- TG4TG2 ---- (port 1) Ruijie-Member-2TG3 ---- (port 1) Ruijie-Member-3Preconditions:1. Ruijie switch platform with stacking CoPP support2. Four switches configured as a stack with Member-1 as master3. Stack is fully formed and stable4. TG1 connected to Member-1 only (single member targeted)5. NOTE: This test is for documentation/comparison only — execute only if Ruijie hardware is availableConfiguration:Stack Master:cpu-protect policyrate-limit arp 1000 (Ruijie CLI syntax — adjust per actual platform)exit |
| Test Procedure | 1. Execute show cpu-protect policy (Ruijie CLI) to confirm rate-limit is 1000 pps2. Execute show switch (Ruijie CLI) to confirm 4 members active3. Configure TG1 to send ARP-request traffic at 1500 pps towards Member-1 port 14. Wait 30 seconds for rate to stabilize5. Execute show cpu-protect statistics (Ruijie CLI) to record Member-1 CPU-received rate6. Stop TG1 traffic7. Configure TG1 to send ARP-request traffic at 800 pps towards Member-1 port 18. Wait 30 seconds for rate to stabilize9. Execute show cpu-protect statistics (Ruijie CLI) to record Member-1 CPU-received rate10. Stop TG1 traffic |
| Expected Results | 1. show cpu-protect policy displays rate-limit as 1000 pps2. Ruijie expected behavior: At 1500 pps input, Member-1 CPU-received rate is approximately 1000 pps (full display value, NOT 250 pps = 1000/4)3. At 800 pps input: Member-1 CPU-received rate is approximately 800 pps (below limit, 0 drops)4. Ruijie design: Each member can individually consume up to the full CLI display rate; the master handles aggregate scheduling across the stack5. Comparison conclusion: PicOS allows 250 pps per device in a 4-stack with 1000 pps limit; Ruijie allows 1000 pps per device with master scheduling |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P2 |
| Hardware Model | Ruijie Stack-capable Switches (comparison reference) |
| Version | Ruijie OS (version TBD) |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-4. Comparison documentation only. Execute if Ruijie hardware is available. Results inform PicOS design trade-off analysis. |

#### 1.4.5.3 PicOS Behavior When Only One Member Receives All Traffic

| Field | Content |
| --- | --- |
| Test Name | MOD4-023: Verify PicOS CoPP behavior when only one member receives all control plane traffic |
| Purpose Of The Test | Verify that in PicOS, when only one stack member receives control plane traffic and other members are idle, the receiving member is still limited to total/N — it cannot use other members’ unused CoPP budget (unlike Ruijie where the single member could use the full rate) |
| Test Topo & Precondition | Topology:TG1 ---- (port 1) Member-1 (Master) ==stack-link== Member-2 (Standby) ==stack-link== Member-3 ==stack-link== Member-4(No traffic generators connected to Member-2, Member-3, Member-4)Preconditions:1. PicOS 4.6 installed on all four switches2. Four switches configured as a stack with Member-1 as master3. Stack is fully formed and stable4. Only TG1 is connected to Member-1; no external traffic sources on other members5. Member-2, Member-3, Member-4 have zero control plane traffic ingressConfiguration:Stack Master:set protocols copp rate-limit 1000commit |
| Test Procedure | 1. Execute run show copp rate-limit to confirm 1000 pps2. Execute run show stack status to confirm 4 members active3. Execute run show copp statistics to confirm zero traffic counters on Members 2, 3, and 44. Configure TG1 to send ARP-request traffic at 1000 pps towards Member-1 port 15. Wait 60 seconds for rate to stabilize6. Execute run show copp statistics to record CPU-received and dropped counters for Member-17. Execute run show copp statistics to confirm Members 2, 3, 4 still have zero or near-zero traffic8. Stop TG1 traffic |
| Expected Results | 1. Member-1 CPU-received rate is approximately 250 pps (1000/4 = 250) — NOT 1000 pps2. Member-1 drops approximately 750 pps (1000 - 250 = 750)3. Members 2, 3, 4 have zero CPU-received and zero drops4. PicOS design limitation confirmed: Unused CoPP budget on idle members (750 pps total unused) is NOT redistributable to Member-15. Total effective system capacity is 250 pps (only 1 out of 4 shares utilized) despite 1000 pps configured6. Contrast with Ruijie: In Ruijie’s implementation, Member-1 would receive up to 1000 pps in this scenario |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-4. This is the key behavioral difference from Ruijie. Documents the worst-case scenario for PicOS CoPP in stacking. |

#### 1.4.5.4 Total System Capacity Across All Members

| Field | Content |
| --- | --- |
| Test Name | MOD4-024: Verify total CoPP system capacity across all stack members equals CLI display value |
| Purpose Of The Test | Verify that when all stack members are simultaneously receiving traffic and each is exactly at its per-device CoPP limit, the total aggregate CPU-received rate across the entire stack equals the CLI display value |
| Test Topo & Precondition | Topology:TG1 ---- (port 1) Member-1 (Master) ==stack-link== Member-2 (Standby) ==stack-link== Member-3 ==stack-link== Member-4 (port 1) ---- TG4TG2 ---- (port 1) Member-2TG3 ---- (port 1) Member-3Preconditions:1. PicOS 4.6 installed on all four switches2. Four switches configured as a stack with Member-1 as master3. Stack is fully formed and stable4. TG1–TG4 connected to Member-1 through Member-4 respectivelyConfiguration:Stack Master:set protocols copp rate-limit 1000commit |
| Test Procedure | 1. Execute run show copp rate-limit to confirm 1000 pps2. Execute run show stack status to confirm 4 members active3. Execute run show copp statistics to record baseline counters for all members (T0)4. Configure TG1, TG2, TG3, TG4 to each send ARP-request traffic at exactly 250 pps towards their respective member ports (per-device share = 1000/4 = 250 pps)5. Start all four traffic generators simultaneously6. Wait 60 seconds for rate to stabilize7. Execute run show copp statistics to record counters for all members (T1)8. Calculate per-member CPU-received rate and aggregate rate from counter deltas9. Stop all traffic generators10. Configure TG1, TG2, TG3, TG4 to each send at 300 pps (above per-device share)11. Start all four traffic generators simultaneously12. Wait 60 seconds for rate to stabilize13. Execute run show copp statistics to record counters for all members (T2)14. Calculate per-member and aggregate rates15. Stop all traffic generators |
| Expected Results | 1. At 250 pps per member (at exact share): Each member CPU-received rate is approximately 250 pps, drops are 0 or near-zero; aggregate rate is approximately 1000 pps (= CLI display value)2. At 300 pps per member (above share): Each member CPU-received rate is approximately 250 pps, drops approximately 50 pps each; aggregate rate is approximately 1000 pps (= CLI display value)3. PicOS total system capacity: When all members are fully utilized, the aggregate rate equals the CLI display value — the display value represents the TOTAL system budget, not a per-device budget4. This confirms the PicOS formula: per-device rate = CLI_display / N, total system capacity = CLI_display |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-4. Validates the fundamental CoPP stacking equation. Contrast with Ruijie where total system capacity = CLI_display × N. |

#### 1.4.5.5 Impact on Protocol Convergence Under CoPP Limiting

| Field | Content |
| --- | --- |
| Test Name | MOD4-025: Verify impact of PicOS CoPP rate division on protocol convergence time in stack |
| Purpose Of The Test | Measure the impact of the PicOS CoPP rate division on OSPF convergence time after a link failure event. In a 4-device stack with rate-limit 1000 pps, each device’s OSPF queue allows only 1/4 of the configured rate — this may slow convergence compared to non-stacked or Ruijie-style implementations |
| Test Topo & Precondition | Topology:Router-A (OSPF, area 0, IP: 10.1.1.2/24) ---- (port 1) Member-1 (Master, IP: 10.1.1.1/24) ==stack-link== Member-2 (Standby) ==stack-link== Member-3 ==stack-link== Member-4 (IP: 10.4.4.1/24) (port 1) ---- Router-B (OSPF, area 0, IP: 10.4.4.2/24)TG1 ---- (port 2) Member-1TG2 ---- (port 1) Member-2TG3 ---- (port 1) Member-3TG4 ---- (port 2) Member-4Preconditions:1. PicOS 4.6 installed on all four stack switches2. Four switches configured as a stack with Member-1 as master3. OSPF area 0 configured between stack and Router-A / Router-B4. OSPF adjacencies are FULL and routes are stable5. Router-B advertises prefix 192.168.0.0/16 via OSPF6. TG1–TG4 generate background CoPP-targeted traffic (ARP broadcast)7. Data-plane traffic from Router-A to 192.168.0.0/16 via stack to Router-BConfiguration:Stack Master:set protocols copp rate-limit 400set protocols ospf area 0.0.0.0 interface ge-1/1/1set protocols ospf area 0.0.0.0 interface ge-4/1/1commit |
| Test Procedure | 1. Execute run show ospf neighbor to confirm FULL adjacency with Router-A and Router-B2. Execute run show ospf route 192.168.0.0/16 to confirm route is present via Router-B3. Execute run show copp rate-limit to confirm 400 pps (per-device = 100 pps in 4-stack)4. Configure TG1, TG2, TG3, TG4 to each send ARP-request broadcast at 200 pps towards their respective member ports (saturating CoPP per-device share)5. Start all traffic generators6. Wait 60 seconds for CoPP rates to stabilize7. Execute run show copp statistics queue ospf to record OSPF queue drop counters8. Shutdown the link between Member-4 and Router-B:set interface ge-4/1/1 disablecommit9. Start a timer to measure convergence10. Execute run show ospf neighbor repeatedly at 1-second intervals to detect when Router-B adjacency transitions to Down11. Execute run show ospf route 192.168.0.0/16 repeatedly at 1-second intervals to detect when the route is withdrawn12. Record the convergence time (from link shutdown to route withdrawal)13. Stop all traffic generators14. Re-enable the link:delete interface ge-4/1/1 disablecommit15. Wait 60 seconds for OSPF adjacency to re-establish |
| Expected Results | 1. OSPF adjacencies are FULL before the test2. With CoPP background traffic: run show copp statistics queue ospf shows non-zero drops on Member-4 (OSPF LSA floods competing with background noise)3. Convergence time after link shutdown is recorded — expected to be longer than non-stacked baseline due to OSPF queue receiving only 100 pps (400/4) instead of 400 pps4. Route 192.168.0.0/16 is eventually withdrawn — convergence completes even under CoPP pressure5. Convergence time is documented for comparison with: (a) non-stacked single device with 400 pps CoPP, (b) Ruijie stack where each device gets full 400 pps OSPF queue6. OSPF adjacency re-establishes after link is restored7. Documented trade-off: PicOS CoPP division may increase convergence time proportionally to stack size |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P1 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-4, TP-6. Critical for understanding the operational impact of PicOS CoPP division on network convergence. Results should be compared with Ruijie baseline if available. |

## Coverage Summary

### Coverage by Sub-Area

| Sub-Area | Description | Test Cases | P0 | P1 | P2 |
| --- | --- | --- | --- | --- | --- |
| 1.4.1 | Basic CoPP Rate Division | 5 | 4 | 0 | 0 |
| 1.4.2 | Per-Queue CoPP Verification | 6 | 5 | 1 | 0 |
| 1.4.3 | CoPP Rate After Stack Events | 5 | 3 | 2 | 0 |
| 1.4.4 | CoPP Burst and Stress | 4 | 2 | 2 | 0 |
| 1.4.5 | PicOS vs Ruijie Comparison Points | 5 | 3 | 1 | 1 |
| Total |  | 25 | 17 | 6 | 1 |

### Coverage by Priority

| Priority | Count | Percentage |
| --- | --- | --- |
| P0 | 17 | 68% |
| P1 | 6 | 24% |
| P2 | 1 | 4% |
| Total | 25 | 100% |

### Coverage by Developer Requirement

| Requirement | Description | Covered By Test Cases |
| --- | --- | --- |
| TP-4 | CoPP rate division formula (total/N) | MOD4-001 through MOD4-011, MOD4-017 through MOD4-025 |
| TP-5 | Dynamic rate adjustment on stack topology change | MOD4-005, MOD4-012 through MOD4-014, MOD4-018 |
| TP-6 | Runtime CoPP configuration change propagation | MOD4-015, MOD4-016, MOD4-019, MOD4-025 |

### Risk Coverage Matrix

| Risk Area | Risk Level | Test Cases Covering | Count | Sufficient? |
| --- | --- | --- | --- | --- |
| Rate division correctness | P0 | MOD4-001,002,003,004,005,021,023,024 | 8 | Yes (≥8) |
| Per-queue enforcement | P0 | MOD4-006,007,008,009,010,011 | 6 | Yes (≥5) |
| Stack event handling | P0 | MOD4-012,013,014,015,016 | 5 | Yes (≥5) |
| Stress/overload stability | P1 | MOD4-017,018,019,020 | 4 | Yes (≥4) |
| Vendor comparison documentation | P2 | MOD4-022 | 1 | N/A (documentation) |
| Protocol convergence impact | P1 | MOD4-019,025 | 2 | Yes (≥2) |

### Show Commands Coverage

| Show Command | Used In Test Cases |
| --- | --- |
| run show copp rate-limit | All 25 test cases |
| run show copp statistics | MOD4-001 through MOD4-025 |
| run show copp statistics queue <name> | MOD4-006 through MOD4-011, MOD4-019, MOD4-025 |
| run show stack status | All 25 test cases |
| run show cpu-utilization | MOD4-018, MOD4-020 |
| run show system memory | MOD4-020 |
| run show ospf neighbor | MOD4-019, MOD4-025 |
| run show lacp interface | MOD4-019 |
| run show ospf route | MOD4-025 |
| run show log messages | MOD4-019 |

### Key PicOS vs Ruijie Design Differences (Documented)

| Aspect | PicOS Behavior | Ruijie Behavior |
| --- | --- | --- |
| Per-device CoPP rate | CLI_display / N | CLI_display (full rate) |
| Total system capacity | CLI_display | CLI_display × N |
| Rate redistribution | Not supported (fixed division) | Master scheduling allows dynamic allocation |
| Single-member traffic scenario | Limited to CLI_display / N | Can use full CLI_display |
| Impact on protocol convergence | Proportional to N (more members = lower per-device rate) | Minimal (each member has full rate) |