# PicOS Stacking Functional Test Cases — MOD-1 & MOD-2

Platform: PicOS
Feature: Port Pre-Configuration & Inband Management
Generated: 2026-03-16
Developer Requirement: TP-1, TP-2
Total Cases: 27

## Table of Contents

- 1. FUNCTIONAL TESTING

  - 1.1 Port Pre-Configuration (TP-1)

    - 1.1.1 Basic Pre-Configuration

    - 1.1.2 Module Feature Interaction

    - 1.1.3 Pre-Config Persistence

  - 1.2 Inband Management (TP-2)

    - 1.2.1 Basic Inband Connectivity

    - 1.2.2 Inband During Stack Events

    - 1.2.3 Inband Stability

- Coverage Summary

## 1. FUNCTIONAL TESTING

### 1.1 Port Pre-Configuration (TP-1)

#### 1.1.1 Basic Pre-Configuration

##### 1.1.1.1 Pre-configure port on offline member — member joins and config applied

| Field | Content |
| --- | --- |
| Test Name | PRECONFIG-01: Port pre-config applied on member join |
| Purpose Of The Test | Verify that a port configured on an offline stack member is automatically applied when that member joins the stack |
| Test Topo & Precondition | Topology:Master (Unit 1) ---- [Stack Link] ---- Standby (Unit 2)Member (Unit 3) — disconnected from stackHost-A ---- (ge-1/0/3 on Unit 3)Preconditions:1. PicOS 4.6 running on all switches2. Unit 1 is master, Unit 2 is standby, both operational3. Unit 3 is powered off and disconnected from the stack4. Host-A is connected to port ge-1/0/3 on Unit 3Configuration:Master (Unit 1):set interface gigabit-ethernet ge-1/0/3 description "Pre-configured-for-Unit3"set interface gigabit-ethernet ge-1/0/3 speed 1000set interface gigabit-ethernet ge-1/0/3 mtu 9216set interface gigabit-ethernet ge-1/0/3 enable truecommit |
| Test Procedure | 1. Configure port ge-1/0/3 on Unit 3 from the master (Unit 1) while Unit 3 is offline2. Execute run show running-config interface ge-1/0/3 on the master3. Connect Unit 3 to the stack via stack cable4. Power on Unit 35. Wait 120 seconds for Unit 3 to boot and join the stack6. Execute run show stack on the master7. Execute run show interface ge-1/0/3 on the master |
| Expected Results | 1. Step 2: show running-config displays pre-configured description, speed, MTU, and enable state for ge-1/0/32. Step 6: show stack displays Unit 3 in “Ready” state with role “Member”3. Step 7: show interface ge-1/0/3 displays description “Pre-configured-for-Unit3”, speed 1000 Mbps, MTU 9216, and admin status “up” |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-1 |

##### 1.1.1.2 Pre-configure VLAN assignment on offline member port

| Field | Content |
| --- | --- |
| Test Name | PRECONFIG-02: VLAN assignment pre-config on offline member |
| Purpose Of The Test | Verify that VLAN membership configured on an offline member port is applied when the member joins the stack |
| Test Topo & Precondition | Topology:Master (Unit 1) ---- [Stack Link] ---- Standby (Unit 2)Member (Unit 3) — disconnected from stackHost-A (VLAN 100) ---- (ge-1/0/3 on Unit 3)Host-B (VLAN 100) ---- (ge-1/0/1 on Unit 1)Preconditions:1. PicOS 4.6 running on all switches2. Unit 1 is master, Unit 2 is standby, both operational3. Unit 3 is powered off and disconnected from the stack4. VLAN 100 is created on the stack5. Host-A and Host-B are configured with IP addresses in the same subnet (10.10.100.0/24)Configuration:Master (Unit 1):set vlans vlan-id 100 description "Test-VLAN"set interface gigabit-ethernet ge-1/0/3 family ethernet-switching port-mode accessset interface gigabit-ethernet ge-1/0/3 family ethernet-switching vlan members 100set interface gigabit-ethernet ge-1/0/1 family ethernet-switching port-mode accessset interface gigabit-ethernet ge-1/0/1 family ethernet-switching vlan members 100commit |
| Test Procedure | 1. Configure VLAN 100 and assign ge-1/0/3 (Unit 3) and ge-1/0/1 (Unit 1) as access ports in VLAN 100 from the master while Unit 3 is offline2. Execute run show vlans on the master3. Execute run show running-config interface ge-1/0/3 on the master4. Connect Unit 3 to the stack via stack cable and power on5. Wait 120 seconds for Unit 3 to boot and join the stack6. Execute run show stack on the master7. Execute run show vlans vlan-id 100 on the master8. Execute run show interface ge-1/0/3 on the master9. Initiate ping from Host-A (10.10.100.2) to Host-B (10.10.100.1) |
| Expected Results | 1. Step 2: VLAN 100 exists in the VLAN table with description “Test-VLAN”2. Step 3: ge-1/0/3 shows access mode with VLAN 100 membership in running-config3. Step 6: Unit 3 appears in “Ready” state4. Step 7: VLAN 100 member list includes both ge-1/0/1 and ge-1/0/35. Step 8: ge-1/0/3 shows admin status “up”, access mode, VLAN 1006. Step 9: Ping from Host-A to Host-B succeeds with 0% packet loss |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-1 |

##### 1.1.1.3 Pre-configure IP address on offline member L3 interface

| Field | Content |
| --- | --- |
| Test Name | PRECONFIG-03: L3 IP address pre-config on offline member |
| Purpose Of The Test | Verify that an IP address configured on a routed interface of an offline member is applied when the member joins the stack |
| Test Topo & Precondition | Topology:Master (Unit 1) ---- [Stack Link] ---- Standby (Unit 2)Member (Unit 3) — disconnected from stackRouter-A (192.168.10.2/24) ---- (ge-1/0/3 on Unit 3, 192.168.10.1/24)Preconditions:1. PicOS 4.6 running on all switches2. Unit 1 is master, Unit 2 is standby, both operational3. Unit 3 is powered off and disconnected from the stack4. Router-A interface is configured with 192.168.10.2/24Configuration:Master (Unit 1):set interface gigabit-ethernet ge-1/0/3 family inet address 192.168.10.1/24set interface gigabit-ethernet ge-1/0/3 routing-interface enableset interface gigabit-ethernet ge-1/0/3 enable truecommit |
| Test Procedure | 1. Configure L3 IP address 192.168.10.1/24 on ge-1/0/3 (Unit 3) from the master while Unit 3 is offline2. Execute run show running-config interface ge-1/0/3 on the master3. Connect Unit 3 to the stack via stack cable and power on4. Wait 120 seconds for Unit 3 to boot and join the stack5. Execute run show stack on the master6. Execute run show interface ge-1/0/3 on the master7. Execute run show ip interface brief on the master8. Initiate ping from Router-A (192.168.10.2) to 192.168.10.1 |
| Expected Results | 1. Step 2: Running-config shows IP address 192.168.10.1/24 and routing-interface enabled on ge-1/0/32. Step 5: Unit 3 appears in “Ready” state with role “Member”3. Step 6: ge-1/0/3 shows admin “up”, link “up”, IP address 192.168.10.1/244. Step 7: ge-1/0/3 listed with IP 192.168.10.1, status “up/up”5. Step 8: Ping from Router-A to 192.168.10.1 succeeds with 0% packet loss |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-1 |

##### 1.1.1.4 Remove pre-configured port config before member joins

| Field | Content |
| --- | --- |
| Test Name | PRECONFIG-04: Remove pre-config before member join |
| Purpose Of The Test | Verify that removing a pre-configured port configuration while the member is still offline results in the member joining with default port settings |
| Test Topo & Precondition | Topology:Master (Unit 1) ---- [Stack Link] ---- Standby (Unit 2)Member (Unit 3) — disconnected from stackHost-A ---- (ge-1/0/3 on Unit 3)Preconditions:1. PicOS 4.6 running on all switches2. Unit 1 is master, Unit 2 is standby, both operational3. Unit 3 is powered off and disconnected from the stack4. Port ge-1/0/3 has been pre-configured with description, speed, and VLAN 100 assignmentConfiguration:Master (Unit 1) — pre-existing:set interface gigabit-ethernet ge-1/0/3 description "Pre-configured-for-Unit3"set interface gigabit-ethernet ge-1/0/3 speed 1000set interface gigabit-ethernet ge-1/0/3 family ethernet-switching vlan members 100commit |
| Test Procedure | 1. Execute run show running-config interface ge-1/0/3 on the master to confirm pre-configuration exists2. Execute delete interface gigabit-ethernet ge-1/0/3 on the master3. Execute commit on the master4. Execute run show running-config interface ge-1/0/3 on the master5. Connect Unit 3 to the stack via stack cable and power on6. Wait 120 seconds for Unit 3 to boot and join the stack7. Execute run show stack on the master8. Execute run show interface ge-1/0/3 on the master |
| Expected Results | 1. Step 1: Running-config shows the previously configured description, speed, and VLAN membership for ge-1/0/32. Step 4: Running-config shows no custom configuration for ge-1/0/3 (interface reverts to default)3. Step 7: Unit 3 appears in “Ready” state with role “Member”4. Step 8: ge-1/0/3 shows default settings — no custom description, auto speed negotiation, default VLAN |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P1 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-1 |

##### 1.1.1.5 Pre-configure multiple ports on same offline member

| Field | Content |
| --- | --- |
| Test Name | PRECONFIG-05: Multiple port pre-configs on same offline member |
| Purpose Of The Test | Verify that multiple ports on the same offline member can be pre-configured simultaneously and all configurations are applied when the member joins |
| Test Topo & Precondition | Topology:Master (Unit 1) ---- [Stack Link] ---- Standby (Unit 2)Member (Unit 3) — disconnected from stackHost-A (VLAN 100) ---- (ge-1/0/3 on Unit 3)Host-B (VLAN 200) ---- (ge-1/0/4 on Unit 3)Host-C ---- (ge-1/0/5 on Unit 3, trunk port)Preconditions:1. PicOS 4.6 running on all switches2. Unit 1 is master, Unit 2 is standby, both operational3. Unit 3 is powered off and disconnected from the stack4. VLANs 100 and 200 are created on the stackConfiguration:Master (Unit 1):set interface gigabit-ethernet ge-1/0/3 family ethernet-switching port-mode accessset interface gigabit-ethernet ge-1/0/3 family ethernet-switching vlan members 100set interface gigabit-ethernet ge-1/0/3 description "Access-VLAN100"set interface gigabit-ethernet ge-1/0/4 family ethernet-switching port-mode accessset interface gigabit-ethernet ge-1/0/4 family ethernet-switching vlan members 200set interface gigabit-ethernet ge-1/0/4 description "Access-VLAN200"set interface gigabit-ethernet ge-1/0/5 family ethernet-switching port-mode trunkset interface gigabit-ethernet ge-1/0/5 family ethernet-switching vlan members 100set interface gigabit-ethernet ge-1/0/5 family ethernet-switching vlan members 200set interface gigabit-ethernet ge-1/0/5 description "Trunk-VLAN100-200"commit |
| Test Procedure | 1. Configure ge-1/0/3 as access port in VLAN 100, ge-1/0/4 as access port in VLAN 200, and ge-1/0/5 as trunk port carrying VLANs 100 and 200 on the master while Unit 3 is offline2. Execute run show running-config interface ge-1/0/3 on the master3. Execute run show running-config interface ge-1/0/4 on the master4. Execute run show running-config interface ge-1/0/5 on the master5. Connect Unit 3 to the stack via stack cable and power on6. Wait 120 seconds for Unit 3 to boot and join the stack7. Execute run show stack on the master8. Execute run show interface ge-1/0/3 on the master9. Execute run show interface ge-1/0/4 on the master10. Execute run show interface ge-1/0/5 on the master11. Execute run show vlans on the master |
| Expected Results | 1. Steps 2–4: Running-config shows correct configuration for each port (access VLAN 100, access VLAN 200, trunk VLANs 100+200)2. Step 7: Unit 3 appears in “Ready” state with role “Member”3. Step 8: ge-1/0/3 shows access mode, VLAN 100, description “Access-VLAN100”, admin “up”4. Step 9: ge-1/0/4 shows access mode, VLAN 200, description “Access-VLAN200”, admin “up”5. Step 10: ge-1/0/5 shows trunk mode, VLANs 100 and 200, description “Trunk-VLAN100-200”, admin “up”6. Step 11: VLAN 100 member list includes ge-1/0/3 and ge-1/0/5; VLAN 200 member list includes ge-1/0/4 and ge-1/0/5 |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-1 |

#### 1.1.2 Module Feature Interaction

##### 1.1.2.1 Pre-configure ACL on offline member port

| Field | Content |
| --- | --- |
| Test Name | PRECONFIG-06: ACL pre-config on offline member port |
| Purpose Of The Test | Verify that an Access Control List (ACL) applied to an offline member port takes effect when the member joins the stack |
| Test Topo & Precondition | Topology:Master (Unit 1) ---- [Stack Link] ---- Standby (Unit 2)Member (Unit 3) — disconnected from stackHost-A (10.10.10.2/24) ---- (ge-1/0/3 on Unit 3)Server (10.10.10.100/24) ---- (ge-1/0/1 on Unit 1)Preconditions:1. PicOS 4.6 running on all switches2. Unit 1 is master, Unit 2 is standby, both operational3. Unit 3 is powered off and disconnected from the stack4. VLAN 10 is created; ge-1/0/3 and ge-1/0/1 are in VLAN 105. Host-A and Server are in the same subnet 10.10.10.0/24Configuration:Master (Unit 1):set policy access-list BLOCK_TELNET type ipset policy access-list BLOCK_TELNET rule 10 action denyset policy access-list BLOCK_TELNET rule 10 match protocol tcpset policy access-list BLOCK_TELNET rule 10 match destination-port 23set policy access-list BLOCK_TELNET rule 20 action permitset policy access-list BLOCK_TELNET rule 20 match protocol anyset interface gigabit-ethernet ge-1/0/3 access-list in BLOCK_TELNETcommit |
| Test Procedure | 1. Configure ACL “BLOCK_TELNET” denying TCP port 23 and permitting all other traffic on the master2. Apply ACL to ge-1/0/3 (Unit 3) in the inbound direction while Unit 3 is offline3. Execute run show running-config policy access-list BLOCK_TELNET on the master4. Execute run show running-config interface ge-1/0/3 on the master5. Connect Unit 3 to the stack via stack cable and power on6. Wait 120 seconds for Unit 3 to boot and join the stack7. Execute run show stack on the master8. Execute run show access-list interface ge-1/0/3 on the master9. Initiate ping from Host-A (10.10.10.2) to Server (10.10.10.100)10. Initiate telnet connection from Host-A to Server (10.10.10.100 port 23) |
| Expected Results | 1. Step 3: ACL “BLOCK_TELNET” displayed with rule 10 deny TCP/23 and rule 20 permit any2. Step 4: ge-1/0/3 running-config shows ACL “BLOCK_TELNET” applied inbound3. Step 7: Unit 3 appears in “Ready” state with role “Member”4. Step 8: show access-list interface ge-1/0/3 displays ACL “BLOCK_TELNET” active in inbound direction5. Step 9: Ping from Host-A to Server succeeds with 0% packet loss6. Step 10: Telnet connection from Host-A to Server is refused/dropped (TCP RST or timeout) |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P1 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-1 |

##### 1.1.2.2 Pre-configure QoS policy on offline member port

| Field | Content |
| --- | --- |
| Test Name | PRECONFIG-07: QoS policy pre-config on offline member port |
| Purpose Of The Test | Verify that a QoS policy configured on an offline member port is applied and enforces traffic shaping when the member joins the stack |
| Test Topo & Precondition | Topology:Master (Unit 1) ---- [Stack Link] ---- Standby (Unit 2)Member (Unit 3) — disconnected from stackTraffic Generator ---- (ge-1/0/3 on Unit 3)Traffic Receiver ---- (ge-1/0/1 on Unit 1)Preconditions:1. PicOS 4.6 running on all switches2. Unit 1 is master, Unit 2 is standby, both operational3. Unit 3 is powered off and disconnected from the stack4. Traffic generator and receiver are connected and configured5. VLAN 10 is created; ge-1/0/3 and ge-1/0/1 are in VLAN 10Configuration:Master (Unit 1):set qos policy-map RATE_LIMIT_50M type ingressset qos policy-map RATE_LIMIT_50M class default police rate 50000 burst 1000set interface gigabit-ethernet ge-1/0/3 qos policy-map RATE_LIMIT_50Mcommit |
| Test Procedure | 1. Configure QoS policy-map “RATE_LIMIT_50M” with 50 Mbps rate limit on the master2. Apply policy-map to ge-1/0/3 (Unit 3) while Unit 3 is offline3. Execute run show running-config qos policy-map RATE_LIMIT_50M on the master4. Execute run show running-config interface ge-1/0/3 on the master5. Connect Unit 3 to the stack via stack cable and power on6. Wait 120 seconds for Unit 3 to boot and join the stack7. Execute run show stack on the master8. Execute run show qos interface ge-1/0/3 on the master9. Start traffic generator sending 100 Mbps continuous stream through ge-1/0/310. Wait 30 seconds for traffic to stabilize11. Execute run show interface ge-1/0/3 counters on the master |
| Expected Results | 1. Step 3: Policy-map “RATE_LIMIT_50M” displayed with 50 Mbps rate and 1000 Kbps burst2. Step 4: ge-1/0/3 running-config shows policy-map “RATE_LIMIT_50M” applied3. Step 7: Unit 3 appears in “Ready” state4. Step 8: show qos interface ge-1/0/3 displays active policy-map “RATE_LIMIT_50M” with configured rate5. Step 11: Interface counters show ingress throughput rate-limited to approximately 50 Mbps (±5%), with excess traffic counted as dropped |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P1 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-1 |

##### 1.1.2.3 Pre-configure port-channel/LAG with offline member ports

| Field | Content |
| --- | --- |
| Test Name | PRECONFIG-08: LAG pre-config with offline member ports |
| Purpose Of The Test | Verify that a port-channel (LAG) configured with ports on an offline member becomes operational when the member joins the stack |
| Test Topo & Precondition | Topology:Master (Unit 1) ---- [Stack Link] ---- Standby (Unit 2)Member (Unit 3) — disconnected from stackRemote Switch ---- (ae1: ge-1/0/3, ge-1/0/4 on Unit 3)Preconditions:1. PicOS 4.6 running on all switches2. Unit 1 is master, Unit 2 is standby, both operational3. Unit 3 is powered off and disconnected from the stack4. Remote Switch has LACP port-channel configured with two member ports connected to Unit 3’s ge-1/0/3 and ge-1/0/4Configuration:Master (Unit 1):set interface aggregate-ethernet ae1 aggregated-ether-options lacp activeset interface gigabit-ethernet ge-1/0/3 ether-options aggregate-ethernet ae1set interface gigabit-ethernet ge-1/0/4 ether-options aggregate-ethernet ae1set interface aggregate-ethernet ae1 family ethernet-switching port-mode trunkset interface aggregate-ethernet ae1 family ethernet-switching vlan members 100set interface aggregate-ethernet ae1 family ethernet-switching vlan members 200commit |
| Test Procedure | 1. Configure aggregate interface ae1 with LACP active mode and members ge-1/0/3, ge-1/0/4 (Unit 3) on the master while Unit 3 is offline2. Execute run show running-config interface ae1 on the master3. Execute run show running-config interface ge-1/0/3 on the master4. Execute run show running-config interface ge-1/0/4 on the master5. Connect Unit 3 to the stack via stack cable and power on6. Wait 120 seconds for Unit 3 to boot and join the stack7. Execute run show stack on the master8. Wait 30 seconds for LACP negotiation9. Execute run show lacp interface on the master10. Execute run show interface ae1 on the master |
| Expected Results | 1. Steps 2–4: Running-config shows ae1 with LACP active, ge-1/0/3 and ge-1/0/4 as members, trunk mode with VLANs 100 and 2002. Step 7: Unit 3 appears in “Ready” state3. Step 9: show lacp interface displays ae1 with ge-1/0/3 and ge-1/0/4 both in “Collecting/Distributing” state4. Step 10: ae1 shows link “up”, operational members 2, trunk mode, VLANs 100 and 200 |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P1 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-1 |

##### 1.1.2.4 Pre-configure spanning-tree settings on offline member port

| Field | Content |
| --- | --- |
| Test Name | PRECONFIG-09: STP pre-config on offline member port |
| Purpose Of The Test | Verify that spanning-tree parameters (edge port, BPDU guard) configured on an offline member port take effect when the member joins the stack |
| Test Topo & Precondition | Topology:Master (Unit 1) ---- [Stack Link] ---- Standby (Unit 2)Member (Unit 3) — disconnected from stackHost-A ---- (ge-1/0/3 on Unit 3, STP edge port)Remote Switch ---- (ge-1/0/4 on Unit 3, non-edge STP port)Preconditions:1. PicOS 4.6 running on all switches2. Unit 1 is master, Unit 2 is standby, both operational3. Unit 3 is powered off and disconnected from the stack4. Spanning-tree (RSTP) is enabled globally on the stack5. VLAN 10 is created; ge-1/0/3 and ge-1/0/4 are in VLAN 10Configuration:Master (Unit 1):set protocols rstpset interface gigabit-ethernet ge-1/0/3 family ethernet-switching port-mode accessset interface gigabit-ethernet ge-1/0/3 family ethernet-switching vlan members 10set protocols rstp interface ge-1/0/3 edge-port trueset protocols rstp interface ge-1/0/3 bpdu-guard trueset interface gigabit-ethernet ge-1/0/4 family ethernet-switching port-mode trunkset interface gigabit-ethernet ge-1/0/4 family ethernet-switching vlan members 10set protocols rstp interface ge-1/0/4 cost 20000commit |
| Test Procedure | 1. Configure STP edge-port and BPDU guard on ge-1/0/3, and STP cost 20000 on ge-1/0/4 (both Unit 3) from the master while Unit 3 is offline2. Execute run show running-config protocols rstp interface ge-1/0/3 on the master3. Execute run show running-config protocols rstp interface ge-1/0/4 on the master4. Connect Unit 3 to the stack via stack cable and power on5. Wait 120 seconds for Unit 3 to boot and join the stack6. Execute run show stack on the master7. Execute run show spanning-tree interface ge-1/0/3 on the master8. Execute run show spanning-tree interface ge-1/0/4 on the master |
| Expected Results | 1. Step 2: Running-config shows ge-1/0/3 with edge-port true and bpdu-guard true2. Step 3: Running-config shows ge-1/0/4 with cost 200003. Step 6: Unit 3 appears in “Ready” state4. Step 7: ge-1/0/3 displays STP state “Forwarding” immediately (edge port behavior), with BPDU guard “enabled”5. Step 8: ge-1/0/4 displays STP path cost 20000, and port transitions through Discarding → Learning → Forwarding |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P1 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-1 |

##### 1.1.2.5 Pre-configure OSPF interface on offline member port

| Field | Content |
| --- | --- |
| Test Name | PRECONFIG-10: OSPF interface pre-config on offline member port |
| Purpose Of The Test | Verify that OSPF interface parameters configured on an offline member port enable OSPF adjacency formation when the member joins the stack |
| Test Topo & Precondition | Topology:Master (Unit 1) ---- [Stack Link] ---- Standby (Unit 2)Member (Unit 3) — disconnected from stackOSPF Neighbor Router (192.168.1.2/24, Area 0) ---- (ge-1/0/3 on Unit 3, 192.168.1.1/24)Preconditions:1. PicOS 4.6 running on all switches2. Unit 1 is master, Unit 2 is standby, both operational3. Unit 3 is powered off and disconnected from the stack4. OSPF neighbor router is configured with area 0, hello-interval 10, router-id 2.2.2.25. Stack router-id is 1.1.1.1Configuration:Master (Unit 1):set interface gigabit-ethernet ge-1/0/3 family inet address 192.168.1.1/24set interface gigabit-ethernet ge-1/0/3 routing-interface enableset protocols ospf router-id 1.1.1.1set protocols ospf area 0.0.0.0 interface ge-1/0/3set protocols ospf area 0.0.0.0 interface ge-1/0/3 hello-interval 10set protocols ospf area 0.0.0.0 interface ge-1/0/3 dead-interval 40commit |
| Test Procedure | 1. Configure IP address and OSPF area 0 on ge-1/0/3 (Unit 3) from the master while Unit 3 is offline2. Execute run show running-config protocols ospf on the master3. Execute run show running-config interface ge-1/0/3 on the master4. Connect Unit 3 to the stack via stack cable and power on5. Wait 120 seconds for Unit 3 to boot and join the stack6. Execute run show stack on the master7. Wait 60 seconds for OSPF adjacency formation8. Execute run show ip ospf neighbor on the master9. Execute run show ip ospf interface ge-1/0/3 on the master10. Execute run show ip route ospf on the master |
| Expected Results | 1. Step 2: Running-config shows OSPF area 0.0.0.0 with interface ge-1/0/3, hello-interval 10, dead-interval 402. Step 3: Running-config shows IP address 192.168.1.1/24 and routing-interface enabled3. Step 6: Unit 3 appears in “Ready” state4. Step 8: show ip ospf neighbor displays neighbor 2.2.2.2 in “Full” state on ge-1/0/35. Step 9: ge-1/0/3 shows OSPF interface state “DR” or “BDR”, area 0.0.0.0, hello 10, dead 406. Step 10: OSPF routes learned from neighbor appear in the routing table |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P1 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-1 |

#### 1.1.3 Pre-Config Persistence

##### 1.1.3.1 Pre-config survives master/standby switchover

| Field | Content |
| --- | --- |
| Test Name | PRECONFIG-11: Pre-config persists after master/standby switchover |
| Purpose Of The Test | Verify that port pre-configuration for an offline member is retained in running-config after a master/standby switchover occurs |
| Test Topo & Precondition | Topology:Master (Unit 1) ---- [Stack Link] ---- Standby (Unit 2)Member (Unit 3) — disconnected from stackHost-A ---- (ge-1/0/3 on Unit 3)Preconditions:1. PicOS 4.6 running on all switches2. Unit 1 is master, Unit 2 is standby, both operational3. Unit 3 is powered off and disconnected from the stack4. Pre-configuration applied on ge-1/0/3: description, VLAN 100 access, speed 1000Configuration:Master (Unit 1) — pre-existing:set interface gigabit-ethernet ge-1/0/3 description "Persist-Test"set interface gigabit-ethernet ge-1/0/3 family ethernet-switching port-mode accessset interface gigabit-ethernet ge-1/0/3 family ethernet-switching vlan members 100set interface gigabit-ethernet ge-1/0/3 speed 1000commit |
| Test Procedure | 1. Execute run show running-config interface ge-1/0/3 on the master (Unit 1)2. Execute run show stack on the master (Unit 1)3. Execute request stack switchover on the master (Unit 1) to trigger failover4. Wait 120 seconds for switchover to complete5. Execute run show stack on the new master (Unit 2)6. Execute run show running-config interface ge-1/0/3 on the new master (Unit 2)7. Connect Unit 3 to the stack via stack cable and power on8. Wait 120 seconds for Unit 3 to boot and join the stack9. Execute run show stack on the master10. Execute run show interface ge-1/0/3 on the master |
| Expected Results | 1. Step 1: Pre-configuration for ge-1/0/3 visible (description, VLAN 100, speed 1000)2. Step 5: Unit 2 is now master, Unit 1 is standby; Unit 3 still absent3. Step 6: Running-config on new master still contains ge-1/0/3 pre-config with description “Persist-Test”, access VLAN 100, speed 1000 — identical to Step 14. Step 9: Unit 3 appears in “Ready” state5. Step 10: ge-1/0/3 shows description “Persist-Test”, access VLAN 100, speed 1000, admin “up” |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-1 |

##### 1.1.3.2 Pre-config retained when member leaves stack

| Field | Content |
| --- | --- |
| Test Name | PRECONFIG-12: Pre-config retained after member leaves |
| Purpose Of The Test | Verify that pre-configured port settings remain in the running-config when a previously online member leaves the stack, and re-apply when the member rejoins |
| Test Topo & Precondition | Topology:Master (Unit 1) ---- [Stack Link] ---- Standby (Unit 2)Member (Unit 3) ---- [Stack Link] ---- Master (Unit 1)Host-A (VLAN 100) ---- (ge-1/0/3 on Unit 3)Preconditions:1. PicOS 4.6 running on all switches2. Unit 1 is master, Unit 2 is standby, Unit 3 is member — all operational3. ge-1/0/3 on Unit 3 is configured with access VLAN 100, description “Member3-Port”4. Host-A traffic is passing through ge-1/0/3Configuration:Master (Unit 1) — pre-existing applied config:set interface gigabit-ethernet ge-1/0/3 description "Member3-Port"set interface gigabit-ethernet ge-1/0/3 family ethernet-switching port-mode accessset interface gigabit-ethernet ge-1/0/3 family ethernet-switching vlan members 100commit |
| Test Procedure | 1. Execute run show stack on the master to confirm all 3 units operational2. Execute run show interface ge-1/0/3 on the master to confirm config applied3. Disconnect Unit 3 from the stack by removing the stack cable4. Wait 60 seconds for the master to detect member loss5. Execute run show stack on the master6. Execute run show running-config interface ge-1/0/3 on the master7. Reconnect Unit 3 to the stack via stack cable8. Wait 120 seconds for Unit 3 to rejoin9. Execute run show stack on the master10. Execute run show interface ge-1/0/3 on the master |
| Expected Results | 1. Step 1: All three units visible with Unit 1 master, Unit 2 standby, Unit 3 member2. Step 2: ge-1/0/3 shows access VLAN 100, description “Member3-Port”, link “up”3. Step 5: Unit 3 status changes to “Absent” or is removed from stack display4. Step 6: Running-config still contains ge-1/0/3 configuration (description, VLAN 100) despite Unit 3 being absent5. Step 9: Unit 3 reappears in “Ready” state6. Step 10: ge-1/0/3 shows access VLAN 100, description “Member3-Port”, admin “up”, link “up” |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-1 |

##### 1.1.3.3 Pre-config on member with hardware port count mismatch

| Field | Content |
| --- | --- |
| Test Name | PRECONFIG-13: Pre-config with hardware port count mismatch |
| Purpose Of The Test | Verify system behavior when pre-configured ports reference port indices that do not exist on the joining member hardware (e.g., config references ge-1/0/48 but hardware only has 24 ports) |
| Test Topo & Precondition | Topology:Master (Unit 1, 48-port model) ---- [Stack Link] ---- Standby (Unit 2, 48-port model)Member (Unit 3, 24-port model) — disconnected from stackPreconditions:1. PicOS 4.6 running on all switches2. Unit 1 (48-port) is master, Unit 2 (48-port) is standby, both operational3. Unit 3 is a 24-port model, powered off and disconnected from the stack4. Pre-configuration applied on ge-1/0/3 (valid port) and ge-1/0/48 (non-existent on 24-port model)Configuration:Master (Unit 1):set interface gigabit-ethernet ge-1/0/3 description "Valid-Port"set interface gigabit-ethernet ge-1/0/3 family ethernet-switching vlan members 100set interface gigabit-ethernet ge-1/0/48 description "Nonexistent-Port"set interface gigabit-ethernet ge-1/0/48 family ethernet-switching vlan members 200commit |
| Test Procedure | 1. Configure ge-1/0/3 and ge-1/0/48 for Unit 3 from the master while Unit 3 (24-port) is offline2. Execute run show running-config interface ge-1/0/3 on the master3. Execute run show running-config interface ge-1/0/48 on the master4. Connect Unit 3 (24-port model) to the stack via stack cable and power on5. Wait 120 seconds for Unit 3 to boot and join the stack6. Execute run show stack on the master7. Execute run show interface ge-1/0/3 on the master8. Execute run show interface ge-1/0/48 on the master9. Execute run show logging ¦ grep ge-1/0/48 on the master |
| Expected Results | 1. Steps 2–3: Both pre-configurations present in running-config2. Step 6: Unit 3 appears in “Ready” state (stack join not blocked by mismatched config)3. Step 7: ge-1/0/3 shows description “Valid-Port”, VLAN 100, admin “up”, link “up” — config applied successfully4. Step 8: ge-1/0/48 output indicates port does not exist or shows “not present” status5. Step 9: System log contains a warning message indicating port ge-1/0/48 configuration could not be applied due to hardware mismatch |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P2 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-1. Requires mixed-hardware stack (48-port and 24-port models). |

##### 1.1.3.4 Pre-config applied correctly after stack reboot

| Field | Content |
| --- | --- |
| Test Name | PRECONFIG-14: Pre-config applied after full stack reboot |
| Purpose Of The Test | Verify that pre-configured port settings persist through a full stack reboot (all units power-cycled simultaneously) and are correctly applied to all members upon boot |
| Test Topo & Precondition | Topology:Master (Unit 1) ---- [Stack Link] ---- Standby (Unit 2) ---- [Stack Link] ---- Member (Unit 3)Host-A (VLAN 100) ---- (ge-1/0/3 on Unit 3)Host-B (VLAN 200) ---- (ge-1/0/2 on Unit 2)Preconditions:1. PicOS 4.6 running on all switches2. Unit 1 is master, Unit 2 is standby, Unit 3 is member — all operational3. ge-1/0/3 (Unit 3) is configured with access VLAN 100, description “Unit3-Access”4. ge-1/0/2 (Unit 2) is configured with access VLAN 200, description “Unit2-Access”5. Startup-config has been savedConfiguration:Master (Unit 1) — pre-existing saved config:set interface gigabit-ethernet ge-1/0/3 description "Unit3-Access"set interface gigabit-ethernet ge-1/0/3 family ethernet-switching port-mode accessset interface gigabit-ethernet ge-1/0/3 family ethernet-switching vlan members 100set interface gigabit-ethernet ge-1/0/2 description "Unit2-Access"set interface gigabit-ethernet ge-1/0/2 family ethernet-switching port-mode accessset interface gigabit-ethernet ge-1/0/2 family ethernet-switching vlan members 200commit |
| Test Procedure | 1. Execute run show running-config interface ge-1/0/3 on the master2. Execute run show running-config interface ge-1/0/2 on the master3. Execute run copy running-config startup-config on the master4. Execute request system reboot on the master to reboot the entire stack5. Wait 300 seconds for all stack members to complete reboot6. Execute run show stack on the master7. Execute run show running-config interface ge-1/0/3 on the master8. Execute run show running-config interface ge-1/0/2 on the master9. Execute run show interface ge-1/0/3 on the master10. Execute run show interface ge-1/0/2 on the master |
| Expected Results | 1. Steps 1–2: Pre-existing configuration displayed correctly for both ports2. Step 6: All three units appear in stack — Unit 1 master, Unit 2 standby, Unit 3 member, all “Ready”3. Step 7: ge-1/0/3 running-config matches pre-reboot config (description “Unit3-Access”, access VLAN 100)4. Step 8: ge-1/0/2 running-config matches pre-reboot config (description “Unit2-Access”, access VLAN 200)5. Step 9: ge-1/0/3 shows admin “up”, link “up”, access VLAN 100, description “Unit3-Access”6. Step 10: ge-1/0/2 shows admin “up”, link “up”, access VLAN 200, description “Unit2-Access” |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-1 |

##### 1.1.3.5 Pre-config consistency — running-config matches across stack members

| Field | Content |
| --- | --- |
| Test Name | PRECONFIG-15: Running-config consistency across stack members |
| Purpose Of The Test | Verify that the running-config for pre-configured ports is synchronized and consistent when viewed from any stack member (master, standby, or member) |
| Test Topo & Precondition | Topology:Master (Unit 1) ---- [Stack Link] ---- Standby (Unit 2) ---- [Stack Link] ---- Member (Unit 3)Host-A ---- (ge-1/0/3 on Unit 3)Host-B ---- (ge-1/0/2 on Unit 2)Host-C ---- (ge-1/0/1 on Unit 1)Preconditions:1. PicOS 4.6 running on all switches2. Unit 1 is master, Unit 2 is standby, Unit 3 is member — all operational3. ge-1/0/1 (Unit 1) configured with access VLAN 10, description “Unit1-Port”4. ge-1/0/2 (Unit 2) configured with access VLAN 20, description “Unit2-Port”5. ge-1/0/3 (Unit 3) configured with access VLAN 30, description “Unit3-Port”Configuration:Master (Unit 1):set interface gigabit-ethernet ge-1/0/1 description "Unit1-Port"set interface gigabit-ethernet ge-1/0/1 family ethernet-switching vlan members 10set interface gigabit-ethernet ge-1/0/2 description "Unit2-Port"set interface gigabit-ethernet ge-1/0/2 family ethernet-switching vlan members 20set interface gigabit-ethernet ge-1/0/3 description "Unit3-Port"set interface gigabit-ethernet ge-1/0/3 family ethernet-switching vlan members 30commit |
| Test Procedure | 1. Execute run show running-config interface ge-1/0/1 on the master (Unit 1)2. Execute run show running-config interface ge-1/0/2 on the master (Unit 1)3. Execute run show running-config interface ge-1/0/3 on the master (Unit 1)4. Execute run show running-config interface ge-1/0/1 on Unit 2 via session redirect5. Execute run show running-config interface ge-1/0/2 on Unit 2 via session redirect6. Execute run show running-config interface ge-1/0/3 on Unit 2 via session redirect7. Execute run show running-config interface ge-1/0/1 on Unit 3 via session redirect8. Execute run show running-config interface ge-1/0/2 on Unit 3 via session redirect9. Execute run show running-config interface ge-1/0/3 on Unit 3 via session redirect10. Execute run show interface ge-1/0/1 on the master11. Execute run show interface ge-1/0/2 on the master12. Execute run show interface ge-1/0/3 on the master |
| Expected Results | 1. Steps 1–3: Master shows correct running-config for all three interfaces2. Steps 4–6: Unit 2 (standby) shows identical running-config output for all three interfaces as master3. Steps 7–9: Unit 3 (member) shows identical running-config output for all three interfaces as master4. Step 10: ge-1/0/1 shows description “Unit1-Port”, VLAN 10, admin “up”5. Step 11: ge-1/0/2 shows description “Unit2-Port”, VLAN 20, admin “up”6. Step 12: ge-1/0/3 shows description “Unit3-Port”, VLAN 30, admin “up” |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P1 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-1 |

### 1.2 Inband Management (TP-2)

#### 1.2.1 Basic Inband Connectivity

##### 1.2.1.1 Inband management interface reachable via SSH

| Field | Content |
| --- | --- |
| Test Name | INBAND-01: SSH access via inband management interface |
| Purpose Of The Test | Verify that the stack inband management interface is reachable via SSH from a remote management station |
| Test Topo & Precondition | Topology:Master (Unit 1) ---- [Stack Link] ---- Standby (Unit 2) ---- [Stack Link] ---- Member (Unit 3)Management Station ---- [L3 Network] ---- (VLAN 1 SVI: 10.0.0.1/24 on Stack)Preconditions:1. PicOS 4.6 running on all switches2. Unit 1 is master, Unit 2 is standby, Unit 3 is member — all operational3. Inband management VLAN interface (VLAN 1 SVI) configured with IP 10.0.0.1/244. SSH service enabled on the stack5. Management Station has IP 10.0.0.100/24 with route to 10.0.0.16. User account “admin” exists with password configuredConfiguration:Master (Unit 1):set interface vlan vlan1 family inet address 10.0.0.1/24set system services ssh enable trueset system services ssh port 22commit |
| Test Procedure | 1. Execute run show interface vlan vlan1 on the master via console2. Execute run show system services ssh on the master via console3. Initiate SSH connection from Management Station: ssh admin@10.0.0.14. Wait 10 seconds for SSH session establishment5. Execute run show stack through the SSH session6. Execute run show interface vlan vlan1 through the SSH session7. Disconnect the SSH session: exit |
| Expected Results | 1. Step 1: VLAN 1 SVI shows IP 10.0.0.1/24, admin “up”, link “up”2. Step 2: SSH service shows “enabled”, port 223. Step 3: SSH connection established successfully with authentication prompt4. Step 5: show stack output displays all three units with correct roles — accessible from remote SSH session5. Step 6: VLAN 1 SVI status displayed correctly6. Step 7: SSH session closes cleanly |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-2 |

##### 1.2.1.2 Inband management interface reachable via SNMP

| Field | Content |
| --- | --- |
| Test Name | INBAND-02: SNMP access via inband management interface |
| Purpose Of The Test | Verify that the stack inband management interface responds to SNMP queries (v2c and v3) from a remote NMS station |
| Test Topo & Precondition | Topology:Master (Unit 1) ---- [Stack Link] ---- Standby (Unit 2) ---- [Stack Link] ---- Member (Unit 3)NMS Station ---- [L3 Network] ---- (VLAN 1 SVI: 10.0.0.1/24 on Stack)Preconditions:1. PicOS 4.6 running on all switches2. Unit 1 is master, Unit 2 is standby, Unit 3 is member — all operational3. Inband management VLAN interface (VLAN 1 SVI) configured with IP 10.0.0.1/244. SNMP v2c enabled with community string “public”5. NMS Station has IP 10.0.0.101/24 with snmpwalk/snmpget tools installedConfiguration:Master (Unit 1):set interface vlan vlan1 family inet address 10.0.0.1/24set system services snmp community public authorization read-onlyset system services snmp enable truecommit |
| Test Procedure | 1. Execute run show system services snmp on the master via console2. Execute SNMP GET from NMS Station: snmpget -v2c -c public 10.0.0.1 sysDescr.03. Wait 5 seconds for response4. Execute SNMP WALK from NMS Station: snmpwalk -v2c -c public 10.0.0.1 ifTable5. Wait 15 seconds for walk completion6. Execute SNMP GET for sysUpTime from NMS Station: snmpget -v2c -c public 10.0.0.1 sysUpTimeInstance |
| Expected Results | 1. Step 1: SNMP service shows “enabled”, community “public” with read-only access2. Step 2: SNMP GET returns sysDescr containing “PicOS” and version information3. Step 4: SNMP WALK returns ifTable entries for all stack interfaces including ge-1/0/x ports across all three units4. Step 6: sysUpTimeInstance returns a valid timetick value representing system uptime |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-2 |

##### 1.2.1.3 Inband management interface handles Syslog output

| Field | Content |
| --- | --- |
| Test Name | INBAND-03: Syslog output via inband management interface |
| Purpose Of The Test | Verify that the stack sends syslog messages to a remote syslog server through the inband management interface |
| Test Topo & Precondition | Topology:Master (Unit 1) ---- [Stack Link] ---- Standby (Unit 2) ---- [Stack Link] ---- Member (Unit 3)Syslog Server (10.0.0.200/24) ---- [L3 Network] ---- (VLAN 1 SVI: 10.0.0.1/24 on Stack)Preconditions:1. PicOS 4.6 running on all switches2. Unit 1 is master, Unit 2 is standby, Unit 3 is member — all operational3. Inband management VLAN interface (VLAN 1 SVI) configured with IP 10.0.0.1/244. Remote syslog server at 10.0.0.200 listening on UDP port 5145. Syslog server is capturing incoming messagesConfiguration:Master (Unit 1):set interface vlan vlan1 family inet address 10.0.0.1/24set system syslog host 10.0.0.200 facility any severity infoset system syslog host 10.0.0.200 transport udp port 514commit |
| Test Procedure | 1. Execute run show system syslog on the master via console2. Start packet capture on Syslog Server filtering UDP port 514 from 10.0.0.13. Shutdown interface ge-1/0/1 on the master: set interface gigabit-ethernet ge-1/0/1 enable false then commit4. Wait 10 seconds for syslog message generation5. Re-enable interface ge-1/0/1: set interface gigabit-ethernet ge-1/0/1 enable true then commit6. Wait 10 seconds for syslog message generation7. Stop packet capture on Syslog Server8. Execute run show logging ¦ tail 20 on the master |
| Expected Results | 1. Step 1: Syslog configuration shows host 10.0.0.200, UDP 514, facility any, severity info2. Step 7: Packet capture on Syslog Server shows UDP packets from 10.0.0.1 to port 514 containing interface down and interface up messages for ge-1/0/13. Step 8: Local log shows interface ge-1/0/1 state change events (down and up) with timestamps |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P1 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-2 |

##### 1.2.1.4 Inband management traffic on non-default VLAN

| Field | Content |
| --- | --- |
| Test Name | INBAND-04: Inband management on non-default VLAN |
| Purpose Of The Test | Verify that inband management works correctly when configured on a non-default VLAN (e.g., VLAN 999) instead of the default VLAN 1 |
| Test Topo & Precondition | Topology:Master (Unit 1) ---- [Stack Link] ---- Standby (Unit 2) ---- [Stack Link] ---- Member (Unit 3)Management Station (10.99.0.100/24) ---- [Trunk Port ge-1/0/48 on Unit 1, VLAN 999] ---- StackPreconditions:1. PicOS 4.6 running on all switches2. Unit 1 is master, Unit 2 is standby, Unit 3 is member — all operational3. VLAN 999 created as management VLAN4. Management Station connected to ge-1/0/48 on Unit 1 with tagged VLAN 9995. Management Station has IP 10.99.0.100/24Configuration:Master (Unit 1):set vlans vlan-id 999 description "Management-VLAN"set interface vlan vlan999 family inet address 10.99.0.1/24set interface gigabit-ethernet ge-1/0/48 family ethernet-switching port-mode trunkset interface gigabit-ethernet ge-1/0/48 family ethernet-switching vlan members 999set system services ssh enable truecommit |
| Test Procedure | 1. Execute run show vlans vlan-id 999 on the master via console2. Execute run show interface vlan vlan999 on the master via console3. Initiate ping from Management Station (10.99.0.100) to 10.99.0.14. Wait 10 seconds for ping results5. Initiate SSH connection from Management Station: ssh admin@10.99.0.16. Wait 10 seconds for SSH session establishment7. Execute run show stack through the SSH session8. Execute run show vlans vlan-id 999 through the SSH session9. Disconnect the SSH session: exit |
| Expected Results | 1. Step 1: VLAN 999 exists with description “Management-VLAN” and ge-1/0/48 as member2. Step 2: VLAN 999 SVI shows IP 10.99.0.1/24, admin “up”, link “up”3. Step 3: Ping from Management Station to 10.99.0.1 succeeds with 0% packet loss4. Step 5: SSH connection established successfully5. Step 7: show stack output displays all three units — management accessible via non-default VLAN6. Step 8: VLAN 999 information displayed correctly through SSH session |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P1 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-2 |

#### 1.2.2 Inband During Stack Events

##### 1.2.2.1 Inband continuity during master/standby switchover

| Field | Content |
| --- | --- |
| Test Name | INBAND-05: Inband continuity during master/standby switchover |
| Purpose Of The Test | Verify inband management connectivity during a master/standby switchover, measuring management downtime |
| Test Topo & Precondition | Topology:Master (Unit 1) ---- [Stack Link] ---- Standby (Unit 2) ---- [Stack Link] ---- Member (Unit 3)Management Station (10.0.0.100/24) ---- [L3 Network] ---- (VLAN 1 SVI: 10.0.0.1/24 on Stack)Preconditions:1. PicOS 4.6 running on all switches2. Unit 1 is master, Unit 2 is standby, Unit 3 is member — all operational3. Inband management VLAN interface configured with IP 10.0.0.1/244. SSH established from Management Station to 10.0.0.1 and verified working5. Management Station has continuous ping running to 10.0.0.1Configuration:Master (Unit 1):set interface vlan vlan1 family inet address 10.0.0.1/24set system services ssh enable truecommit |
| Test Procedure | 1. Start continuous ping from Management Station to 10.0.0.1 with timestamps: ping -D -i 1 10.0.0.12. Wait 10 seconds to establish baseline ping response times3. Execute request stack switchover on the master (Unit 1) via console4. Wait 180 seconds for switchover to complete5. Stop continuous ping on Management Station and record results6. Execute run show stack on the new master (Unit 2)7. Initiate new SSH connection from Management Station: ssh admin@10.0.0.18. Execute run show stack through the SSH session |
| Expected Results | 1. Step 2: Baseline ping shows consistent response times (e.g., < 5 ms)2. Step 5: Continuous ping log shows packet loss period — management downtime duration recorded. Downtime should not exceed 120 seconds3. Step 6: Unit 2 is now master, Unit 1 is standby; all units in “Ready” state4. Step 7: SSH connection to 10.0.0.1 established successfully after switchover5. Step 8: show stack accessible via SSH, displaying updated roles |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-2. Record exact management downtime in seconds for KPI tracking. |

##### 1.2.2.2 Inband recovery after stack member joins

| Field | Content |
| --- | --- |
| Test Name | INBAND-06: Inband management recovery after member join |
| Purpose Of The Test | Verify that inband management remains stable and accessible when a new stack member joins the stack |
| Test Topo & Precondition | Topology:Master (Unit 1) ---- [Stack Link] ---- Standby (Unit 2)Member (Unit 3) — disconnected from stackManagement Station (10.0.0.100/24) ---- [L3 Network] ---- (VLAN 1 SVI: 10.0.0.1/24 on Stack)Preconditions:1. PicOS 4.6 running on all switches2. Unit 1 is master, Unit 2 is standby — both operational3. Unit 3 is powered off and disconnected from the stack4. Inband management VLAN interface configured with IP 10.0.0.1/245. SSH session active from Management Station to 10.0.0.1Configuration:Master (Unit 1):set interface vlan vlan1 family inet address 10.0.0.1/24set system services ssh enable truecommit |
| Test Procedure | 1. Start continuous ping from Management Station to 10.0.0.1: ping -D -i 1 10.0.0.12. Establish SSH session from Management Station: ssh admin@10.0.0.13. Wait 10 seconds to confirm stable management4. Connect Unit 3 to the stack via stack cable and power on5. Wait 120 seconds for Unit 3 to boot and join the stack6. Execute run show stack through the existing SSH session7. Stop continuous ping on Management Station and record results8. Execute run show interface vlan vlan1 through the SSH session |
| Expected Results | 1. Step 3: Ping and SSH session both stable with consistent response times2. Step 6: show stack displays all three units — Unit 1 master, Unit 2 standby, Unit 3 member in “Ready” state3. Step 7: Continuous ping shows zero or near-zero packet loss during member join event4. Step 8: VLAN 1 SVI remains up with IP 10.0.0.1/24 — no disruption to management interface |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-2 |

##### 1.2.2.3 Inband recovery after stack member leaves

| Field | Content |
| --- | --- |
| Test Name | INBAND-07: Inband management recovery after member leave |
| Purpose Of The Test | Verify that inband management remains stable when a stack member is removed from the stack |
| Test Topo & Precondition | Topology:Master (Unit 1) ---- [Stack Link] ---- Standby (Unit 2) ---- [Stack Link] ---- Member (Unit 3)Management Station (10.0.0.100/24) ---- [L3 Network] ---- (VLAN 1 SVI: 10.0.0.1/24 on Stack)Preconditions:1. PicOS 4.6 running on all switches2. Unit 1 is master, Unit 2 is standby, Unit 3 is member — all operational3. Inband management VLAN interface configured with IP 10.0.0.1/244. SSH session active from Management Station to 10.0.0.15. Management uplink is connected to Unit 1 (master)Configuration:Master (Unit 1):set interface vlan vlan1 family inet address 10.0.0.1/24set system services ssh enable truecommit |
| Test Procedure | 1. Start continuous ping from Management Station to 10.0.0.1: ping -D -i 1 10.0.0.12. Establish SSH session from Management Station: ssh admin@10.0.0.13. Execute run show stack through the SSH session to confirm 3 units4. Disconnect Unit 3 from the stack by removing the stack cable5. Wait 60 seconds for the master to detect member loss6. Execute run show stack through the SSH session7. Stop continuous ping on Management Station and record results8. Execute run show logging ¦ tail 10 through the SSH session |
| Expected Results | 1. Step 3: Three units displayed in the stack2. Step 6: show stack shows Unit 3 in “Absent” state or removed from the list; Unit 1 master and Unit 2 standby remain operational3. Step 7: Continuous ping shows zero or near-zero packet loss during member leave event — management not disrupted4. Step 8: Log messages indicate Unit 3 departure from the stack with timestamp |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P0 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-2 |

##### 1.2.2.4 Inband behavior during stack link failure

| Field | Content |
| --- | --- |
| Test Name | INBAND-08: Inband management during stack link failure |
| Purpose Of The Test | Verify inband management behavior when one of the inter-switch stack links fails, measuring any management disruption |
| Test Topo & Precondition | Topology:Master (Unit 1) ---- [Stack Link A] ---- Standby (Unit 2) ---- [Stack Link B] ---- Member (Unit 3)Master (Unit 1) ---- [Stack Link C] ---- Member (Unit 3) (ring topology)Management Station (10.0.0.100/24) ---- [L3 Network] ---- (VLAN 1 SVI: 10.0.0.1/24 on Stack)Preconditions:1. PicOS 4.6 running on all switches2. Unit 1 is master, Unit 2 is standby, Unit 3 is member — all operational3. Stack is in ring topology (3 stack links forming a ring)4. Inband management VLAN interface configured with IP 10.0.0.1/245. SSH session active from Management Station to 10.0.0.1Configuration:Master (Unit 1):set interface vlan vlan1 family inet address 10.0.0.1/24set system services ssh enable truecommit |
| Test Procedure | 1. Execute run show stack topology on the master to confirm ring topology2. Start continuous ping from Management Station to 10.0.0.1: ping -D -i 1 10.0.0.13. Establish SSH session from Management Station: ssh admin@10.0.0.14. Wait 10 seconds to confirm stable baseline5. Disconnect Stack Link A (cable between Unit 1 and Unit 2)6. Wait 60 seconds for stack topology reconvergence7. Execute run show stack topology through the SSH session8. Execute run show stack through the SSH session9. Stop continuous ping on Management Station and record results10. Reconnect Stack Link A11. Wait 60 seconds for ring topology restoration12. Execute run show stack topology through the SSH session |
| Expected Results | 1. Step 1: Ring topology confirmed with 3 active stack links2. Step 7: Stack topology shows degraded to chain topology; Unit 1 reaches Unit 2 via Unit 33. Step 8: All three units still present, roles unchanged, all in “Ready” state4. Step 9: Continuous ping shows minimal packet loss (< 5 packets) during topology reconvergence5. Step 12: Ring topology restored with 3 active stack links after cable reconnection |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P1 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-2. Record exact packet loss count and reconvergence time. |

#### 1.2.3 Inband Stability

##### 1.2.3.1 Concurrent SSH sessions through inband during switchover

| Field | Content |
| --- | --- |
| Test Name | INBAND-09: Concurrent SSH sessions during switchover |
| Purpose Of The Test | Verify behavior of multiple concurrent SSH sessions through the inband management interface during a master/standby switchover |
| Test Topo & Precondition | Topology:Master (Unit 1) ---- [Stack Link] ---- Standby (Unit 2) ---- [Stack Link] ---- Member (Unit 3)Management Station A (10.0.0.100/24) ---- [L3 Network] ---- (VLAN 1 SVI: 10.0.0.1/24 on Stack)Management Station B (10.0.0.101/24) ---- [L3 Network] ---- (VLAN 1 SVI: 10.0.0.1/24 on Stack)Management Station C (10.0.0.102/24) ---- [L3 Network] ---- (VLAN 1 SVI: 10.0.0.1/24 on Stack)Preconditions:1. PicOS 4.6 running on all switches2. Unit 1 is master, Unit 2 is standby, Unit 3 is member — all operational3. Inband management VLAN interface configured with IP 10.0.0.1/244. SSH service enabled, max-sessions set to allow at least 5 concurrent sessions5. Three management stations availableConfiguration:Master (Unit 1):set interface vlan vlan1 family inet address 10.0.0.1/24set system services ssh enable trueset system services ssh max-sessions 10commit |
| Test Procedure | 1. Establish SSH session from Station A: ssh admin@10.0.0.1 (Session 1)2. Establish SSH session from Station B: ssh admin@10.0.0.1 (Session 2)3. Establish SSH session from Station C: ssh admin@10.0.0.1 (Session 3)4. Execute run show stack on each session simultaneously to confirm all three active5. Execute request stack switchover on the master (Unit 1) via console6. Wait 180 seconds for switchover to complete7. Execute run show stack on each SSH session (attempt on all three)8. Record which sessions survived the switchover and which dropped9. Establish new SSH sessions from any disconnected stations: ssh admin@10.0.0.110. Execute run show stack through the new sessions |
| Expected Results | 1. Step 4: All three SSH sessions active and responsive, show stack returns correct output on each2. Step 7: SSH sessions may drop during switchover (expected behavior)3. Step 8: Record count of dropped sessions and time-to-drop4. Step 9: New SSH sessions to 10.0.0.1 established successfully after switchover completes5. Step 10: show stack on new sessions shows Unit 2 as master, Unit 1 as standby |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P1 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-2. Document session survival rate and reconnection time. |

##### 1.2.3.2 Inband under high management traffic load

| Field | Content |
| --- | --- |
| Test Name | INBAND-10: Inband stability under high management traffic |
| Purpose Of The Test | Verify that the inband management interface remains responsive under high management traffic load (concurrent SNMP polls, syslog output, and SSH sessions) |
| Test Topo & Precondition | Topology:Master (Unit 1) ---- [Stack Link] ---- Standby (Unit 2) ---- [Stack Link] ---- Member (Unit 3)NMS Station (10.0.0.101/24) ---- [L3 Network] ---- (VLAN 1 SVI: 10.0.0.1/24 on Stack)Management Station (10.0.0.100/24) ---- [L3 Network] ---- (VLAN 1 SVI: 10.0.0.1/24 on Stack)Syslog Server (10.0.0.200/24) ---- [L3 Network] ---- (VLAN 1 SVI: 10.0.0.1/24 on Stack)Preconditions:1. PicOS 4.6 running on all switches2. Unit 1 is master, Unit 2 is standby, Unit 3 is member — all operational3. Inband management VLAN interface configured with IP 10.0.0.1/244. SSH, SNMP, and Syslog services enabled5. NMS station has SNMP polling tool capable of rapid pollingConfiguration:Master (Unit 1):set interface vlan vlan1 family inet address 10.0.0.1/24set system services ssh enable trueset system services snmp community public authorization read-onlyset system services snmp enable trueset system syslog host 10.0.0.200 facility any severity debugcommit |
| Test Procedure | 1. Establish 5 concurrent SSH sessions from Management Station to 10.0.0.12. Start SNMP walk loop on NMS Station: while true; do snmpwalk -v2c -c public 10.0.0.1 ifTable; done3. Configure syslog severity to “debug” to generate high syslog output volume4. Start continuous ping from Management Station to 10.0.0.1: ping -D -i 0.2 10.0.0.1 (5 pings/sec)5. Wait 300 seconds under sustained load6. Execute run show stack through one SSH session7. Execute run show system cpu through one SSH session8. Stop SNMP walk loop on NMS Station9. Stop continuous ping and record results10. Execute run show interface vlan vlan1 counters on the master |
| Expected Results | 1. Step 5: All 5 SSH sessions remain responsive throughout the 300-second load period2. Step 6: show stack returns correct output with acceptable response time (< 5 seconds)3. Step 7: CPU utilization remains below 90% — system not saturated by management traffic4. Step 9: Continuous ping shows < 1% packet loss over the 300-second period5. Step 10: VLAN 1 interface counters show high packet counts with no error increments |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P2 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-2. Record CPU utilization peaks and SSH response latency. |

##### 1.2.3.3 Inband long-running stability — 24-hour continuous management

| Field | Content |
| --- | --- |
| Test Name | INBAND-11: 24-hour continuous inband management stability |
| Purpose Of The Test | Verify that the inband management interface remains stable and responsive over a 24-hour continuous operation period |
| Test Topo & Precondition | Topology:Master (Unit 1) ---- [Stack Link] ---- Standby (Unit 2) ---- [Stack Link] ---- Member (Unit 3)Management Station (10.0.0.100/24) ---- [L3 Network] ---- (VLAN 1 SVI: 10.0.0.1/24 on Stack)NMS Station (10.0.0.101/24) ---- [L3 Network] ---- (VLAN 1 SVI: 10.0.0.1/24 on Stack)Preconditions:1. PicOS 4.6 running on all switches2. Unit 1 is master, Unit 2 is standby, Unit 3 is member — all operational3. Inband management VLAN interface configured with IP 10.0.0.1/244. SSH and SNMP services enabled5. Automated monitoring script prepared on Management StationConfiguration:Master (Unit 1):set interface vlan vlan1 family inet address 10.0.0.1/24set system services ssh enable trueset system services snmp community public authorization read-onlyset system services snmp enable truecommit |
| Test Procedure | 1. Start continuous ping from Management Station to 10.0.0.1 with logging: ping -D -i 5 10.0.0.1 > /tmp/ping_24h.log2. Start SNMP polling script on NMS Station polling sysUpTime every 60 seconds for 24 hours with logging3. Establish SSH session from Management Station and start automated script executing run show stack every 300 seconds (5 minutes), logging output with timestamps4. Wait 86400 seconds (24 hours)5. Stop continuous ping and record results from /tmp/ping_24h.log6. Stop SNMP polling script and record results7. Stop SSH monitoring script and record results8. Execute run show system cpu on the master9. Execute run show system memory on the master10. Execute run show interface vlan vlan1 counters on the master |
| Expected Results | 1. Step 5: 24-hour ping log shows > 99.9% success rate with no sustained outage > 30 seconds2. Step 6: SNMP polling log shows all 1440 polls (every 60s for 24h) returned valid sysUpTime values, monotonically increasing3. Step 7: SSH monitoring log shows all 288 show stack executions (every 5 min for 24h) returned valid output with all units in expected roles4. Step 8: CPU utilization within normal range (< 50% idle-state average)5. Step 9: Memory utilization shows no significant growth over 24 hours (no memory leak indication)6. Step 10: VLAN 1 interface counters show no error increments over the 24-hour period |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P2 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-2. Requires dedicated 24-hour test window. Record memory and CPU trends. |

##### 1.2.3.4 Inband with multiple management protocols simultaneously

| Field | Content |
| --- | --- |
| Test Name | INBAND-12: Multiple management protocols simultaneously |
| Purpose Of The Test | Verify that the inband management interface supports SSH, SNMP, Syslog, and NTP running simultaneously without conflict or performance degradation |
| Test Topo & Precondition | Topology:Master (Unit 1) ---- [Stack Link] ---- Standby (Unit 2) ---- [Stack Link] ---- Member (Unit 3)Management Station (10.0.0.100/24) ---- [L3 Network] ---- (VLAN 1 SVI: 10.0.0.1/24 on Stack)NMS Station (10.0.0.101/24) ---- [L3 Network] ---- (VLAN 1 SVI: 10.0.0.1/24 on Stack)Syslog Server (10.0.0.200/24) ---- [L3 Network] ---- (VLAN 1 SVI: 10.0.0.1/24 on Stack)NTP Server (10.0.0.201/24) ---- [L3 Network] ---- (VLAN 1 SVI: 10.0.0.1/24 on Stack)Preconditions:1. PicOS 4.6 running on all switches2. Unit 1 is master, Unit 2 is standby, Unit 3 is member — all operational3. Inband management VLAN interface configured with IP 10.0.0.1/244. SSH, SNMP, Syslog, and NTP services enabled5. NTP server at 10.0.0.201 synchronized and serving timeConfiguration:Master (Unit 1):set interface vlan vlan1 family inet address 10.0.0.1/24set system services ssh enable trueset system services snmp community public authorization read-onlyset system services snmp enable trueset system syslog host 10.0.0.200 facility any severity infoset system ntp server 10.0.0.201set system ntp enable truecommit |
| Test Procedure | 1. Establish SSH session from Management Station: ssh admin@10.0.0.12. Start SNMP polling from NMS Station: snmpwalk -v2c -c public 10.0.0.1 system3. Start packet capture on Syslog Server filtering UDP port 5144. Execute run show system ntp associations through the SSH session5. Shutdown and re-enable interface ge-1/0/1 to generate syslog events: set interface gigabit-ethernet ge-1/0/1 enable false then commit, wait 5 seconds, set interface gigabit-ethernet ge-1/0/1 enable true then commit6. Wait 30 seconds for all protocols to process7. Execute run show system services ssh through the SSH session8. Execute SNMP GET from NMS Station: snmpget -v2c -c public 10.0.0.1 sysUpTimeInstance9. Stop packet capture on Syslog Server10. Execute run show system ntp status through the SSH session11. Execute run show system cpu through the SSH session |
| Expected Results | 1. Step 1: SSH session established and responsive2. Step 2: SNMP walk returns complete system group MIB objects3. Step 4: NTP associations show server 10.0.0.201 with “synchronized” or “reach” status4. Step 8: SNMP GET returns valid sysUpTimeInstance value5. Step 9: Syslog server packet capture contains interface down/up events from 10.0.0.16. Step 10: NTP status shows synchronization with 10.0.0.201, stratum value present, offset within acceptable range7. Step 11: CPU utilization remains below 80% with all four management protocols active simultaneously |
| Automated or Not | Not Yet |
| Related Scripts | N/A |
| Level | P1 |
| Hardware Model | All PicOS Switches |
| Version | PicOS 4.6 |
| Actual Results |  |
| Test Results |  |
| Remark | Developer requirement TP-2 |

## Coverage Summary

### Module Coverage Matrix

| Module ID | Module Name | Sub-Feature | P0 Cases | P1 Cases | P2 Cases | Total |
| --- | --- | --- | --- | --- | --- | --- |
| MOD-1 (1.1) | Port Pre-Configuration | 1.1.1 Basic Pre-Configuration | 3 | 1 | 0 | 4 |
| MOD-1 (1.1) | Port Pre-Configuration | 1.1.1 Basic Pre-Configuration (multi-port) | 1 | 0 | 0 | 1 |
| MOD-1 (1.1) | Port Pre-Configuration | 1.1.2 Module Feature Interaction | 0 | 5 | 0 | 5 |
| MOD-1 (1.1) | Port Pre-Configuration | 1.1.3 Pre-Config Persistence | 2 | 1 | 1 | 4 |
| MOD-1 (1.1) | Port Pre-Configuration | 1.1.3 Pre-Config Persistence (reboot) | 1 | 0 | 0 | 1 |
| MOD-2 (1.2) | Inband Management | 1.2.1 Basic Inband Connectivity | 2 | 2 | 0 | 4 |
| MOD-2 (1.2) | Inband Management | 1.2.2 Inband During Stack Events | 3 | 1 | 0 | 4 |
| MOD-2 (1.2) | Inband Management | 1.2.3 Inband Stability | 0 | 2 | 2 | 4 |

### Priority Distribution

| Priority | MOD-1 (Pre-Config) | MOD-2 (Inband) | Total |
| --- | --- | --- | --- |
| P0 | 7 | 5 | 12 |
| P1 | 6 | 5 | 11 |
| P2 | 2 | 2 | 4 |
| Total | 15 | 12 | 27 |

### Feature Coverage Checklist

| # | Coverage Item | Status |
| --- | --- | --- |
| 1 | Pre-config basic port settings (speed, MTU, description) | Covered (PRECONFIG-01) |
| 2 | Pre-config VLAN assignment (access/trunk) | Covered (PRECONFIG-02, -05) |
| 3 | Pre-config L3 IP address | Covered (PRECONFIG-03) |
| 4 | Pre-config removal before member join | Covered (PRECONFIG-04) |
| 5 | Pre-config multiple ports on same member | Covered (PRECONFIG-05) |
| 6 | Pre-config ACL interaction | Covered (PRECONFIG-06) |
| 7 | Pre-config QoS interaction | Covered (PRECONFIG-07) |
| 8 | Pre-config LAG/port-channel interaction | Covered (PRECONFIG-08) |
| 9 | Pre-config STP interaction | Covered (PRECONFIG-09) |
| 10 | Pre-config OSPF interaction | Covered (PRECONFIG-10) |
| 11 | Pre-config survives switchover | Covered (PRECONFIG-11) |
| 12 | Pre-config retained on member leave | Covered (PRECONFIG-12) |
| 13 | Pre-config hardware mismatch | Covered (PRECONFIG-13) |
| 14 | Pre-config after stack reboot | Covered (PRECONFIG-14) |
| 15 | Pre-config consistency across members | Covered (PRECONFIG-15) |
| 16 | Inband SSH access | Covered (INBAND-01) |
| 17 | Inband SNMP access | Covered (INBAND-02) |
| 18 | Inband Syslog output | Covered (INBAND-03) |
| 19 | Inband non-default VLAN | Covered (INBAND-04) |
| 20 | Inband during switchover (downtime measurement) | Covered (INBAND-05) |
| 21 | Inband during member join | Covered (INBAND-06) |
| 22 | Inband during member leave | Covered (INBAND-07) |
| 23 | Inband during stack link failure | Covered (INBAND-08) |
| 24 | Concurrent SSH during switchover | Covered (INBAND-09) |
| 25 | Inband under high traffic load | Covered (INBAND-10) |
| 26 | Inband 24-hour stability | Covered (INBAND-11) |
| 27 | Inband multiple protocols simultaneously | Covered (INBAND-12) |

### Developer Requirement Traceability

| Requirement | Test Cases | Coverage |
| --- | --- | --- |
| TP-1 (Port Pre-Configuration) | PRECONFIG-01 through PRECONFIG-15 | 15 cases |
| TP-2 (Inband Management) | INBAND-01 through INBAND-12 | 12 cases |

End of document — 27 test cases total